3GPP TSG-RAN3 Meeting #129	R3-255333

Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	(Draft reply LS) LS on parallel service request

Document for:	Discussion

1	Discussion

In this contribution, we’d like to discuss the LS from RAN2 on parallel service request in [1].

RAN2 discussed the support of parallel service requests for overlapping reader scenario, and made below agreements: Agreements on parallel service request - Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification. - The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.

According to RAN3 previous agreements, multiple readers may cover the same A-IoT area, and and parallel sessions are possible between the RAN and the AIOTF, which may potentially resulting in parallel session over A-IoT radio.

From the RAN2 perspective, to minimize the complexity of device management, parallel service requests targeting the same device are not expected. The RAN2 agreement outlines two specific scenarios:

Case 1: A single reader should not send parallel service requests simultaneously

Case 2: Multiple readers covering the same area should not send parallel service requests simultaneously.

Case 1 is straightforward and can be addressed by reader implementation, while Case 2 can be managed via gNB implementation.

However, if such scenarios do occur, according to RAN2 agreement, the device will always respond to the most recent service request, and any previous request will be terminated.

It is recommended that RAN3 confirm the RAN2 agreements and include a “NOTE” in the Stage 2 inventory procedures to reflect this behavior as below.

Figure 16.xx.x3-1: Inventory procedure <<<<<<<<<<<<<<<<omit the irrelevant parts>>>>>>>>>>>>>>>>>>>>>> 2. The gNB allocates and co-ordinates the usage of A-IoT radio resources. NOTE X, the gNB ensures that there is no parallel service request for the overlapping reader scenario at A-IoT radio by implementation.

Observation 1: While parallel sessions between RAN and AIOTF are possible, parallel sessions for the device can be mitigated by gNB implementation. Specifically, the gNB can prevent different readers or the same reader from sending overlapping inventory requests to the same area.

Proposal 1: RAN3 confirm the RAN2 agreement.

We have no strong view whether to send a reply LS, if needed, an draft reply LS is provided in the Annex.

2	Conclusion

In this contribution, we discussed the issues and questions in the LS from RAN2 and provide the draft reply LS in the Annex.

Observation 1: While parallel sessions between RAN and AIOTF are possible, parallel sessions for the device can be avoided by gNB implementation. Specifically, the gNB can prevent different readers or the same reader from sending overlapping inventory requests to the same area.

Proposal 1: RAN3 confirm the RAN2 agreement.

We have no strong view whether to send a reply LS, if needed, an draft reply LS is provided in the Annex.

- 3				Reference

[1]R3-255015/R2-2504935, LS on parallel service request, RAN2

4	Annex (Reply LS to RAN2 )

Title:	LS Reply on LS on parallel service request

Response to:	R3-255015/R2-2504935

Release:	Release 19

Work Item:	Ambient_IoT_solutions

Source:	RAN3

To:	RAN2

Cc:

Contact person:	Lisi Li

lilisi@xiaomi.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 thanks RAN2 LS on parallel service request.

From RAN3 perspective, while parallel sessions between RAN and AIOTF are possible, parallel sessions for the device can be avoided by gNB implementation. Specifically, the gNB can prevent different readers or the same reader from sending parallel inventory requests to the same area.

2	Actions

To RAN2

ACTION: 	RAN3 respectfully requests RAN2 to take above replies into account.

3	Dates of next TSG RAN WG 3 meetings

TSG-RAN3 Meeting #129-bis	13-17 Oct 2025			Prague, CZ

TSG-RAN3 Meeting #130	17-21 Nov 2025			Dallas , US