3GPP TSG-RAN3 Meeting #129	R3-255334

Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	14.3

Source:	Xiaomi

Title:	(TP to BL CR TS 38.300) Support of NG suspend/resume

Document for:	Other

1	Introduction

In this contribution, we’d like to discuss the following open issue

NG Connection interruption

UE context handling

2	Discussion

NG Connection interruption

RAN3 has discussed suspend and resume for NG connection and had the following working assumption.

WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure

The controversial part is whether this suspend/resume indication can also be used for the case where on-board gNB moves out of the AMF serving area.

We provide the following table to compare the the two cases of NG suspend/resume:

Case 1, NG suspend/resume due to feeder link switchover

Case 2, NG suspend/resume due to gNB movement out of the AMF serving area

|  | Case 1 feeder link switchover | Case 2, gNB movement out of the AMF serving area |
| --- | --- | --- |
| Cause | Due to feeder link switch over | Due to gNB movement out of the AMF connection area |
| Interruption time | The interruption time is quite low | The interruption time is depends on the satellite speed and longer than the interruption time due to feeder link switch |
| AMF behavior | AMF keeps the UE context and RAN node context. For siganlling, the AMF suspend/resume the signalling transmission accordingly. For user data, the AMF notifies the the SMF, then the SMF notify UPF to suspend/resume data transmission accordingly. | AMF only keeps the RAN node context The UE context in AMF may be updated as UE may be released or handover to other gNB No signalling/data suspend/resume |
| gNB behavior | gNB keeps the UE context and AMF node context | gNB either perform handover or release for the connected UE gNB keeps AMF node context |
| UE service continuity | UE is still be served by the same gNB and CN | UE should be released or handover/relocated to other gNBs. |

It is noticed that the distinctions between Case 1 and Case 2 lie in the interruption time, AMF/gNB behavior, and UE service continuity.

In general, we support NG suspend/resume in the context of feeder link switchover, as it helps reduce packet loss. However, if this indication is also applied to scenarios where the gNB moves out of the AMF serving area, the AMF behaviors may differ. Therefore, it is important for SA2 to be aware of this and define the corresponding AMF behavior.Additionally, we observed that suspending or resuming user data could have implications for both SA2 and CT4. Therefore, a mechanism is needed for the AMF to notify the SMF/UPF. To support this, we have provided a draft liaison statement in [1].

Proposal 1, RAN3 agrees to support NG suspend/resume for feeder link switchover and NG connection handling.

Observation 1, AMF behaviors are different for the two cases, which have SA2 impacts.

Observation 2,  In case of feeder link switchover, the AMF should notify SMF/UPF to suspend/resume the data transmission, which may have SA2 and CT4 impacts.

Proposal 2, RAN3 agrees to send SA2, cc CT4 on the core network support of the NG suspend/resume.

UE context handling

RAN3 had agreed to introduce NG removal procedure in the scenrio where the on-board gNB moves out of the AMF serving area, to ensure UE service continuity, we think how to handle the UE context due to NG removal should be discussed. When a gNB onboard decides to trigger NG removal, before it sends the NG removal request, it should trigger handovers for the RRC_CONNECETED UEs, and relocate/release the contexts for the RRC_INACTIVE UEs.

The corresponding TP in the Annex is provided to clarify this scenario, considering NG suspend/resume may also be introduced, a separate section for the UE context handling  is used.

Proposal 3,  Capture the following in stage 2 “When an onboard gNB moves out of the serving area of an AMF/AMF set, it may trigger either the NG removal procedure or send an NG suspend notification. Before doing so, the gNB should initiate handovers to transfer any RRC_CONNECTED UEs associated with the NG connection to other AMFs or gNBs. At the same time, it should either release or relocate the contexts of UEs in the RRC_INACTIVE state. ”

3	Conclusion

Based on the discussion, we have the following observations and proposals.

Proposal 1, RAN3 agrees to support NG suspend/resume for feeder link switchover and NG connection handling.

Observation 1, AMF behaviors are different for the two cases, which have SA2 impacts.

Observation 2,  In case of feeder link switchover, the AMF should notify SMF/UPF to suspend/resume the data transmission, which may have SA2 and CT4 impacts.

Proposal 2, RAN3 agrees to send SA2, cc CT4 on the core network support of the NG suspend/resume.

Proposal 3, Capture the following in stage 2 “When an onboard gNB moves out of the serving area of an AMF/AMF set, it may trigger either the NG removal procedure or send an NG suspend notification. Before doing so, the gNB should initiate handovers to transfer any RRC_CONNECTED UEs associated with the NG connection to other AMFs or gNBs. At the same time, it should either release or relocate the contexts of UEs in the RRC_INACTIVE state. ”

4	Reference

[1]  R3-255335, (Draft LS out) Support of NG suspend resume, Xiaomi

5	Annex (TP to BL CR 38.300)

<<<<<<<<<<<<<<<<<<<< Change starts>>>>>>>>>>>>>>>>>>>>

### 16.14.y	UE context handling

When an onboard gNB moves out of the serving area of  an AMF/AMF set, it may trigger either the NG removal procedure or send an NG suspend notification. Before doing so, the gNB should initiate handovers to transfer any RRC_CONNECTED UEs associated with the NG connection to other AMFs or gNBs. At the same time, it should either release or relocate the contexts of UEs in the RRC_INACTIVE state.

<<<<<<<<<<<<<<<<<<<< Change ends>>>>>>>>>>>>>>>>>>>>