3GPP TSG-RAN WG3 Meeting #129				   		      	   R3-255373

Bengaluru, India, 25 – 29 August, 2025

Title: 	(Draft Reply LS to RAN2) Discussion on R3-255016 (R2-2504950) LS 	Logged Data Handling During Handover

Source: 	Huawei

Agenda item:	20.2

Document for:	Discussion/Decision

# Introduction

In the last meeting RAN2 discussed about the handing of logged data in the UE collected for NW-side AI/ML model training when the UE is handed over from the source gNB to the target gNB, and agreed to send a LS to RAN3 in [1] listing the relevant RAN2 agreements. As part of the ACTION in the LS, RAN2 asks RAN3 to consider the listed agreements and provide concerns, if any, on such agreements.

This paper provides our understanding of logged data handling scenario discussed in RAN2 and lists some aspects having RAN3 impacts to support such scenario, for which RAN2 may need to provide additional information for further clarification.

# Discussion

## Collected data handling during HO

In the LS [1], RAN2 indicates following agreements have been achieved in RAN2:

At RAN2#127bis: UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network. At RAN2#129: - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. At RAN2#130: - (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced. In case of HO, the source sends the 1-bit indication to target cell in HandoverPreparationInformation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data). We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it. Notify RAN3 - The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

The above agreements reflect the discussion occurring in RAN2 on the two NW-side data collection approaches, i.e., OAM-centric and gNB-centric data collection: the former, i.e., OAM-centric data collection, means that the termination point of the collected data is the OAM, hence the data should be made available at the OAM; while, in gNB-centric data collection, the termination point of the collected data is the gNB, and collected data should be made available at the gNB. These two approaches can be represented in the following call flow and related steps description:

Figure 1 - NW-side data collection in case of UE handover.

Step 0a: In case of OAM-centric data collection, the data collection configuration is provided by the OAM to the UE, via the source gNB

Step 0: The source gNB sends the data collection configuration to the UE, which includes data logging and report configurations.

NOTE: Step 0 above is valid for both OAM-centric and gNB-centric data collection. All the Steps 	described below are common to both approaches.

Step 1: Source gNB sends 1 bit indication within the RRC container (HandoverPreparationInformation) of the HANDOVER REQUEST message to the target gNB to indicate the need for retrieving the logged data in the handed over UE.

Step 2: Target gNB performs admission control and decides whether to keep the un-retrieved data from the UE.

Step 3: If target gNB decides to keep the UE logged data, it sends the 1 bit indication (as received in Step 1 by the source gNB) within the RRC container (HandoverCommand) of the HANDOVER REQUEST ACKNOWLEDGE message to indicate the UE to retain the logged data.

Step 4: Source gNB sends the RRCReconfiguration message to the UE, which includes 1 bit indication as received in Step 3.

Step 5: the UE keeps the logged un-retrieved data and sends the data availability indication to the target gNB within the RRCReconfigurationComplete message, also including the CGI of the serving cell or, if unavailable, the PCI-ARFCN.

Step 6: The target gNB sends UEInformationRequest message to the UE to retrieve the logged data.

Step 7: The UE sends the logged data to the target gNB via UEInformationResponse message.

From the above call-flow, it is not clear in the context of the RAN2 agreements mentioned in the LS what should the target gNB do when it retrieves the logged data from the UE. It is our understanding that the logged data retrieved by the UE needs to be made available at the source gNB, hence following the gNB-centric data collection approach mentioned above, where, according to the RAN2 LS, the data collection configuration could also originate from the source gNB (see highlighted text in yellow below):

RAN2 would like to clarify that the data collection configuration could originate in the source gNB or from OAM.

Proposal 1: RAN3 to ask RAN2 whether the data retrieved from the UE by the target gNB should be made available at the source gNB.

Assuming that the retrieved UE data should be made available at the source gNB, one aspect that RAN2 should provide clarifications on is how time critical the data delivery is. In our understanding, the data to be retrieved from the handed over UE should be used for offline training of the AI/ML model hosted in the source gNB, therefore there should not be strict timing requirements to provide such training data to the source gNB.

Proposal 2: RAN3 to ask RAN2 information related to how time critical the data delivery from target gNB to source gNB is.

In principle the UE data retrieved by the target gNB could be either forwarded directly to the source gNB (over Xn) or via the OAM, and RAN3 should discuss these two options for data delivery.

To make a reasoned decision on which option to go for, RAN3 should know the typical type and size of the UE data retrieved by the target gNB.

In our understanding, the data collected by the UE is L1 RSRP of different multiple beams measured periodically by the UE which then logs such measurements. When it comes to the data size of the UE logged data, this is related to UE memory, and even if the UE memory size has not  been decided yet in RAN2, it is reasonable to assume the baseline UE memory size of 64KB as maximum volume of data collected and stored in the UE, even if there could be additional sizes supported as UE capability. Currently, the SCTP protocol used for the XnAP has a size limitation of 16KB as maximum data size. Therefore, and still assuming that the process of UE data delivery from the target gNB to the source gNB is non time-critical, we think RAN3 should make RAN2 aware of the current limitation of the SCTP protocol used for the Xn interface in terms of maximum data size that may prevent transferring UE data to the source gNB directly over XnAP.

Proposal 3: RAN3 to make RAN2 aware of the current protocol limitations in terms of maximum amount of data to be delivered over XnAP that may prevent sending the retrieved UE data from target gNB directly to the source gNB over Xn.

Considering that, in our understanding, the data delivery process towards the source gNB is non time-critical, and considering the SCTP protocol limitations in terms of maximum data size mentioned earlier, we think RAN3 should indicate RAN2 that retrieved UE data could be made available at the target gNB via the OAM. Specifically, once the UE data has been retrieved by the target gNB, this data is sent to OAM and it will be up to the source gNB to then decide when to retrieve such data from the OAM, also taking into account the time interval during which it keeps the context of the handed over UE stored in the gNB itself (and which is implementation-specific). And, in our view, this option has no RAN3 impacts and hence easy to achieve in Rel-19 considering that RAN3 has no TUs available.

Proposal 4: RAN3 to indicate to RAN2 that, in Rel-19, retrieved UE data could be made available at the source gNB via the OAM.

A final aspect which is not mentioned in the RAN2 LS is related to whether the source gNB should know to which UE the retrieved data belong to. We think that RAN3 should ask clarification on this to RAN2, as it could be possible that, before the UE is handed over, the gNB source might have already retrieved some data from this UE, and hence it would be desirable to combine this data to determine a complete training dataset. This however is pending RAN2 confirmation.

Proposal 5: RAN3 to ask RAN2 whether the source gNB should know the UE from which data has been retrieved by the target gNB.

A draft Reply LS to RAN2 (Cc SA5) is provided in the Annex of this paper.

# Conclusion

Proposal 1: RAN3 to ask RAN2 whether the data retrieved from the UE by the target gNB should be made available at the source gNB.

Proposal 2: RAN3 to ask RAN2 information related to how time critical the data delivery from target gNB to source gNB is.

Proposal 3: RAN3 to make RAN2 aware of the current protocol limitations in terms of maximum amount of data to be delivered over XnAP that may prevent sending the retrieved UE data from target gNB directly to the source gNB over Xn.

Proposal 4: RAN3 to indicate to RAN2 that, in Rel-19, retrieved UE data could be made available at the source gNB via the OAM.

Proposal 5: RAN3 to ask RAN2 whether the source gNB should know the UE from which data has been retrieved by the target gNB.

A draft Reply LS to RAN2 (Cc SA5) is provided in the Annex of this paper.

# References

- [1] R3-255016 (R2-2504950), Logged Data Handling During Handover, RAN2 (Nokia)

# Annex – Draft Reply LS to RAN2

3GPP TSG-RAN3 Meeting #129	R3-25xxxx

Bengaluru, India, 25th – 29th August, 2025

Title:	[Draft] LS reply to LS on Logged Data Handling During Handover

Response to:	R3-255016 (R2-2504950: Logged Data Handling During Handover)

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	Huawei [to be RAN3]

To:	RAN2

Cc:	SA5

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN3 thanks RAN2 for their reply LS on Logged Data Handling During Handover. RAN3 discussed the content of the LS, and would like to provide the following feedback.

RAN2 is kindly requested to confirm the scenario assumed by RAN3 that, after the target gNB has retrieved the data from the handed over UE, such data needs to be made available at the source gNB where the data collection configuration originated. Under this understanding of the scenario, RAN3 further asks RAN2 to clarify:

- whether the data delivery from target gNB to source gNB is time-critical: it is RAN3 understanding that data collected by the UE may be for offline training of AI/ML models, and hence non time-critical;

- which is the typical type and size of the UE data retrieved by the target gNB and that needs to be transferred to the source gNB: in this regards, it is worth noting to mention that current SCTP protocol used for the Xn signalling has a maximum data size of 16KB;

- whether the source gNB should know to which UE the retrieved data belong to: this may enable the source gNB to correlate data collected from the same UE before it has been handed over to the target gNB.

Pending RAN2 feedback on the above, RAN3 may consider a OAM-based solution in Rel-19 based on which, once the UE data has been retrieved by the target gNB, this data is sent to OAM and it will be up to the source gNB to then decide when to retrieve such data from the OAM. Such OAM-based solution may have no RAN3 impacts, also considering that RAN3 has no TUs available in this Release.

# 2	Actions

To RAN2

ACTION: 	RAN3 respectfully asks RAN2 to take the above into account and provide feedback accordingly.

# 3	Dates of next TSG RAN WG3 meetings

RAN3#129bis		13 October – 19 October 2025			Prague, CZ

RAN3#130		17 November – 21 November			Dallas, US