3GPP TSG-RAN WG3 Meeting #129	R3-255433

Bengaluru, India, 25th – 29th August, 2025

Title: 	SON/MDT for Intra-NTN Mobility

Source: 	Jio Platforms

Agenda item:	10.3.1

Document Type:	Discussion and Decision

# 1. Introduction

During RAN3#128 in Malta, discussions progressed on enhancing Minimization of Drive Test (MDT) support for Non-Terrestrial Networks (NTN), specifically for intra-NTN mobility scenarios, as captured in the meeting minutes:

From RAN3#128:

To introduce a geographical area scope for NTN MDT including either reference location/radius or a polygon-based area for NG and Xn.

FFS on whether to add the geographical area scope inside or outside the existing area scope IE.

FFS on whether the geographical area scope applies to both logged and immediate MDT or only logged MDT.

A key topic was the introduction of a geographical area scope for NTN MDT, enabling more context-aware data collection. Delegates considered defining this scope either through a reference location with a radius or a polygon-based area, applicable across NG and Xn interfaces. Further study was agreed upon regarding the optimal placement of this area scope—whether inside or outside the existing area scope Information Element (IE). Additionally, clarification is required on whether this enhancement should apply solely to logged MDT or also to immediate MDT reporting. These considerations aim to improve mobility insights and optimize coverage and performance in NTN environments.

This contribution contributes further to this discussion focusing on the two FFS open issues.

# 2. Background Discussion on Geographical Area Scope

Following the progress during RAN3#128 in Malta, discussions have focused on introducing a geographical area scope to support more efficient and context-aware Minimization of Drive Test (MDT) reporting in Non-Terrestrial Networks (NTN).

One open item remains under further study (FFS):

(1) Whether to add the geographical area scope inside or outside the existing area scope IE.

Indeed during RAN3#128, the group explored enhancement options for enabling Minimization of Drive Test (MDT) capabilities in Non-Terrestrial Network (NTN) environments, particularly for intra-NTN mobility. A critical topic was the introduction of a geographical area scope specific to NTN-MDT operation, aiming to facilitate selective and contextually-relevant logging of UE measurements. Two main representation schemes were discussed: a reference point with radius and a polygonal boundary, to be signaled via NG and Xn interfaces.

The current open question pertains to the placement of this new NTN-specific geographical area scope, i.e., whether it should be encapsulated inside the existing Area Scope IE, or defined outside as a separate configuration element. This document offers a detailed technical assessment of the implications of each approach, with attention to specification clarity, UE implementation complexity, and future extensibility. Integrating the NTN-specific area scope within the existing Area Scope IE offers a simpler implementation model for UEs. It reduces the number of conditions the UE must evaluate for measurement logging triggers—i.e., from two concurrent conditions to a single boundary check. While this is valid from a software and signaling overhead standpoint, this approach also entails a number of implicit design assumptions and downstream implications, as well as advantages.

In this approach the UE only checks a single configured area boundary to determine whether to initiate MDT logging. This reduces processing and avoids ambiguity during transitions across geographical zones. Moreover reusing of the Area Scope IE avoids defining new signaling semantics or extending the information model across NG and Xn interfaces. Finally no new IEs or encoding logic are introduced into the specification, preserving consistency with current MDT signaling frameworks.

Observation 1: Introducing the geographical area scope inside the existing area Scope IE may simplify UE implementation and signalling interpretation.

However, a number of technical concerns and considerations arise that merit careful discussion. The current Area Scope IE was originally intended for Terrestrial Network (TN) operation. Its reuse for NTN may blur the semantics of "area scope" as it now encompasses both orbital and terrestrial coverage logic, which are governed by distinct mobility and coverage dynamics. Moreover in future Rel-20 or beyond, MDT support for hybrid NTN-TN mobility scenarios may become relevant. Embedding NTN logic within a legacy IE might restrict flexibility and necessitate deprecations or extensions later.

Considering also the existing IEs overload, may create confusion for readers or implementers, particularly when interpreting whether an area scope applies to TN, NTN, or both. This ambiguity could lead to misconfiguration, especially in multi-mode UEs operating across TN and NTN. Finally the argument that a dual-condition evaluation introduces undue complexity at the UE side may be overstated. UEs already handle multi-condition triggers (e.g., conditional HO, dual connectivity thresholds, etc.). Modern UE chipsets are capable of evaluating multiple geofenced areas or temporal thresholds efficiently.

Observation 2: Introducing the geographical area scope inside the existing area Scope IE could limit extensibility for future TN–NTN aligned MDT scenarios.

Proposal 1: RAN3 to add as one option the geographical area scope inside the existing area scope IE.

In light of the above concerns, we propose to consider as another option the definition of the NTN-specific geographical scope as a separate optional IE, referenced alongside the existing Area Scope IE, rather than embedded within it. This maintains clean semantic separation, ensures extensibility, and allows implementations to evolve with NTN-specific logic, without overloading legacy constructs. Such an approach also allows for clear tagging of scope types—e.g., a new field could explicitly indicate whether the scope pertains to TN, NTN, or both—enabling cleaner interworking across networks and future enhancements for hybrid coverage areas.

This alternative solution does not mandate dual-condition evaluation in the UE but allows implementations to treat both scopes conjunctively or disjunctively based on deployment preferences. Moreover, it improves specification clarity and supports differentiated handling of TN vs NTN without risking misinterpretation.

Observation 3: Defining the NTN-specific geographical scope as a separate optional IE, referenced alongside the existing Area Scope IE, could enhance specification clarity, support NTN–TN differentiation, and enable flexible UE behaviour without overloading legacy IEs.

Proposal 2: RAN3 to consider as a second option, defining a dedicated optional IE for NTN geographical area scope, referenced alongside the existing Area Scope IE, with explicit indication of applicability (e.g., TN, NTN, or both).

Proposal 3: Further analysis is recommended before finalizing the IE structure

# 3. Background Discussion on Logged and Immediate MDT for Geographical Area

Following the progress during RAN3#128 in Malta, discussions have focused on introducing a geographical area scope to support more efficient and context-aware Minimization of Drive Test (MDT) reporting in Non-Terrestrial Networks (NTN). Another open items remain under further study (FFS):

(2) whether the enhancement should apply to logged MDT only or to both logged and immediate MDT.

Minimization of Drive Tests (MDT) has evolved as a foundational feature for mobility and coverage optimization in both terrestrial and non-terrestrial contexts. The introduction of a “geographical area scope” in NTN scenarios is intended to enable localized, context-aware data collection, particularly beneficial in areas of intermittent coverage or dynamic beam footprints. In terrestrial deployments, geographical constraints for MDT have primarily focused on static cells or overlapping terrestrial sectors. In contrast, the NTN environment introduces unique challenges due to the spatiotemporal nature of satellite footprints, moving coverage beams, and varying GNSS visibility for UEs.

If we try to justify the two options dilemma on whether the geographical area scope would apply to logged MDTonly or to both logged and immediate MDT, we should first analyse the two options with the implementation advantages and disadvantages. 

Option A: Apply to both Logged and Immediate MDT

Applying the geographical area scope to both logged and immediate MDT ensures a unified framework for context-aware measurements. This supports optimization actions during both active and idle states of the UE. The benefits and technical justification may be summarized as following:

- Contextual Consistency: Both logged and immediate MDT data can be associated with a common area of interest, enabling consistent spatial analysis.

- Proactive Optimization: Immediate MDT within defined areas allows the network to detect transient degradations and mobility failures in real-time.

- Standards Consistency: TS 38.331 already allows for GNSS-based UE location reporting during RRC_CONNECTED, aligning with immediate MDT applicability.

- NTN Adaptability: Many NTN use cases involve predictable satellite tracks and beam steering, where real-time measurements (via Immediate MDT) can capture mobility degradations during handovers between moving beams or GEO/LEO satellites.

- Position Availability: In Release 18/19, it is assumed that UEs in NTN possess valid GNSS data pre-connection. Thus, location-triggered immediate MDT remains feasible.

- Backhaul and Beam Association Usefulness: Immediate feedback supports enhanced association between measurements and moving satellite beams, helping to distinguish beam-specific coverage issues.

Observation 4: supporting the geographical area scope for both Logged and Immediate MDT enables consistent area-based analysis and aligns with GNSS-assisted Immediate MDT reporting in RRC_CONNECTED, particularly beneficial for dynamic NTN beam mobility.

However, there are two main concerns and challenges including:

- Accuracy Risk: Positioning errors in active UEs may lead to undesired or erroneous data triggering if GNSS is degraded or temporarily unavailable.

- Energy Overhead: Immediate MDT can impose power consumption impacts on battery-limited UEs in NTN scenarios, especially where long RTTs and limited uplink windows apply.

Observation 5: applying geographical area scope to Immediate MDT in NTN may introduce risks due to GNSS positioning inaccuracies and increased UE power consumption, especially under limited uplink access and long RTT conditions.

Option B: Apply Only to Logged MDT

Restricting geographical scope to logged MDT simplifies implementation and minimizes overhead. This option is motivated by several advantages. Indeed as discussed during RAN3#128, GNSS availability may not be guaranteed during all active sessions—particularly in obstructed indoor NTN scenarios or initial access with degraded visibility. Moreover Logged MDT is performed during idle mode, avoiding the signalling burden and processing load on UEs that might be critical in power-constrained NTN use cases. An argument to be considered is the Mobility Pattern Granularity, where logged MDT enables post-processed trajectory analysis over a longer time window, which is beneficial in GEO constellations with relatively slower beam transitions. Finally the lower Complexity on UE-Network Synchronization, where no real-time triggering is needed, reduces protocol complexity on the Xn or NG interface.

Observation 6: restricting geographical area scope to logged MDT reduces UE signaling and power overhead, supports post-processed trajectory analysis, and simplifies synchronization, which is beneficial in power-limited NTN environments and GEO constellations.

However, this approach lacks real-time insight and may delay detection of performance drops, especially for rapidly moving satellites or in LEO constellations with short dwell times.

Observation 7: applying geographical scope only to logged MDT may delay detection of real-time performance degradation, limiting effectiveness in dynamic NTN scenarios such as LEO constellations with short beam dwell times.

In view of the above analysis, the geographical area scope should ideally apply to both logged and immediate MDT, under the precondition that GNSS availability can be confirmed or assisted during connected mode. This aligns with the trend in NTN system design where more intelligence is pushed to real-time optimization, particularly to handle beam transitions and handover failures between satellite links.

Therefore:

Proposal 4: RAN3 to agree that the geographical area scope for MDT in NTN shall apply to both Logged MDT and Immediate MDT, subject to availability of UE location information (e.g., GNSS, RAT-assisted, or hybrid methods) during active mode.

Finally as an Optional Forward-Looking Alternative Proposal, to further future-proof the standard and support broader applicability:

Proposal 5: RAN3 to consider introducing a conditional applicability mechanism, where the geographical area scope for immediate MDT in NTN is activated only when the UE location confidence metric exceeds a predefined threshold (e.g., GNSS fix quality or HDOP index).

This last proposal would ensure that immediate MDT triggers are based on reliable location data, mitigating unnecessary measurement collection while maintaining optimization accuracy.

# 4. Conclusions

The introduction of a geographical area scope for MDT in NTN presents a pivotal enhancement to support targeted, context-aware data collection under evolving mobility dynamics. While embedding this scope within the existing Area Scope IE offers implementation simplicity, defining it as a separate optional IE ensures semantic clarity, extensibility, and clean support for future TN–NTN hybrid scenarios. Similarly, applying the scope to both Logged and Immediate MDT enables real-time optimization, provided that location reliability is ensured. A conditional activation mechanism based on UE location confidence could offer a balanced and robust approach for future-proofing the specification.

This paper makes the following observations and proposals:

Observation 1: Introducing the geographical area scope inside the existing area Scope IE may simplify UE implementation and signalling interpretation.

Observation 2: Introducing the geographical area scope inside the existing area Scope IE could limit extensibility for future TN–NTN aligned MDT scenarios.

Proposal 1: RAN3 to add as one option the geographical area scope inside the existing area scope IE.

Observation 3: Defining the NTN-specific geographical scope as a separate optional IE, referenced alongside the existing Area Scope IE, could enhance specification clarity, support NTN–TN differentiation, and enable flexible UE behaviour without overloading legacy IEs.

Proposal 2: RAN3 to consider as a second option, defining a dedicated optional IE for NTN geographical area scope, referenced alongside the existing Area Scope IE, with explicit indication of applicability (e.g., TN, NTN, or both).

Proposal 3: Further analysis is recommended before finalizing the IE structure

Observation 4: supporting the geographical area scope for both Logged and Immediate MDT enables consistent area-based analysis and aligns with GNSS-assisted Immediate MDT reporting in RRC_CONNECTED, particularly beneficial for dynamic NTN beam mobility.

Observation 5: applying geographical area scope to Immediate MDT in NTN may introduce risks due to GNSS positioning inaccuracies and increased UE power consumption, especially under limited uplink access and long RTT conditions.

Observation 6: restricting geographical area scope to logged MDT reduces UE signaling and power overhead, supports post-processed trajectory analysis, and simplifies synchronization, which is beneficial in power-limited NTN environments and GEO constellations.

Observation 7: applying geographical scope only to logged MDT may delay detection of real-time performance degradation, limiting effectiveness in dynamic NTN scenarios such as LEO constellations with short beam dwell times.

Proposal 4: RAN3 to agree that the geographical area scope for MDT in NTN shall apply to both Logged MDT and Immediate MDT, subject to availability of UE location information (e.g., GNSS, RAT-assisted, or hybrid methods) during active mode.

Proposal 5: RAN3 to consider introducing a conditional applicability mechanism, where the geographical area scope for immediate MDT in NTN is activated only when the UE location confidence metric exceeds a predefined threshold (e.g., GNSS fix quality or HDOP index).