3GPP TSG-RAN WG3 Meeting #129	R3-255442

Bangalore, India, 24 – 29 August 2025

Agenda Item:	14.3

Source: 				Jio Platforms Limited

Title:			(TP for TS 38.413) Introduce gNB-initiated AMF switch signaling to support UE context transfer for regenerative NTN gNB mobility

Document for:		Agreement

# 1.	Introduction

This document contains the TP for Introduce gNB-initiated AMF switch signaling to support UE context transfer for regenerative NTN gNB mobility for TS 38.413.

Background: Release-19 extends NR support for Non-Terrestrial Networks (NTN) with regenerative payloads (i.e. gNB on-board a satellite). In such scenarios, as a satellite (regenerative gNB) moves, its NG control-plane connection may need to handover between different AMFs (Access and Mobility Management Functions) serving different ground gateway regions. Without protocol support, ongoing UE contexts might be lost or not seamlessly transferred when the gNB re-homes to a new AMF.

Objective: This Change Request adds an optional NGAP signaling mechanism for gNB-initiated AMF relocation during regenerative NTN gNB mobility. The gNB can indicate the target AMF identity (region/set/pointer) to its current AMF via a RAN Configuration Update message. Upon receiving this indication, the source AMF can trigger UE context transfer to the target AMF over the N14 interface (AMF-to-AMF) using existing procedures from TS 29.518 (e.g. N1MessageNotify and UeContextTransfer service operations). This ensures that when the gNB attaches to the new AMF, the UE’s context is already available, minimizing service interruption.

# 2.	TP for TS 38.413

This appendix provides the Text proposal for TS 38.413 based on RAN3 agreements.

Change Summary: A new Information Element (IE) called GNB-MobilityInformation is introduced in NGAP:

- It contains the identifiers of the target AMF: AMF Region ID (8 bits), AMF Set ID (10 bits), and an optional AMF Pointer (6 bits). This format aligns with the existing AMF identity components defined in NGAP (as part of GUAMI).

- The IE is included optionally in the RAN CONFIGURATION UPDATE message (NG-RAN node → AMF) when a gNB intends to switch to a different AMF (e.g., due to NTN gateway handover). The presence of this IE signals the upcoming mobility to a target AMF.

- The new IE’s criticality is set to “ignore” and its presence is optional, ensuring backward compatibility. If an AMF or network does not support this IE, it will ignore it and continue operation without triggering context transfer (the procedure remains as per legacy behavior). If supported, the source AMF uses the provided AMF identity to initiate context relocation via N14.

- ASN.1 definitions are provided for the new IE, including proper extensibility (iE-Extensions) to allow future enhancements, consistent with 3GPP coding guidelines.

This CR is aligned with Work Item NR_NTN_Ph3-Core (RP-243300) for Release-19 Stage 3 (RAN3). It provides the necessary signaling support in TS 38.413 to fulfill the work item’s objective of enabling seamless mobility for regenerative NTN architectures. The changes are contained to NGAP (no impact to UEs or RAN2 procedures) and are backward-compatible.

## Clauses and Sections Affected

- Clause 8.7.2 (RAN Configuration Update procedure) – Enhanced to describe gNB-initiated AMF switch using the new IE.

- Clause 9.2.6.4 (RAN CONFIGURATION UPDATE message) – Updated IE list to include GNB-MobilityInformation as an optional IE.

- Clause 9.3.x (New subclause for GNB-MobilityInformation IE) – Definition and semantics of the new IE (structure containing AMF Region ID, AMF Set ID, AMF Pointer).

- Clause 9.4 (ASN.1 definitions) – Addition of the new IE type and its inclusion in the RAN Configuration Update message ASN.1.

No other clauses are affected. (For reference, the existing definitions are reused from GUAMI and related IEs: AMF Region ID (within GUAMI), AMF Set ID (9.3.3.12), AMF Pointer (9.3.3.19).)

----------------Start of the First Change---------------

### 8.7.2	RAN Configuration Update

#### 8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the RAN CONFIGURATION UPDATE message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the Global RAN Node ID IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall associate the TNLA to the NG-C interface instance using the Global RAN Node ID.

If the RAN CONFIGURATION UPDATE message includes the NG-RAN TNL Association to Remove List IE, the AMF shall, if supported, initiate removal of the TNL association(s) indicated by NG-RAN TNL endpoint(s) and AMF TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is present, or the TNL association(s) indicated by NG-RAN TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the NG-RAN TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN TNL endpoints correspond to all NG-RAN TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

-    if the received TNL Association Transport Layer Address at AMF IE includes the Port Number IE, the AMF TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the AMF TNL endpoints correspond to all AMF TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the RAN CONFIGURATION UPDATE message includes the RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node. If the RAN CONFIGURATION UPDATE message includes the Extended RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite any previously stored NB-IoT default paging DRX value for the NG-RAN node.

If the RAT Information IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the RAN CONFIGURATION UPDATE message includes the GNB-MobilityInformation IE, the AMF may, if supported, use the indicated AMF identity for preparatory actions internal to the 5GC (e.g., via N14 as specified in TS 29.518) in view of a potential NG-RAN node re-allocation to the indicated target AMF. This does not modify any UE-associated NGAP context as part of this procedure. If the IE is not present or not understood, the AMF shall ignore it and proceed with the RAN Configuration Update as normal.

If the AMF determines that the indicated AMF identity is not reachable or not authorized for the NG-RAN node’s PLMN/S-NSSAI(s), it shall ignore the GNB-MobilityInformation IE.

-----------------Next Change-----------------------------

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| GNB-MobilityInformation | O |  | see clause 9.4 (GNB-MobilityInformation) | Indicates the target AMF identity for gNB-initiated mobility (AMF Region ID, AMF Set ID, and optional AMF Pointer) | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

Explanation: The new IE is optional (“O”) in this message. If present, this IE carries the identifiers of the target AMF. If not present, or if the AMF doesn’t support it, the RAN Configuration Update procedure has no core network relocation implications (backward compatible behavior).

-----------------Next Change---------------

#### 9.3.3.X	<NEW_SUBCLAUSE_NO> GNB-MobilityInformation

This IE specifies the target AMF information to support gNB-initiated AMF switching (e.g., for regenerative NTN gNB mobility). It contains the identifiers of the target AMF within the operator’s network.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GNB-MobilityInformation | M |  | SEQUENCE | Target AMF identity container |

NOTE: This IE serves as a container for the three-part AMF identifier. The combination of AMF Region ID, AMF Set ID, and AMF Pointer (if provided) corresponds to a target AMF’s identity within the 5GC. If the AMF Pointer is omitted, the indication may refer to any AMF in the given set (the source AMF can select one). The NG-RAN node includes this IE to signal that it will move its NG interface association to the AMF identified by these values. Upon receiving this information, the current AMF may prepare the target AMF by transferring UE contexts (via N14). This IE does not directly trigger any NGAP response; it provides information for the AMF’s internal decision-making. If an AMF receives this IE and does not support the feature, it shall ignore the IE (treat as if not present) – per criticality “ignore”, no error is raised.

m-----------------Next Change---------------

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

-- ========= New IE type (Release 19) =========

GNB-MobilityInformation ::= SEQUENCE {

amfRegionID    AMFRegionID,   -- BIT STRING (SIZE(8))

amfSetID       AMFSetID,      -- BIT STRING (SIZE(10))

amfPointer     AMFPointer OPTIONAL,  -- BIT STRING (SIZE(6))

iE-Extensions  ProtocolExtensionContainer {{GNB-MobilityInformation-ExtIEs}} OPTIONAL,

...

}

GNB-MobilityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

... -- No extensions defined in this release

}

-- ========= New ProtocolIE-ID assignment (FFS by editor) =========

id-GNB-MobilityInformation  ProtocolIE-ID ::= <FFS_ProtocolIE_ID>

-- ========= Add IE to the RAN CONFIGURATION UPDATE message =========

RANConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

...,

{ ID id-GNB-MobilityInformation

CRITICALITY ignore

TYPE GNB-MobilityInformation

PRESENCE optional },

...

}

Note - If the last assigned ID in Release-18 was N, this would be N+1). The entry in RANConfigurationUpdateIEs ensures that the GNB-MobilityInformation IE is recognized as part of the RAN CONFIGURATION UPDATE message. The criticality is set to ignore and presence to optional, as discussed.

-----------------End of the First Change---------------