3GPP TSG-RAN WG3 Meeting #129	R3-255543

# Bengaluru, India, 25th - 29th August 2025

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for BL CR 38.300 and 38.413] Applicability of A-IoT procedures for multiple devices

Document for:	Discussions & Approval

# 1	Introduction

This TP followsproposals 4 and 5 as discussed in R3-255541:

Proposal 4:	Remove the "page all" possibility in the Inventory Request procedure. If seen necessary, we could liaise to RAN2 and SA3 on that decision.

Proposal 5:	Introduce in the NGAP and stage 2 BL CR the possibility to indicate multiple individual device IDs in the Inventory Request message.

# 2	Text Proposal for BL CR 38.300 (R3-255072)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for either a single device or multiple devices or a group of devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the Expected D2R message size and may include Inventory Assistance Information e.g. Approximate number of Target A-IoT devices.

Editor’s Note 1:	Whether the Expected D2R message size is mandatory is FFS.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 2:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

# 3	Text Proposal for BL CR 38.413 (R3-255076)

-----------------Start of the Next Change-----------------

#### 9.3.3.xx5	A-IoT Device Identification Requested

This IE represents the Requested Identity of the A-IoT device(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Device Identification for Paging | M |  |  |  |
| >Single Device |  |  |  |  |
| >>Single Device Identity | M |  | OCTET STRING | Includes the AIoT device Identifier defined in TS 23.369 [z]. |
| >Multiple Devices |  |  |  |  |
| >>Device Identity List |  | <2..maxnoofDeviceIDs> |  |  |
| >>>Single Device Identity | M |  | OCTET STRING | Includes the AIoT device Identifier defined in TS 23.369 [z]. |
| >Group Devices |  |  |  |  |
| >>Group Device Identity | M |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDeviceIDs | Maximum no. of Device identities within an Inventory Request. Value is 65535. |

-----------------Start of the Next Change-----------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofCellsUEMovingTrajectory,

maxnoofDevices,

maxnoofDeviceIDs,

maxnoofDRBs,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

AIoT-DeviceIdentificationRequested ::= CHOICE {

singleDevice					OCTET STRING,

multipleDevices					A-IoT-MultipleDeviceIDs,

groupDevices					FFS,

choice-Extensions		ProtocolIE-SingleContainer { { AIoT-DeviceIdentificationRequested-ExtIEs} }

}

AIoT-DeviceIdentificationRequested-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

A-IoT-MultipleDeviceIDs ::= SEQUENCE (SIZE(2..maxnoofDeviceIDs)) OF OCTET STRING

-----------------Start of the Next Change-----------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofReaders								INTEGER ::= 65535

maxnoofDevices								INTEGER ::= 65535

maxnoofDeviceIDs							INTEGER ::= 65535

maxnoofAIoTAreas							INTEGER ::= 256