3GPP TSG-RAN WG3 Meeting #129	R3-255567

Bangalore, India, 25 – 29 August 2025

Agenda Item:	18.2

Source:	Ericsson

Title:	Discussion on forwarding of paging capabilities

Document for:	Discussion, other

# 1	Introduction

This paper discusses the question raised by SA2 in their LS [1] on paging capability loss issue.

SA2 thanks RAN2 for this LS and their earlier LS in R2-2501388=S2-2504576. In the latest LS, RAN 2 state that: In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”. The 5GS design means that the UE Radio Capability for Paging is loaded into the AMF only in the cell where the UE attaches (does “initial registration”) or first moves from LTE to NR. After that the UE can move throughout the whole PLMN and normally there is no event that triggers any update of the UE Radio Capability for Paging information stored in the AMF. Hence the “certain area” mentioned by RAN 2 is the whole NR coverage of that PLMN. It is essential that operators are able to use RAN equipment from different vendors and that competition between those RAN vendors is not inhibited by RAN 2 requiring all RAN vendors to upgrade their equipment before new UE functionality can be used with the first RAN vendor to develop a solution. Hence this RAN2 solution is highly undesirable compared to what was proposed in RAN2’s earlier LS in R2-2501388=S2-2504576. SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem. For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF. 2. Actions: To RAN3, RAN2 SA2 asks RAN3 and RAN2 to provide views on the potential solution described above.

# 2	Discussion

RAN2 have previously agreed to introduce a new Rel-19 UE radio paging capability container [2] as mentioned in the previous LS [2] to SA2 and RAN3:

For UE Radio Paging Capabilities introduced up to Rel-18, RAN2 agreed not to introduce any standard solution, i.e., rely on proper network implementation, as follows. - We will not change legacy UE capabilities and rely on network implementation To address the issue for UE Radio Paging Capabilities introduced in Rel-19 onwards, RAN2 made the following agreement: - Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for gNB that support this Rel-19 “container” In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

The new Rel-19 Container will be included in the RRC UERadioPagingInformation message as defined in TS 38.331 (green-marked in the table below).

9.3.1.68             UE Radio Capability for Paging

This IE contains paging specific UE Radio Capability information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Radio Capability for Paging of NR | O |  | OCTET STRING | Includes the UERadioPagingInformation message as defined in TS 38.331 [18]. |
| UE Radio Capability for Paging of E-UTRA | O |  | OCTET STRING | Includes the UERadioPagingInformation message as defined in TS 36.331 [21]. |
| UE Radio Capability for Paging of NB-IoT | O |  | OCTET STRING | Includes the UERadioPagingInformation-NB message as defined in TS 36.331 [21]. |

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability ::=            SEQUENCE {

…

UE-NR-Capability-v1830 ::=               SEQUENCE {

sib19-Support-r18                        ENUMERATED {supported}                 OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v19xy       OPTIONAL

}

UE-NR-Capability-v19xy ::=      SEQUENCE {

ue-RadioPagingInfo-r19          OCTET STRING (CONTAINING UE-RadioPagingInfo-r19) OPTIONAL,

nonCriticalExtension         SEQUENCE{}                                          OPTIONAL

}

The IE UE-RadioPagingInfo contains UE capability information needed for paging.

UE-RadioPagingInfo-r17 ::=            SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

...

}

UE-RadioPagingInfo-r19 ::=            SEQUENCE {

newPagingCapabilitField-r19          ENUMERATED {ffs}                                        OPTIONAL,

...

}

This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs.

UERadioPagingInformation ::= SEQUENCE {

…

-- New UE Radio Paging Capabilities shall be introduced in IE UE-RadioPagingInfo-r19.

UERadioPagingInformation-v19xy-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r19       OCTET STRING (CONTAINING UE-RadioPagingInfo-r19)     OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                              OPTIONAL

}

UERadioPagingInformation field descriptions

ue-RadioPagingInfo The field is used to transfer UE capability information used for paging. The gNB generates the ue-RadioPagingInfo-r17 and the contained UE capability information is absent when not supported by the UE. The content of ue-RadioPagingInfo-r19 is generated by the UE.

The UE creates the Rel-19 container ue-RadioPagingInfo-r19 IE (including Rel-19 and later UE paging capabilities) and the gNB copies that container received in the UECapabilityInformation message into the UE-RadioPagingInfo IE to the CN.

The problem from SA2’s point of view, is that there can be inter-operability problem in case the UE performs initial registration with a (legacy) gNB that does not forward e.g., the PEI/LP-WUS paging capability (e.g. gNB does not support PEI/LP-WUS) and the UE is later served by a gNB supporting PEI/LP-WUS. In such case, the PEI/LP-WUS paging capabilities are not stored in the CN, and the CN does not provide the PEI/LP-WUS paging capabilities to the gNB supporting PEI/LP-WUS, while the UE expects the gNB to use PEI/LP-WUS for paging.

Observation 1: There can be inter-operability problem in case the Rel-19 UE performs initial registration in a legacy gNB then later is served by an upgraded gNB, where the forwarding of paging capabilities from legacy gNB to AMF does not provide the full UE paging capabilities.

SA2 in the LS [1] states that some operators challenged the RAN2 proposed solution relying on network implementation to solve the issues of mixed deployments with different releases.

Observation 2: SA2 challenges the RAN2 solution relying on network implementation to solve the issues of mixed deployments with different gNB releases.

From SA2’s LS [1], the gNB should instead check if the paging capabilities reported by the AMF during the Initial UE Context Request message are correctly including all UE’s supported paging capabilities that can be derived from the UE Radio Capabilities. In NGAP, the Initial UE Context Request message (38.413 section 9.2.2.1) includes:

- UE Radio Capability for Paging (38.413 section 9.3.1.68), i.e. the UE paging capabilities (see 38.331 UERadioPagingInformation IE).

Upon receiving the IE from AMF, the gNB can use the UE Radio Access Capabilities from the UE to verify if the UE Radio Capability for Paging contains all the proper paging capabilities for a function to work. (Note: the verification can be done even in case the RRC_INACTIVE feature is not used and no need for assistance information from the CN). If a paging capability information is missing from the RRC container IE stored in AMF, the NG-RAN will, if supported, reconstruct and upload the full UE Radio Capability for Paging to AMF, so that AMF overwrites the previous IE with the new full one.

Observation 3: The NG-RAN can use the UE Radio Access Capabilities to verify if the UE Radio Capability for Paging stored at AMF and signalled Initial UE Context Request message contains all the UE’s necessary paging capabilities. If an omission is detected for a function to work properly, the NG-RAN reconstruct and uploads, if supported, the newest URCP to AMF.

Note that the above solution corresponds fully to the spirit of the clarification LS from SA2 sent in [3], which is already describing the solution, and for which NGAP CRs capturing corresponding gNB behaviour since Rel-16 have been submitted in AI 8.1.

Observation 4: The SA2 issue is solved by agreeing to the NGAP CRs (from Rel-16) submitted in AI 8.1 that are capturing the NG-RAN behaviour as described in Observation 3.

Hence, it is proposed to reply to SA2 that for functions (e.g., RedCap, LP-WUS) relying on having all UE Radio Capability for Paging to work properly, RAN3 has added necessary text description in NGAP from Rel-16.

Proposal 1: Reply to SA2 that RAN3 has already agreed to capture in NGAP the gNB behaviour that RAN node shall, if supported, verify and upload a full UE Radio Capability for Paging as a single IE if it found out that the received UE Radio Capability for Paging from AMF during Initial Context Request Setup does not include the necessary paging information for a function to work properly

Also, this solution should not prevent from having an implementation-based solution where all gNBs support the forwarding of the Rel-19 container. A draft LS reply to SA2 is provided in annex of this paper.

# 3	Conclusion

Observation 1: There can be inter-operability problem in case the Rel-19 UE performs initial registration in a legacy gNB then later is served by an upgraded gNB, where the forwarding of paging capabilities from legacy gNB to AMF does not provide the full UE paging capabilities.

Observation 2: SA2 challenges the RAN2 solution relying on network implementation to solve the issues of mixed deployments with different gNB releases.

Observation 3: The NG-RAN can use the UE Radio Access Capabilities to verify if the UE Radio Capability for Paging stored at AMF and signalled Initial UE Context Request message contains all the UE’s necessary paging capabilities. If an omission is detected for a function to work properly, the NG-RAN reconstruct and uploads, if supported, the newest URCP to AMF.

Observation 4: The SA2 issue is solved by agreeing to the NGAP CRs (from Rel-16) submitted in AI 8.1 that are capturing the NG-RAN behaviour as described in Observation 3.

Proposal 1: Reply to SA2 that RAN3 has already agreed to capture in NGAP the gNB behaviour that RAN node shall, if supported, verify and upload a full UE Radio Capability for Paging as a single IE if it found out that the received UE Radio Capability for Paging from AMF during Initial Context Request Setup does not include the necessary paging information for a function to work properly.

# 4	References

[1] R3-255026, Response to Reply LS on paging capability loss issue, SA2 (Vodafone)

[2] R3-253012, Reply LS on paging capability loss issue, RAN2

[3] R3-255025, Reply LS on Handling of UE Radio Capability for Paging, SA2

# 5	Annex, Reply LS to SA2

3GPP TSG RAN WG3 Meeting #129	R3-25xxxx

Bengaluru, India, 25 - 29 August, 2025

# Title:	[DRAFT] Reply LS on Response to Reply LS on paging capability loss issue

# Response to:	Response to Reply LS on paging capability loss issue  (R3-255026/ S2-2506083)

# Release:	Release 19

# Work Item:	NR_LPWUS-Core, TEI19

Source:	Ericsson (to be RAN3)

To:	SA2

Cc:	RAN2, CT1, TSG RAN

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 thanks SA2 for their LS response.

RAN3 would like to inform SA2, as they may have already noticed from the ongoing LS exchange in R3-255025/S2-2506082 between SA2 and RAN3, that RAN3 has agreed to capture in NGAP since Rel-16 the description where an NG-RAN node shall, if supported, verify and provide the UE Radio Capability for Paging as a single IE if it found out that the received UE Radio Capability for Paging from AMF during Initial Context Request Setup does not include the necessary paging information for a function to work properly.

RAN3 considers that this standardization solution can address the concern of operators in SA2 to enable mixed deployments in case the network implementation solution, e.g., relying on homogeneous gNB support forwarding the new UE capabilities, is not sufficient.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into account.

3. Date of Next TSG RAN3 Meetings:

TSG-RAN3 Meeting #129bis	13-17 October 2025	Pragues, CZ

TSG-RAN3 Meeting #130	17-21 November 2025	Dallas, Texas