3GPP TSG-RAN WG3 Meeting #129	R3-255590

# Bengaluru, India, 25th – 29th Aug, 2025

Title: 	Discussion on update/cancel procedures for Predicted CCO issue vs. Detected CCO issue

Source: 	Jio Platforms

Agenda item:	11.3

Document Type:	Discussion and decision

# 1	Introduction

During last RAN3#128 meeting the following was agreed:

Current mechanism enables the update of predicted CCO issue and/or future CCO state. A new detected CCO issue/a new predicted CCO issue for the same affected cells and beams after a predicted CCO issue will cancel the prediction.

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

This discussion document examines scenarios requiring modification (update or cancellation) of previously communicated CCO-related predictions, building upon RAN3#128 agreements. We analyze cases where an existing predicted CCO issue needs revision due to:

- detection of an actual CCO problem in previously predicted and indicated cells/beams,

- receipt of a new conflicting prediction for the same resources,

The document explores how these modifications should be handled through existing F1 and Xn interfaces without introducing new IEs, leveraging the agreed identification method through affected cells and beams. Particular focus is given to temporal aspects - how quickly predictions should be updated when new information becomes available, and how to handle overlapping prediction time windows. We also consider edge cases where multiple predictions for the same resources arrive in rapid succession, proposing methods to determine which prediction takes precedence and priority.

# 2	Detected vs. Predicted CCO Issues

##### 2.1 Interaction Between Detected and Predicted CCO Issues

Let us consider a case where the AI/ML function in the gNB-CU has previously issued a predicted CCO assistance message, which includes the predicted CCO issue type, list of affected cells or beams, and a forecasted time window, as specified in R3-253988 for the BL CR to TS 38.473. This prediction has already been transmitted to the gNB-DU(s) via F1 and optionally to neighboring gNBs via Xn.

If a detected CCO issue now arises that impacts at least one of the same cell/SSBs covered in the prediction, then:

- In cases where a detected issue overlaps with the scope of a previous prediction, the real-time issue takes precedence. Detected events require immediate resolution to preserve user experience and prevent performance degradation. This priority should be explicitly acknowledged in the specification.

Proposal 1: Prioritize detected CCO issues against predicted CCO issues.

- The resolution of the detected issue will likely cause a change in the local CCO state of one or more involved cells/SSBs. As a result, the validity of any prior prediction becomes questionable or invalid due to the network topology or parameter rearrangement.

Observation 1: the validity of any prior prediction becomes questionable or invalid under the scope of a prioritized resolved detected CCO issue

Proposal 2: Detected CCO issues affecting any subset of cells/SSBs covered by a prior prediction shall supersede the predicted event in priority and trigger immediate mitigation

There are two post-resolution actions to be taken after the detected issue is resolved. In a general approach the AI/ML model should re-evaluate the updated network state and generate a new prediction, if applicable. Two clear outcomes may follow:

Case I (Prediction Cancellation):

If, due to the state change following the detected issue resolution, the model finds that no predicted issue is likely to occur for the previously listed cells/SSBs, the previous prediction shall be explicitly cancelled via appropriate signaling on F1 and Xn.

Proposal 3: After mitigation, the gNB-CU AI/ML model shall regenerate predictive assistance reflecting the post-resolution state. If no further CCO issues are predicted, the prior prediction shall be cancelled. Otherwise, it shall be updated with refined parameters.

Case II (Prediction Update):

If the re-analysis by the AI/ML model confirms that some (or all) of the previously flagged cells/SSBs are still vulnerable to CCO issues, then a new prediction shall be issued that replaces the earlier one. This allows the DUs and neighboring gNBs to operate with up-to-date assistance information.

Proposal 4: After mitigation, the gNB-CU AI/ML model shall regenerate predictive assistance reflecting the post-resolution state. If  further CCO issues are predicted, the prior prediction shall be updated with refined parameters.

Proposal 5: A revision identifier or version tag should be considered for prediction messages to facilitate consistent replacement logic and avoid duplicate application of conflicting predictions across F1 and Xn

Proposal 6: It is recommended to introduce an optional field indicating the “originating prediction context” in any update/cancelation message to ensure consistent state tracking between CU and DU nodes, especially when multiple predictions may coexist temporally for different beams.

Proposal 7: The AI/ML function may optionally maintain a short-term history of prediction-to-detection transitions. This can assist in future model refinements, increasing prediction reliability by capturing observed false positives/negatives in forecasted CCO issues.

##### 2.2 Interaction Between Detected and Predicted CCO Issues with time reference

Current F1/Xn signalling does not distinguish between prediction time window (τₚ) and detection reaction time (τd). Such decoupling is critical. If τₚ ≫ τd, the prior prediction becomes obsolete, justifying cancellation or replacement. Conversely, if τₚ ≈ τ_d, the predictive model might still hold partial validity. To maintain transparency and consistency across the distributed RAN entities (CU, DU, neighbouring gNB), any updates to predicted CCO data must carry timestamp metadata to facilitate reconciliation of detected vs. predicted validity scopes.

After a detected CCO event is resolved, the AI/ML model shall re-compute the CCO landscape. Two non-mutually exclusive subcases are identified:

Subcase I – Full Resolution Supersedes the Prediction:

Post-resolution inference indicates no further CCO risks for any of the originally predicted SSBs or cells. In this case, cancellation of the prior prediction must be signaled across F1 and Xn to eliminate outdated optimization intents and avoid conflicting or redundant behavior in the DUs and neighbors.

Subcase II – Partial Persistence of the Predicted Condition:

The model concludes that despite the detected issue being resolved, some of the cells/SSBs from the prior prediction remain forecasted to exhibit CCO risk. This mandates replacement of the previous prediction with a newly scoped prediction, refreshed in terms of scope, severity, or timing.

Following the 2.1 clause, the same analysis holds but with a new time parameter. If a detected CCO issue overlaps with a prior predicted CCO assistance information affecting at least one common resource (cell/SSB), the detected event shall override the prediction and be mitigated with highest priority. Furthermore once the detected issue is resolved, the AI/ML model in the gNB-CU shall re-evaluate the network and issue updated predictions. However introducing time validity fields in prediction F1/Xn signaling to explicitly separate the validity window of the prediction (τₚ) from the event detection resolution time (τd) will eventually help determine whether a detected event renders the prediction obsolete or only partially invalid.

Observation 2: Time validity fields in prediction F1/Xn signaling would support determine whether a detected event renders the prediction obsolete or only partially invalid as following:

- If no longer applicable, the previous prediction must be cancelled.

- If partial overlap or new risks are identified, the previous prediction shall be replaced

- If τₚ ≫ τd, the prior prediction becomes obsolete, justifying cancellation or replacement.

Proposal 8: RAN3 to consider introducing time validity fields in prediction F1/Xn signaling to explicitly separate the validity window of the prediction (τₚ) from the event detection resolution time (τd).

Proposal 9: RAN3 to consider including a timestamp and unique prediction reference ID in all prediction-related signaling (F1/Xn), allowing systems to:

- Recognize whether a detected issue is inside the active time window of a prediction.

- Associate modifications or cancellations with specific previously signaled predictions

##### 2.3 Detected and Predicted CCO Issues Overlapping

Consider a scenario where the AI/ML model running at the gNB-CU forecasts that Beam A will suffer a coverage gap within a future interval [T1,T2], prompting a proactive prediction message to the gNB-DU. Before T1 arrives, a real-time anomaly is detected in Beam B—a neighbor beam under the same or adjacent gNB—triggering an urgent CCO response. The beams may partially overlap in user population, spatial footprint, interference domains, or antenna array resources. However, NG-RAN lacks an explicit conflict resolution policy in such cases, leaving several open questions:

- Should Beam B’s real-time fix invalidate Beam A’s predicted optimization, even if the predictions are unrelated?

- Does the resolution action for Beam B indirectly mitigate or worsen the issue predicted for Beam A?

- Can the network distinguish whether the two events are correlated, co-causal, or independent?

Such ambiguity may result in contradictory optimization actions if the AI/ML engine lacks global context or prioritization logic. More critically, overlapping beam conflicts may lead to resource contention, non-deterministic scheduling, or counterproductive compensations, especially in tightly packed antenna arrays with beam-based mobility.

Observation 3: Conflicts may arise when a detected issue overlaps spatially or temporally with a predicted issue that affects another beam or SSB. In such cases, a clear policy is required. We propose a hybrid conflict resolution approach that includes:

- Temporal precedence (current issues override future ones),

- Impact weight scoring (e.g., based on predicted UE load or KPI degradation),

- Partial overlap handling (allowing selective replacement or adjustment of predictions rather than wholesale cancellation).

Observation 4: Absence of a prioritization rule may result in suboptimal AI/ML model behavior, causing oscillations or conflicting optimization actions across beams or neighbor gNBs.

After resolving the detected issue, the AI/ML model should recompute its inference considering the new network topology and provide updated assistance information. The outcome may fall into two categories:

Case I: Prediction Cancellation

Post-resolution, if the model determines that no CCO issues are expected for any previously forecasted cell/SSB during the original prediction window, the previously signaled prediction should be explicitly withdrawn via F1/Xn.

Case II: Prediction Replacement

If the model still anticipates a CCO issue affecting all or part of the previously identified cells/SSBs, it must issue a revised prediction reflecting the updated context, replacing the original forecast.

Proposal 10: RAN3 is requested to define a conflict resolution policy (e.g., priority matrix or scheduling algorithm) for overlapping or concurrent detected/predicted issues across different SSBs or neighbor gNBs.

# 3	Proposals for F1/Xn Update/Cancel CCO Issues

To enable the replacement of previously shared AI/ML-driven predicted CCO assistance information—particularly in scenarios requiring cancellation of outdated predictions and provision of updated data—a clear signaling framework is essential across both the F1 and Xn interfaces.

Signaling over the F1 Interface

As discussed during RAN3#128, the method for referencing previously communicated CCO-related prediction data relies on identifying the impacted cells and associated SSBs. This approach avoids the necessity of introducing a distinct prediction identifier. To facilitate proper invalidation of outdated prediction information, it is suggested to define a new enumeration value, such as ‘cancel’, within the Predicted CCO Issue IE. This value would reside within the broader context of the Predicted CCO Assistance Information structure. Once cancellation is indicated, an updated prediction—based on the latest network context—can be conveyed via a new instance of the F1AP gNB-CU CONFIGURATION UPDATE message, which would deliver the revised AI/ML-based prediction parameters to the relevant gNB-DU. This approach ensures the consistency and integrity of CCO prediction data within evolving AI-assisted RAN optimization workflows.

Proposal 11: RAN3 to support the introduction of a “cancel” codepoint in the Predicted CCO Issue IE within F1AP to enable explicit withdrawal of prior prediction content.

Signaling over the Xn Interface

For the Xn interface, previously distributed predicted coverage change information is signaled via the Future Coverage Modification List IE, which contains the set of forecasted CCO states, their corresponding coverage modification causes, and their respective activation timings. To facilitate cancellation:

It is proposed that entire instances of the Future Coverage Modification List be removed from the neighbor node’s configuration upon invalidation of the prediction. Notably, Xn signaling currently permits multiple CCO state entries for the same cell—each associated with distinct causes and activation times—which introduces complexity in identifying which prediction is being invalidated.

Observation 5: Effective cancellation over Xn requires not only the list of cells and SSBs but also correlation with the original predicted modification cause and scheduled activation time.

To ensure robust and deterministic cancellation, extend the Predicted Coverage Modification Cause IE with a “cancel” value and pair it with the proposed To Be Cancelled List IE, ensuring that cancellation intent is explicit and contextually traceable even under concurrent prediction threads.

Proposal 12: RAN3 to agree on adoption of extending the Predicted Coverage Modification Cause IE with a “cancel” value.

# 3	Conclusion

Follows the summary of Observations and Proposals:

Proposal 1: Prioritize detected CCO issues against predicted CCO issues.

Observation 1: the validity of any prior prediction becomes questionable or invalid under the scope of a prioritized resolved detected CCO issue

Proposal 2: Detected CCO issues affecting any subset of cells/SSBs covered by a prior prediction shall supersede the predicted event in priority and trigger immediate mitigation

Proposal 3: After mitigation, the gNB-CU AI/ML model shall regenerate predictive assistance reflecting the post-resolution state. If no further CCO issues are predicted, the prior prediction shall be cancelled. Otherwise, it shall be updated with refined parameters.

Proposal 4: After mitigation, the gNB-CU AI/ML model shall regenerate predictive assistance reflecting the post-resolution state. If  further CCO issues are predicted, the prior prediction shall be updated with refined parameters.

Proposal 5: A revision identifier or version tag should be considered for prediction messages to facilitate consistent replacement logic and avoid duplicate application of conflicting predictions across F1 and Xn

Proposal 6: It is recommended to introduce an optional field indicating the “originating prediction context” in any update/cancelation message to ensure consistent state tracking between CU and DU nodes, especially when multiple predictions may coexist temporally for different beams.

Proposal 7: The AI/ML function may optionally maintain a short-term history of prediction-to-detection transitions. This can assist in future model refinements, increasing prediction reliability by capturing observed false positives/negatives in forecasted CCO issues.

Observation 2: Time validity fields in prediction F1/Xn signaling would support determine whether a detected event renders the prediction obsolete or only partially invalid as following:

- If no longer applicable, the previous prediction must be cancelled.

- If partial overlap or new risks are identified, the previous prediction shall be replaced

- If τₚ ≫ τd, the prior prediction becomes obsolete, justifying cancellation or replacement.

Proposal 8: RAN3 to consider introducing time validity fields in prediction F1/Xn signaling to explicitly separate the validity window of the prediction (τₚ) from the event detection resolution time (τd).

Proposal 9: RAN3 to consider including a timestamp and unique prediction reference ID in all prediction-related signaling (F1/Xn), allowing systems to:

- Recognize whether a detected issue is inside the active time window of a prediction.

- Associate modifications or cancellations with specific previously signaled predictions

Observation 3: Conflicts may arise when a detected issue overlaps spatially or temporally with a predicted issue that affects another beam or SSB. In such cases, a clear policy is required. We propose a hybrid conflict resolution approach that includes:

- Temporal precedence (current issues override future ones),

- Impact weight scoring (e.g., based on predicted UE load or KPI degradation),

- Partial overlap handling (allowing selective replacement or adjustment of predictions rather than wholesale cancellation).

Observation 4: Absence of a prioritization rule may result in suboptimal AI/ML model behavior, causing oscillations or conflicting optimization actions across beams or neighbor gNBs.

Proposal 10: RAN3 is requested to define a conflict resolution policy (e.g., priority matrix or scheduling algorithm) for overlapping or concurrent detected/predicted issues across different SSBs or neighbor gNBs.

Proposal 11: RAN3 to support the introduction of a “cancel” codepoint in the Predicted CCO Issue IE within F1AP to enable explicit withdrawal of prior prediction content.

Observation 5: Effective cancellation over Xn requires not only the list of cells and SSBs but also correlation with the original predicted modification cause and scheduled activation time.

Proposal 12: RAN3 to agree on adoption of extending the Predicted Coverage Modification Cause IE with a “cancel” value.