3GPP TSG-RAN WG3 Meeting #129	R3-255594

# Bangaluru, India, 25 – 29 August 2025

Agenda Item:	10.2

Source:	Ericsson, Huawei, Jio Platforms

Title:	(TP for BL CR to 38.473 for SON) Signaling of MRO for LTM between target DU and source DU

Document for:	Agreement

# TP for SON BL CR for TS 38.473

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

### 8.11.1	Access and Mobility Indication

#### 8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the BFR SSB Index IE within the MRO for LTM Information IE, the gNB-DU shall, if supported, consider that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the Target SSB Index after Cell Switch Failure IE within the MRO for LTM Information IE, the gNB-DU shall, if supported, consider that an LTM Cell Switch Failure due to Wrong Beam has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the TA Value at re-establishment IE within the MRO for LTM Information IE, the gNB-DU may take it into account for optimisation of Timing Advance related parameters for the UE identified by the gNB-DU UE F1AP ID IE.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

[snip]

### 8.11.2	DU-CU Access and Mobility Indication

#### 8.11.2.1	General

This procedure is initiated by the gNB-DU to send the Access and Mobility related Information to the gNB-CU.

The procedure uses non-UE-associated signalling.

#### 8.11.2.2	Successful Operation

Figure 8.11.2.2-1: DU-CU Access and Mobility Indication procedure. Successful operation

The DU-CU Access and Mobility Indication procedure is initiated by DU-CU ACCESS AND MOBILITY INDICATION message sent from the gNB-DU to the gNB-CU.

If the DU-CU ACCESS AND MOBILITY INDICATION message contains the DL LBT Failure Information List IE, the gNB-CU shall take it into account for optimisation of mobility parameters.

#### 8.11.2.3	Abnormal Conditions

Not applicable.

[snip]

# 9	Elements for F1AP Communication

[snip]

## 9.2	Message Functional Definition and Content

[snip]

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU. This IE is included in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| MRO for LTM Information | O |  |  |  | YES | ignore |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| >BFR SSB Index | O |  | INTEGER (0..63) | SSB Index of the recovery beam used at successful Beam Failure Recovery. | - |  |
| >Target SSB Index after Cell Switch Failure | O |  | INTEGER (0..63) | SSB Index of the re-established or recovery beam after LTM Cell Switch Failure due to Wrong Beam. | - |  |
| >TA Value at re-establishment | O |  | INTEGER (0..4095) | Indicates the TA value, as defined in TS 38.213 [31], used at successful Random Access after a Cell Switch failure in same beam. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

#### 9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| > DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| MRO for LTM Information | O |  |  |  | YES | ignore |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >BFR SSB Index | O |  | INTEGER (0..63) | SSB Index of the recovery beam used at successful Beam Failure Recovery. | - |  |
| >Target SSB Index after Cell Switch Failure | O |  | INTEGER (0..63) | SSB Index of the re-established or recovery beam after LTM Cell Switch Failure due to Wrong Beam. | - |  |
| >TA Value at re-establishment | O |  | INTEGER (0..4095) | Indicates the TA value, as defined in TS 38.213 [31], used at successful Random Access after a Cell Switch failure in same beam. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |

<<<<<<<<<<<<<<<<<<<< End of 1st Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

-- TEXT OMITTED –

PLMNIndexNR,

MROForLTM-Information

FROM F1AP-IEs

-- TEXT OMITTED –

id-PLMNIndexNRAssistanceInfoForNetShar,

id-MROForLTM-Information

,

maxCellingNBDU,

-- TEXT OMITTED –

-- **************************************************************

--

#### --  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReportInformationList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional }|

{ ID id-MROForLTM-Information			CRITICALITY ignore 	TYPE MROForLTM-Information  		PRESENCE optional },

...

}

-- TEXT OMITTED –

-- **************************************************************

--

#### --  DU-CU Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU Access And Mobility Indication

--

-- **************************************************************

DUCUAccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DUCUAccessAndMobilityIndicationIEs} },

...

}

DUCUAccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-DLLBTFailureInformationList				CRITICALITY ignore	TYPE DLLBTFailureInformationList	PRESENCE optional}|

{ ID id-MROForLTM-Information		CRITICALITY ignore	TYPE MROForLTM-Information	PRESENCE optional },

...

}

-- TEXT OMITTED –

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

-- TEXT OMITTED –

CSIResourceConfiguration ::= SEQUENCE  {

cSIResourceConfigToAddModList 			OCTET STRING		OPTIONAL,

cSIResourceConfigToReleaseList			OCTET STRING		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { CSIResourceConfiguration-ExtIEs} }	OPTIONAL

}

CSIResourceConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

-- TEXT OMITTED –

DLLBTFailureInformationRequest 	::= ENUMERATED {inquiry, ...}

DLLBTFailureInformationList		::= SEQUENCE (SIZE(1.. maxnoofLBTFailureInformation)) OF DLLBTFailureInformationList-Item

DLLBTFailureInformationList-Item::= SEQUENCE {

uEAssistantIdentifier		GNB-CU-UE-F1AP-ID,

numberOfDLLBTFailures		INTEGER (1..1000,...)							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { DLLBTFailureInformationList-Item-ExtIEs} }	OPTIONAL,

...

}

DLLBTFailureInformationList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- E

-- TEXT OMITTED –

GNB-DU-UE-F1AP-ID		::= INTEGER (0..4294967295)

GNB-CUorDU-UE-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-ID			::= INTEGER (0..68719476735)

-- TEXT OMITTED –

MobilityInitiation-AssistanceInfo	::= SEQUENCE {

servingCellMeasurements				ServingCellMeasurements,

candidateCellwithMeasurementsList	CandidateCellwithMeasurementsList,

iE-Extensions						ProtocolExtensionContainer { { MobilityInitiation-AssistanceInfo-ExtIEs } }	OPTIONAL,

...

}

MobilityInitiation-AssistanceInfo-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MROForLTM-Information ::= SEQUENCE {

gNB-CUorDU-UE-F1AP-ID		GNB-CUorDU-UE-F1AP-ID,

bFR-SSB-Index				SSB-Index						OPTIONAL,

target-SSB-Index-Failure	SSB-Index						OPTIONAL,

tA-Value-reestablish		TAValue							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MROForLTM-Information-ExtIEs } } OPTIONAL,

...

}

MROForLTM-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- N

-- TEXT OMITTED –

id-PLMNIndexNRAssistanceInfoForNetShar						ProtocolIE-ID ::= 861

id-rLFReportFailureType								ProtocolIE-ID ::= 999 --to be assigned by MCC

id-MROForLTM-Information					ProtocolIE-ID ::= 998 --to be assigned by MCC

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>