3GPP TSG-RAN WG3 #129	R3-255597

# Bengaluru, India, 25-29 August 2025

Agenda Item:	10.3.1

Source:	Ericsson

Title: 	SON/MDT enhancements for NTN

Document for:	Discussion

# 1	Introduction

RAN plenary decided during RAN#103 to support further data collection for Self-organizing Networks (SON). One of the topics that was endorsed was the SON enhancements for non-terrestrial networks.

This paper builds on the previous submissions by discussing SON for NTN on a range of different topics such as RLF enhancements and MDT area scope related enhancements.

# 2	Discussion

## 2.1 MDT related enhancements for NTN

An agreement regarding logged MDT Area Scope was reached in RAN3#128 as follows:

To introduce a geographical area scope for NTN MDT including either reference location/radius or a polygon-based area for NG and Xn. FFS on whether to add the geographical area scope inside or outside the existing area scope IE. FFS on whether the geographical area scope applies to both logged and immediate MDT or only logged MDT.

Regarding on the first FFS, we think that when checking geographical area scope, the PLMN should also be checked as the MDT data collected on one operator’s network may not be useful for another operator. The logged MDT configuration consists of an optional MDT PLMN list, indicating the PLMNs where the measurement collection and log reporting is allowed. However, up to operator's policy, the MDT measurements and logging may limit to a subset of the MDT PLMN list.

In the current area scope selection for logged MDT, the UE can collect MDT measurements in a subset of the MDT PLMN list through the existing area scope choices that include e.g. CGI-based, TAC-based, and so on.

Therefore, adding the geographic area scope outside of the existing area scope IE enables logging the measurements within a subset of the allowed MDT PLMN list, based on operator’s policy.

Proposal 1: Add the geographical area scope outside of the existing area scope IE.

Regarding the send FFS, as the purpose of the MDT area scope enhancement is to enable collection of MDT measurement in smaller geographical areas and/or enabling finer resolution when defining the geographical areas, the geographical area scope should also be used for immediate MDT for the same purpose.

On the other hand, the MDT area scope enhancement for logged MDT over NGAP can be applied to immediate MDT using the same IE without any extra specification effort.

Proposal 2: The same MDT Area Scope enhancement applies to both logged and immediate MDT over NGAP.

The Area Scope of MDT needs specific enhancements to indicate to the UE, that the MDT has to be collected in NTN networks. To do so, we need to add an NTN wide indicator in the Area Scope of MDT. This will allow the network to indicate to the UE to collect MDT only in NTN. There is currently no method in the specifications to indicate that MDT must be collected in a specific type of network, NTN in this case. This will allow an operator to build a coverage map of NTN network and detect problems specific to that type of RAT.

Other options such as indicating the CGI of the cell can be used, however they have an upper limit on the number of identifiers and as such is unsuitable for this scenario.

Proposal 3: Include NTN wide indication in the MDT Area Scope to enable MDT collection.

## 2.2 RACH-less HO related enhancements for NTN

During the RAN3 #127bis, MRO for RACH-less handover for NTN was discussed and the following two scenarios were reached:

Divide the MRO for RACH-less handover case into two scenarios: - RACH-less handover was attempted but failed; - UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold

In RAN2 #128 MRO for LTM discussion, it was agreed that “No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues.” Similar with LTM, the NW can implicitly figure out whether the HOF pertains to a RACH-less failure or a RACH-based failure for NTN. Therefore, for scenario 1, the NW can understand if RACH-less handover was attempted but failed. Unlike LTM, the timing advance for RACH-less HO in NTN is always configured with 0 or the same timing advance with source cell. When scenario1 happens, NW can analyze that the SSB for dynamic grant configuration or the SSB and related configuration for CG may not be optimized.

Scenario 2 is the fallback scenario, when uplink grant for RACH-less handover is configured, the UE will initiate RACH-less HO or Random-Access procedure for HO by checking if there is SSB corresponding to the configured uplink grant with SS-RSRP above a RSRP threshold is available. Therefore, as NW knows all the CG RACH-less configuration, when there is RA information reported in the RLF report, NW can figure out that RACH-less fallback happens for the failed handover. However, when RACH-less fallback successes, the NW may not be able to know if it is a RACH-less fallback or a RACH-based handover via the RA Report.

For the failure case, the NW can differentiate scenario 1 and 2. But the NW is lack of necessary information to accurately tune the RACH-less HO configuration, e.g. the information of the measured RSRP for the SSB beam(s) for the configured grant can be used for the NW to optimize the configured uplink grant for RACH-less handover configuration. Similar with the RACH optimization for 2-Step RA in Rel-17, dlPathlossRSRP IE was introduced which can be used by NW to tune the RSRP threshold for selection between 2-step RA type and 4-step RA type. It is therefore proposed to include the measured RSRP information in SON reports to help the NW to optimize the RSRP threshold configured for SSB selection for the CG RACH-less handover.

Proposal 4: UE logs the information of the measured RSRP for the SSB beam(s) corresponding to the configured grant for RACH-less HO.

## 2.3 UHI related enhancements for NTN

The final aspect we would like to introduce in the paper is the collection of UE history information (UHI) for NTN. UE history information is network side collection of history of cells that the UE has visited. Initially, this was used to detect and avoid ping-pong. However, the application of UHI has been expanded to other use-cases such as trajectory prediction and so on.

Upon successful HO of a UE, the network records the CGI, time stayed, cell type and HO cause value. This UHI needs to be enhanced to indicate to the network that the cell type the UE was in, was an NTN cell. The obvious way of doing this is to enhance the Cell Type IE in UHI to denote that the cell is an NTN cell (e.g. LEO, GEO), in addition to the legacy cell size.

Proposal 5: Add new NTN cell types (e.g. LEO, GEO) to Cell Type IE for indicating the NTN cells in UHI.

The cells in a non-terrestrial network may be moving as opposed to the generally static deployment of a terrestrial network. Thus, one of the new handovers causes that might occur is because the cell moved away from the UE, as opposed to the opposite in a terrestrial network. This needs to be added as a cause value in UHI. Additionally, satellite switch with re-sync is a special mobility procedure because both the PCI and the carrier frequency remains the same, in order to identify the satellite switch with re-sync mobility, it should also be considered in the enhancement of cause value in the UHI.

Proposal 6: Enhance Cause value to reflect cell mobility in a non-terrestrial network.

# 3	Summary

In this discussion paper, NTN SON/MDT enhancements were discussed, and the following observations and proposals were made:

Proposal 1: Add the geographical area scope outside of the existing area scope IE.

Proposal 2: The same MDT Area Scope enhancement applies to both logged and immediate MDT over NGAP.

Proposal 3: Include NTN wide indication in the MDT Area Scope to enable MDT collection.

Proposal 4: UE logs the information of the measured RSRP for the SSB beam(s) corresponding to the configured grant for RACH-less HO.

Proposal 5: Add new NTN cell types (e.g. LEO, GEO) to Cell Type IE for indicating the NTN cells in UHI.

Proposal 6: Enhance Cause value to reflect cell mobility in a non-terrestrial network.

# 4	References

- RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, RAN#102, Edinburgh, Scotland, December 2023.