3GPP TSG-RAN WG3 Meeting #129	             R3-255709

Bengaluru, India, Aug 25th- Aug 29th, 2025

Agenda Item:	16.2

Source:	CMCC

Title:	Leftover issues on Inventory and Command Procedure

Document for:	Discussion

# 1	Introduction

In this contribution, we will continue to solve all the leftover issues on Inventory and Command signaling and procedures between A-IoT RAN and A-IoT CN to support A-IoT functions.

# 2	Discussion

## 2.1	Inventory Operation

During the Inventory Request procedure, the Inventory Request Transfer IE providing the inventory request related information from the AIOTF to the NG-RAN node is carried in INVENTORY REQUEST message. Within the Inventory Request Transfer IE, the A-IoT Device Identification Requested, as excerpted below, is agreed to introduce representing the Requested Identity of the A-IoT device(s).

#### 9.3.3.xx5 A-IoT Device Identification Requested This IE represents the Requested Identity of the A-IoT device(s). IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Device Identification for Paging; M. >Single Device. >>Single Device Identity; M; OCTET STRING; Includes the AIoT device Identifier defined in TS 23.369 [z].. >Group Devices. >>Group Device Identity; M; FFS. >All Devices; NULL.

Within the A-IoT Device Identification Requested, the encoding of Group Device Identity is still undecided. In the TS 23.369, the filtering information is used to identify or filter multiple A-IoT Devices and constructed by one or multiple components (i.e. ID Type, PLMN Identifier, NID, third party identifier and Identification Information) of the A-IoT Device Permanent Identifier, this Group Device Identity refers to the filter information defined in SA2 also can be encoded as OCTET STRING.

Proposal 1: The IE type of the Group Device Identity within A-IoT Device Identification Requested IE can be OCTET STRING.

Besides, the Estimate of Expected D2R Message Size IE is already defined in Inventory Request Transfer IE, which used to indicate the size of the Inventory Response message from the A-IoT Device. Followed by the same logic defined for Estimate of Expected D2R Message Size IE in the Command Assistance Information IE, this IE is encoded as an INTEGER with byte unit. Since RAN2 already sent the following LS to RAN3 in last meeting to show their attitude on the presence of Estimate of Expected D2R Message Size IE:

RAN2 discussed segmentation of D2R messages and made the following assumption on inventory response message size. Agreements on segmentation 4 For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.. RAN2 would like to inform SA2 and RAN3 that RAN2 assumes that the inventory response size can always be provided by the core network to the reader.

It can be seen that RAN2 believes the message size information should be mandatory included within the Inventory Assistance Information IE. During the last meeting, RAN3 decided to check SA2’s progress. From the latest SA2 TS 23.369 as excerpted below, the size of the Inventory Response message from the A-IoT Device can be considered as the assistance information provided to NG-RAN node along with the service operation requests.

## 5.4 Assistance information provided to NG-RAN node The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests. For Inventory or Command service operation, following Inventory Assistance Information is included in the Inventory Request from AIOTF to NG-RAN: a) AIoT service type (e.g. Inventory, Command). b) Optionally, approximate number of AIoT devices based on AF request. c) Size of the Inventory Response message from the AIoT Device. d) Optionally, time interval for AIoT Device response aggregation used by the NG-RAN as specified in clause 5.9.

It is found that there is no optional presence defined in the bullet of Size of the Inventory Response message from the AIoT Device. Therefore, the Expected D2R Message Size IE should be mandatory included in Inventory Request Transfer IE.

Proposal 2: The IE type of Estimate of Expected D2R Message Size IE in Inventory Request Transfer IE can be INTEGER with byte unit.

Proposal 3: The Expected D2R Message Size IE should be mandatory included in Inventory Request Transfer IE.

In addition, SA2 has specified that the time interval for A-IoT device response aggregation, used by the NG-RAN, is optionally included in the INVENTORY REQUEST message from AIOTF to NG-RAN. The usage of time information and the method of transmitting the interval are described as follows:

The AIOTF determines the aggregation assistance information based on the request from the AF or local configuration, which includes: - Time interval: the fixed time interval for which NG-RAN collects multiple AIoT Devices’ operation responses before reporting the aggregated AIoT response to the AIOTF. The reporting based on time interval may potentially happen multiple times until the NG-RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to the NG-RAN that is equal or shorter than the time interval received from the AF. NOTE: Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval. If the AIOTF does not provide aggregation assistance information, the aggregation process in the NG-RAN may be determined by implementation.

This IE indicates a fixed time duration for which the NG-RAN collects multiple A-IoT Devices’ operation responses before reporting the aggregated A-IoT response to the AIOTF. As this time interval is already defined by SA2 as assistance information in the Inventory Request message, RAN3 needs to place this IE optionally included in Inventory Request Transfer IE.

Proposal 4: The IE indicating the time interval for A-IoT device response aggregation is optionally included in Inventory Request Transfer IE.

## 2.2	Command Operation

During the last meeting, companies still did not reach consensus on whether to introduce command type in Command Request Transfer IE. From our perspective, making the service type visible to the NG-RAN is necessary to enable scheduling optimizations over A-IoT Uu. With this information, the NG-RAN can apply service-specific resource allocation strategies. For a read operation, the NG-RAN shall allocate sufficient uplink resources to retrieve content from the A-IoT device. For write or disable operations, the NG-RAN shall allocate only the minimal uplink resources required for success or failure acknowledgment signalling. The AIOTF shall interpret the service type provided by the AF and convey the corresponding command service type to the NG-RAN in Command Request Transfer IE via the COMMAND REQUEST message.

Proposal 5: The service type indicator (Read/Write/Disable) needs to be included in Command Request Transfer IE.

## 2.3	AIOTF Identifier IE

Regard the detail encoding of the AIOTF Identifier IE, we think this IE can be referred to the NfInstanceId IE defined in TS 29.571. Correspondingly, the IE type of AIOTF Identifier IE can be OCTET STRING.

Type Name; Type Definition; Description. NfInstanceId; string; String uniquely representing a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 9562 [15]. The hexadecimal letters should be formatted as lower-case characters by the sender, and they shall be handled as case-insensitive by the receiver. Example: "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" (NOTE 3).

Proposal 6: The IE type of AIOTF Identifier IE can be OCTET STRING.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some proposals on the inventory and command procedure:

Proposal 1: The IE type of the Group Device Identity within A-IoT Device Identification Requested IE can be OCTET STRING.

Proposal 2: The IE type of Estimate of Expected D2R Message Size IE in Inventory Request Transfer IE can be INTEGER with byte unit.

Proposal 3: The Expected D2R Message Size IE should be mandatory included in Inventory Request Transfer IE.

Proposal 4: The IE indicating the time interval for A-IoT device response aggregation is optionally included in Inventory Request Transfer IE.

Proposal 5: The service type indicator (Read/Write/Disable) needs to be included in Command Request Transfer IE.

Proposal 6: The IE type of AIOTF Identifier IE can be OCTET STRING.

# 4	Reference

[1] RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TS 23.369 v0.3.0, Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19);