# 3GPP TSG-RAN WG3 Meeting #129bis	R3-256542

# Prague, CZ, 13-17,Oct 2025

# Agenda Item:	10.3.2

# Source:	ZTE Corporation

# Title:	Consideration on 6G RAN-CN interface options

# Document for:	Discussions & Approval

1	Introduction

In the new 6G SI in RP-251881, the following objective was identified from mainly in RAN3’s scope.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

This contribution provide our initial views on 6G RAN-CN interface options.

2	Discussion

2.1	Overview on NR interfaces between RAN and 5GC

In current 5G system, the NG-RAN architecture is illustrated in Figure 2-1 below.

Figure 2-1: Overall Architecture

As we know, NG Interface is the logical connection bridge between RAN node and 5G core network (5GC), which is designed to separate the control plane and user plane functions. It mainly includes two parts. The NG-C interface is responsible for control plane communication between RAN node and the AMF through the Next Generation Application Protocol (NGAP). For the reliable transport of signalling messages, SCTP is added on top of IP and provides guaranteed delivery of application layer messages. The NG-U interface is responsible for user plane data transmission between RAN node and UPF through IP transport and UDP/IP protocol.

On the other hand, service based concept is introduced in 5G CN, SBA divides the functional modules in the traditional core network into independent and reusable services. In SBA, each network function entity (NF) in the core network is decoupled and split at the functional level. These NFs can operate independently without relying on each other, and can be upgraded and elastically adjusted independently. In order to help different NFs to communicate with each other, the Network Repository Function (NRF) is introduced and it is responsible for the service registration management of NFs and provides a service discovery mechanism. Then, as a service consumer, an NF can find the applicable target NF through the NRF.

- In 5G, one Point to point (P2P) interface is defined between RAN and CN, and service based interfaces (SBI) are used for NFs in CN.

- In addition, the network capability exposure mechanism is a core component of the SBA according to which the capabilities and resources of the 5G core network can be exposed to external entities (such as third party application providers, vertical industry platforms) and internal network functions (NFs). In 5G networks, the Common API Framework (CAPIF) and the Network Exposure Function (NEF) are both responsible for implementing this mechanism. CAPIF provides a set of standardized interfaces and frameworks, which is responsible for the interaction with external application functions (AFs), authenticates and authorizes API invokers, and manages API information while NEF provides 5G network capabilities to Application Functions (AFs) through SBA.

- In 5G networks, the CAPIF and the NEF are closely related. CAPIF is a standard capability exposure framework defined by 3GPP, which provides a functional architecture for API callers to access and invoke service APIs. NEF is a network element responsible for network capability exposure in 5G, and provides 5G network capabilities to Application Functions (AFs) through service based architectures. It interacts with internal network elements of 5GC through south bound interfaces and provides open API interfaces to AFs through north bound interfaces. CAPIF provides a basis for the standardized operation of NEF in capability exposure, and NEF is the specific implementer of capability exposure under the CAPIF framework.

- In 5G, network capability exposure mechanism is supported by Common API Framework (CAPIF) and the Network Exposure Function (NEF) based on SBA in CN.

2.2	6G RAN-CN interface options and comparison

The following three options have been proposed for 6G RAN-CN interface based on companies’ contributions:

- Option 1：Reusing existing P2P interface for both signalling and data

Figure 2-2-1: existing NG-U and NG-C Protocol Stack

- Option 2：Introducing Service-based Interface

Figure 2-2-2: SBI Protocol Stack

- Option 3：QUIC based NGAP interface for signalling / QUIC based NG-U for data

Figure 2-2-3: QUIC based NG-U and NG-C Protocol Stack

As we know, the P2P connection between RAN and CN is a traditional connection method inherited from the 4G/5G. The design concept of P2P connection is based on dedicated resource allocation and fixed topology, which could meet the requirements in the network environment at that time but lacks flexibility when facing the diverse service demands of 5G. With the development of 5G technology, the SBI connection is introduced in Release 15, designing the 5G core network as a set of independently deployable and scalable network functions that communicate with each other through SBI interfaces. The emergence of SBI enables the core network to better support new service scenarios such as network slicing and NWDAF, realizing the transformation from a "network-centric" to a "service-centric" paradigm. Particularly as a new transport protocol, QUIC features low latency and high reliability..

In order to help decide which option is best for 6G interface, we propose to compare the above three RAN-CN interface options from the aspects addressing the CAPEX/OPEX , flexibility, future-proof, practical challenges, and reliability in 6G networks.

First, the impacts on legacy CN directly relates to network CAPEX/OPEX: How well each interface option adapts with the existing network architecture needs to be evaluated (e.g., taking the effort on retrofitting, backward compatibility and migration into account) as operators may want to maximize the use of their existing network investment.

Second, Integration & Evolution directly relates to flexibility: Assessing how each option to support new 6G technologies (e.g., sensing, AI) in a future-proof way, enabling operators to deploy new services quickly.

Third, Operational Efficiency also reflects CAPEX/OPE: this includes the factors like fault detection and scalability (e.g., the impacts on other NFs when deploying new services).

Fourth, Security is an important factor in 6G interface design, and it is necessary to consider how to ensure the security of signaling and data transmission. For example, TCP-based signalling may rely on external IPsec, QUIC (Quick UDP Internet Connections) integrates security as a fundamental, non-optional component of the protocol..

Finally, Stability&Reliability addresses practical challenges: this encompasses technical maturity (e.g., standardized protocols, Commercial Deployment Verification) and hardware/software compatibility (e.g., whether existing RAN/CN nodes can run the interface without hardware upgrades).

In conclusion, the above five aspects of comparison do not focus solely on technical specs, instead, they holistically evaluate each option against operators’ current constraints (such as legacy infrastructure and costs), future strategic goals (including scalability and evolution), and non-negotiable requirements (like security and operability). Only by considering these aspects can operators select an interface that balances short-term deployment practicality with long-term business and technical sustainability.

- It is suggested that comparison of different interface options should not only focus on technical metrics but also on deployment and operation.

For QUIC based NGAP mechanism, transport layer protocol for NGAP needs to be changed from TCP to QUIC. Although QUIC bring better performance than TCP in transport layer, its application together with NGAP in 3GPP network lacks verification. Introducing QUIC based NGAP would demand new conformance and interoperability test cases to verify that RAN and CN nodes from different vendors can correctly implement QUIC-based NGAP, including handling connection migration, 0-RTT handshakes, and stream multiplexing as defined by IETF QUIC specifications (RFC 9000). For example, in extreme network conditions with high packet loss rates, the congestion control and retransmission mechanisms of QUIC may not yet match TCP’s maturity in 5G core networks, potentially affecting the stability and robustness of NGAP signaling transmission and service quality. The similar issue raised for QUIC based NG-U.

- The Stability&Reliability of Legacy P2P interface and SBI is good enough while QUIC based NGAP and QUIC based NG-U in 3GPP network lack verification.

- The Stability&Reliability of QUIC based NGAP and QUIC based NG-U should be verified.

A key advantage of service-based interfaces over P2P interfaces lies in their greater ease of exposing RAN information and natively compatible with third-party applications. However, the traditional 5G P2P NG interface is primarily designed to support functions such as interface management, UE context management, mobility handling, NAS transport, paging, PDU session management, and configuration transfer. Notably, these capabilities have no inherent requirement for external exposure or invocation. Considering the costs and efficiency of network evolution, reusing the existing P2P interface for implementing these well-established and basic communication functions can leverage mature infrastructure, minimizes redevelopment efforts, and avoids unnecessary operational disruptions.

- Reusing the existing P2P interface for implementing these well-established and basic communication functions can leverage mature infrastructure, minimizes redevelopment efforts, and avoids unnecessary operational disruptions.

With the rapid emergence of new 6G services (e.g., sensing, AI), a readily extensible interface is critical to be considered in 6G. Compared with P2P interface, the Service Based Interface (SBI) is easier to achieve this due to its inherent flexibility. When new services arise, operators only need to define new standardized APIs for target NFs without overhauling the entire framework, avoiding the reconfiguration bottlenecks of traditional interfaces and unnecessary network entity upgrading. In short, SBI’s modular design and standardized APIs make it ideal for accommodating new services easily and efficiently.

- The Service Based Interface (SBI) is natively compatible with third-party applications and ideal for accommodating new services easily and efficiently.

In existing 5G network system, SBI is only used for signalling transfer in CN side, it may need further evaluation if SBI is also used for data transfer. In 6G, cooperation between RAN AI and a dedicated 6G AI NF in the Core Network is expected to potentially involve the exchange of large size of AI related data. Such data may include, for example, AI models related infor, historical measurement data collected over a long period, or measurements collected from multiple UEs.

- In 6G, cooperation between RAN and AI NF in the Core Network is expected to potentially involve the exchange of large size of AI related data.

In HTTP/2, a large message is segmented into a sequence of frames, starting with a HEADER frame followed by multiple DATA frames. These frames are transmitted over a single, persistent TCP connection. Within a given HTTP/2 stream, identified by a unique Stream ID, frames are delivered in order, allowing the receiver to reassemble the message . HTTP/3 provides a similar segmentation mechanism for large message sending, using the similar frame types (HEADER, DATA). However, these frames are carried over QUIC streams instead of a TCP connection. Each HTTP/3 stream is mapped to an independent QUIC stream, ensuring in-order delivery within the stream while avoiding head-of-line blocking across streams. In general, we think the HTTP2 or HTTP3 based SBI interface can support large size data transmission by segmentation mechanism.

- Large size of data transmission can be supported over SBI by segmentation mechanism.

Based on above analysis, we summarize and provide an comparison among above three options via multiple aspects in  following table 1.

Table 1 comparison among three RAN-CN interface options

|  | Legacy P2P interface | Service-based interface | QUIC based NGAP/NG-U |
| --- | --- | --- | --- |
| Impacts on legacy CN | All RAN to NF/AF signaling is routed via AMF requiring AMF upgrade every time a service added, and it may lead to AMF overload; The latency will be increased due to unnecessary routing. | Enables direct RAN to NF/AF communication without impact on legacy core while providing scalability to signaling RAN information by demand. ☺ | Similar like P2P |
| Integration & Evolution | Different interface and signalling design for different 6G new services; Difficult to support customized RAN information retrieval for third-party applications | Plug-and-play capability to support new NFs in 6G CN; Dynamic service discovery and registration; Easy to support authorized third-party applications via Common API Framework (CAPIF).☺ | Similar like P2P |
| Operational Efficiency | Complex fault isolation due to intermediate entities; All the intermediate entities have to be upgraded even they are not participating in the new service. | Simplified fault detection; Only related NFs need to be upgraded for the new service.☺ | Similar like P2P |
| Security | How to ensure the security of RAN information transmission (NOT UE traffic data) from RAN to NF/AF needs to be studied, CP or UP, if UP, how? | Well applied in CN and better adapted to third-party authorization and access (TLS + OAuth).☺ | QUIC has built-in TLS 1.3 encryption which may have better Security performance than TCP. |
| Stability &Reliability | High Stability &Reliability. The application foundation of the TCP protocol in 5G networks provides good support, and P2P interface can be reused for basic communication functions in 6G.☺ | High Stability &Reliability for signalling transmission. The SBI has been introduced and applied in 5G core network. ☺ | Low Stability &Reliability due to lacking of verification for both signalling and data. |

- RAN3 to capture the comparison table above among three RAN-CN interface options in TR 38.914 for further discussion.

In addition, according to above comparison among three options, we can find that each option has its own strengths, weaknesses, and suitability for distinct scenarios. RAN3 is suggested to consider whether to support multiple interfaces between RAN and CN. One illustration is shown as figure 2-2-4.

- Figure 2-2-4 One illustration of multiple interfaces between RAN and CN in 6G

- Proposal 5 RAN3 to discuss whether to support only one interface or multiple interfaces between RAN and CN in 6G.

3	Conclusions

Following is our observations and proposals:

- In 5G, one Point to point (P2P) interface is defined between RAN and CN, and service based interfaces (SBI) are used for NFs in CN.

- In 5G, network capability exposure mechanism is supported by Common API Framework (CAPIF) and the Network Exposure Function (NEF) based on SBA in CN.

- The Stability&Reliability of Legacy P2P interface and SBI is good enough while QUIC based NGAP and QUIC based NG-U in 3GPP network lack verification.

- Reusing the existing P2P interface for implementing these well-established and basic communication functions can leverage mature infrastructure, minimizes redevelopment efforts, and avoids unnecessary operational disruptions.

- The Service Based Interface (SBI) is natively compatible with third-party applications and ideal for accommodating new services easily and efficiently.

- In 6G, cooperation between RAN and AI NF in the Core Network is expected to potentially involve the exchange of large size of AI related data.

- It is suggested that comparisons of different interface options should not only focus on technical metrics but also on deployment and operation.

- The Stability&Reliability of QUIC based NGAP and QUIC based NG-U should be verified.

- Large size of data transmission can be supported over SBI by segmentation mechanism.

- RAN3 to capture the comparison table above among three RAN-CN interface options in TR 38.914 for further discussion.

- RAN3 to discuss whether to support only one interface or multiple interfaces between RAN and CN in 6G.

4	Reference

[1] RP-251881 New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

5	Text Proposal for TR 38.914

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6	RAN Architecture and Interfaces

Editor’s note: The aim of this section is to describe all aspects related to the interfaces between RAN and CN.

6.1	General Principles and Requirements

6.2 	RAN-CN interface

6.2.1	RAN-CN interface general principles and functions

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services and including control plane and user plane aspects considering interim milestone.

6.2.2	Evaluation of RAN-CN interface options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 5.2.1

The following three options are proposed for 6G RAN-CN interfaces:

- Reusing existing P2P interface for both signalling and data

- Introducing Service-based interface

- QUIC based NGAP interface for signalling / QUIC based NG-U for data

An comparison among above three options via multiple aspects is provided in following table 1.

Table 1 comparison among three RAN-CN interface options

|  | Legacy P2P interface | Service-based interface | QUIC based NGAP/NG-U |
| --- | --- | --- | --- |
| Impacts on legacy CN | All RAN to NF/AF signaling is routed via AMF requiring AMF upgrade every time a service added, and it may lead to AMF overload; The latency will be increased due to unnecessary routing. | Enables direct RAN to NF/AF communication without impact on legacy core while providing scalability to signaling RAN information by demand. | Similar like P2P |
| Integration & Evolution | Different interface and signalling design for different 6G new services; Difficult to support customized RAN information retrieval for third-party applications | Plug-and-play capability to support new NFs in 6G CN; Dynamic service discovery and registration; Easy to support authorized third-party applications via Common API Framework (CAPIF). | Similar like P2P |
| Operational Efficiency | Complex fault isolation due to intermediate entities; All the intermediate entities have to be upgraded even they are not participating in the new service. | Simplified fault detection; Only related NFs need to be upgraded for the new service. | Similar like P2P |
| Security | How to ensure the security of RAN information transmission (NOT UE traffic data) from RAN to NF/AF needs to be studied, CP or UP, if UP, how? | Well applied in CN and better adapted to third-party authorization and access (TLS + OAuth). | QUIC has built-in TLS 1.3 encryption which may have better Security performance than TCP. |
| Stability &Reliability | High Stability &Reliability. The application foundation of the TCP protocol in 5G networks provides good support, and P2P interface can be reused for basic communication functions in 6G. | High Stability &Reliability for signalling transmission. The SBI has been introduced and applied in 5G core network. | Low Stability &Reliability due to lacking of verification for both signalling and data. |

Note 1：More matrics can be added in the table.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>