3GPP TSG-RAN WG3 Meeting #129bis	R3-256545

Prague, Czech Republic, 13th – 17th October, 2025

Title:	Discussion on possible RAN internal architecture evolution towards Service Based Architecture

Source:	FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra

Agenda item:	10.2.2

Document Type:	Discussion and Decision

# 1	Introduction

The Service-Based Architecture (SBA) model, which has already demonstrated its effectiveness in the 5G Core Network (5GC) and in OAM domains since Rel-15, is widely recognized for enabling modularity, improved service flexibility, and streamlined integration processes. In contrast, the current architecture of the RAN remains anchored to a Protocol-Based approach, characterized by tightly coupled interfaces and limited architectural agility.

In contribution [1], the rationale for investigating the adoption of SBA principles in the RAN domain has been introduced, outlining potential advantages, key constraints, and preliminary requirements. That analysis also highlighted the need of identifying which parts of the RAN could realistically benefit from a service-oriented design, taking into account some constraints like strict latency and performance demands.

This contribution builds on those foundations and focuses on exploring possible architectural directions to introduce SBA within the RAN. It aims to outline some alternatives in implementation scenarios, starting from current Network Functions (NFs) within the RAN.

This contribution is grounded on ongoing activities within the research project ORIGAMI [2]. The ORIGAMI project brings together leading industry and academic partners across the European mobile telecommunications ecosystem and is funded by the European Commission under the 6G Smart Networks and Services Joint Undertaking (SNS JU). Running from 2024 to 2026, ORIGAMI focuses on advancing architectural models for next-generation mobile networks and addressing key barriers to the deployment of successful 6G technologies. Specifically, ORIGAMI aims to study 6G architectural innovations, like the Global Service-Based Architecture (GSBA), that enables solutions of Network Intelligence (NI) designed to enhance network performance, security, and flexibility. Beyond technological advances, ORIGAMI is committed to establishing global unified standards, supporting the green transition, improving affordability and accessibility, and fostering the development of groundbreaking applications to create strong ecosystem and novel business models for operators to generate new revenue opportunities and allows operator networks to evolve into future generation with advanced capabilities. This holistic approach positions ORIGAMI at the forefront of shaping SBA that will underpin future 6G networks, as addressed in [3].

# 2	Discussion

## Potential SBA Proposals for the RAN

For the introduction of a SBA approach in the RAN, the current 5G System (5GS) architecture depicted in Figure 1 can be considered as a starting point, where there is the evidence of the difference between the service-based design of the 5GC and the protocol-based architecture of the RAN.

While the 5GC already embraces the SBA paradigm, enabling flexible service exposure, dynamic discovery of network functions, and modular integration, the RAN still relies on tightly specified interfaces and procedural communication models between its NFs. This results in a less flexible and less scalable system design.

Figure 1 illustrates the existing 5GS architectural model, with the Core Network (CN) represented through a clear SBA approach. The RAN, shown in the red dashed ellipse, interfaces with the 5GC through standardized reference points (e.g., N2, N3), and internally does not yet adopt a service-based interaction model. This observation motivates the idea of applying the Service-Based paradigm also to the RAN, by rethinking both:

- the external interfaces between RAN and CN, ensuring alignment with SBA principles where applicable

- the internal architecture of the RAN, with a focus on potential reorganization of functional blocks and NFs into service-oriented structure based on services to be provided and consumed each other

Figure 1 – Starting point: current 5GS architecture and the proposed SBA extension to RAN

From a technical standpoint, applying the SBA paradigm to the RAN implies a fundamental shift in how RAN NFs interact, both internally and with external domains such as the 5GC. In particular, this evolution could involve:

- Service Identification from Signalling Procedures – The transition towards SBA requires an initial step of analysing existing signalling procedures to identify the underlying functionalities they implement. These functionalities can then be mapped into discrete services, allowing a clear definition of which RAN modules act as service producers or consumers. This decomposition is essential for modularizing RAN behaviour and enabling service exposure, invocation, and reusability across functional boundaries

- Service Exposure and Discovery: RAN NFs could expose their capabilities as discoverable services via standardized APIs, enabling other NFs (within RAN or external domains) to dynamically locate and invoke such services. This would require introducing mechanisms, similar to the Network Function Repository Function (NRF) used in the 5GC

- Consumer/Provider Roles within RAN: functions such as the one related to the gNB Central Unit for the Control Plane (CU-CP), gNB Distributed Unit (DU), or higher-layer schedulers could act as service providers, offering functionalities (e.g., resource management, UE context handling, QoS provisioning) that other RAN functions consume via service calls, rather than relying on tightly coupled protocol procedures

- RAN Interface re-architecture: traditional point-to-point interfaces like F1, E1 and Xn could be re-evaluated and potentially redesigned as service-based reference points, allowing more flexible and extensible communication patterns, including support for multi-tenancy, programmability, and network slicing

- Interworking with SBA-based CN: applying SBA to the RAN would also allow more coherent and native integration with the CN, possibly enabling RAN services to be easily invoked or orchestrated, especially in the context of AI/ML-driven network automation and intent-based management

This conceptual transition raises several architectural and performance-related challenges, including service granularity, latency budgets, reliability, and control/user plane separation.

In the following, high-level architectural schemes are analysed, based on how and where the SBA paradigm is introduced within the RAN. The analysis follows a top-down approach, starting from the interaction between the RAN and the CN, addressed in [4], and progressively considering lower-level internal interfaces and functional splits within the RAN architecture.

While the diagrams refer to the existing 5G NFs (e.g., CU, DU, RU) for continuity and clarity, the actual entities involved in service-based interactions are the internal services identified within these functional modules. These services may act as producers, consumers, or both, depending on the specific functionality they provide or require. The objective is to illustrate how the RAN could evolve from a protocol-based model to a more flexible, modular, and service-oriented architecture, without disrupting the logical structure defined in current 5G deployments and facilitating the requirements fulfilment of the new 6G RAN design (e.g., CP/UP separation, multi-vendor interoperability, RAN sharing) [3].

Scenario #1: SBA Applied to E1 and Xn-C Interfaces

In this scenario, the SBA is applied to the E1 interface (between CU-CP and CU-UP) and to the Xn-C interface addressing communication between CU-CPs.

As depicted in Figure 3, this scenario introduces a Service Bus internal to the RAN. This internal Service Bus enables service-based interactions among functionalities of CU-CP and CU-UP components or among CU-CPs, transforming traditional E1 and Xn communication into service exposure and consumption, while maintaining unchanged the underlying RAN architecture.

The following two service-based reference points could be introduced:

- Nngcucp*: this SAP refers to services produced by the CU-CP and consumed by CU-UP or other CU-CPs

- Nngcuup: this SAP refers to services produced by the CU-UP and consumed by the CU-CP

Figure 3 – High level scheme of SBA applied to E1 and Xn-C interfaces

The motivation for including E1 and Xn-C in the SBA evolution path follows from the fact that these interfaces do not impose strict latency or real-time constraints. Transitioning E1 and Xn-C to a service-based interface enables CU-CP and CU-UP components to interact via standardized, discoverable services, avoiding the need for a proliferation of static point-to-point (P2P) connections. In deployment scenarios where a single CU-CP is associated with multiple CU-UP instances (1:N relationship), the adoption of a producer/consumer model allows for more efficient and scalable interactions. Instead of establishing and maintaining dedicated E1 interfaces for each CU-UP, the CU-CP can dynamically discover and consume services offered by available CU-UPs via a logical Service Bus. This decouples control and user plane functions at the service level, simplifies configuration management, and facilitates dynamic association based on policies such as load, location, or resource availability.

For example, in scenarios involving dynamic user-plane function selection – due to mobility, traffic steering, or resource optimization – the CU-CP can invoke user-plane services exposed by the most appropriate CU-UP instance without requiring pre-established, rigid bindings. This architectural flexibility is particularly beneficial in cloud-native or disaggregated RAN environments, where CU-UP instances may be dynamically instantiated, scaled, or relocated.

Ultimately, this represents a possible step in the progressive evolution toward a fully service-based RAN architecture, leveraging the benefits of cloud-native design without disrupting the essential functional roles already defined in 5G.

Scenario #2: SBA Applied to E1 (and Xn-C), and F1-C Interfaces

Building on the previous scenarios, in this scenario, the SBA is further extended to cover the F1-C interface, which connects the CU-CP and the DU in the control plane domain.

As shown in Figure 4, the RAN Service Bus introduced in previous scenario is reused and logically extended to support service exposure and consumption by both CU-CP and DU functions. The goal is to enable a consistent, modular, and scalable control plane across all RAN components, including distributed units.

As for the previous scenario, the following two service-based reference points could be introduced:

- Nngcucp**: this SAP refers to services also produced by the CU-CP and consumed by DUs

- Nngdu: this SAP refers to services produced by the DU and consumed by the CU-CP

Figure 4 – High level scheme of SBA applied to E1, Xn-C and F1-C interfaces

Transitioning F1-C to a service-based interface allows CU-CP and DU components to interact via standardized, discoverable services over the common RAN Service Bus, avoiding the need for static point-to-point (P2P) configurations. In scenarios where a CU-CP manages multiple DU instances (1:N relationship), this producer/consumer model simplifies interaction patterns, improves flexibility, and enables dynamic association between control and distributed units.

Instead of maintaining dedicated F1-C interfaces per DU instance, the CU-CP can dynamically discover and invoke services exposed by the available DUs. These services may include, for example, cell configuration, radio resource reporting, or scheduling-related feedback. Conversely, the CU-CP may expose services that the DU can consume, such as updated RRC configurations or policy-based instructions.

For example, in a disaggregated or cloud-native deployment, where DUs may be instantiated, scaled, or reallocated based on coverage or load requirements, the use of service-based F1-C communication allows the CU-CP to interact with new or relocated DUs without manual reconfiguration. The DU registers its services with the RAN Service Bus and becomes discoverable to the CU-CP, which can then invoke only the necessary services based on its operational context.

This approach eliminates the need for static F1-C bindings and enables more adaptive and automated lifecycle management of RAN nodes, especially valuable in large-scale deployments with frequent topology changes or dynamic resource allocation strategies.

Final remarks

The analysis presented across the proposed scenarios highlights how the principles of the SBA, originally defined within the 5GC, can be progressively extended to different segments of the RAN architecture. This extension aims to increase modularity, enable more flexible function interaction, and support scalable deployment models.

In particular, the SBA paradigm proves especially beneficial in architectural points where services are dynamically consumed by multiple entities, i.e., scenarios involving 1:N or N:1 relationships, as the ones existing on N2, E1 and F1-C interfaces.

Observation 1: SBA approach could be beneficial to evolve current architecture to enable higher flexibility, e.g. allowing a dynamic interaction with multiple service consumers and producers.

Based on the aspects outlined above, the introduction of SBA in the RAN should be considered and the alternative scenarios addressed could be taken into account.

Proposal 1: RAN3 to study SBA introduction in the RAN for different internal RAN interfaces.

Notice that the evolution of RAN internal interface in an SBA approach, is compatible and extend the evolution towards SBA approach applied to RAN-CN interface, described in [4], and depicted in following Figure 5 for sake of completeness.

Figure 5 – High level scheme of SBA applied to N2, E1, Xn-C and F1-C interfaces

# 3	Conclusion

In this paper we discuss different scenarios of SBA introduction in the RAN and the following is proposed:

Proposal 1: RAN3 to study SBA introduction in the RAN for different internal RAN interfaces.

# 4	References

[1]	R3-256544, Discussion on Feasibility of RAN Service Based Architecture (FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra)

[2]	https://sns-origami.eu/

[3]	3GPP TR 38.914, “Study on 6G Scenarios and requirements” (Rel-20)

[4]	R3-256719, Discussion on possible RAN-CN evolution towards Service Based Architecture (FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra)