# 3GPP TSG RAN WG3 Meeting #129-bis														  R3-256551

# Prague, CZ, 13 - 17 October, 2025

# Title: 	Discussion on R20 A-IoT for Topology 2

# Source: 	ZTE Corporation, China Telecom

Agenda item:	14.2

# Document for:	Discussion and Decision

- Introduction

In the RAN plenary #108 meeting, the new WID “New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2” is approved in RP-251885.

In the RAN plenary #109 meeting, the WID is revised in RP-252894, mainly updated the RRC based solution to align with SA2.

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN1 - The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS. - No physical layer impact for signaling A-IoT resources to the intermediate UE. - No additional specification change on R2D and D2R for Device 1 in Rel-20 RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling. RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-node resource coordination is not specified. - Specify the necessary signaling support for inter-node RRC-connected UE Reader Mobility RAN4 - Specify RF requirements for intermediate UE towards the A-IoT device. - Specify necessary updates to RF requirements for A-IoT device 1. - Specify A-IoT active device(s) RF requirements in D2T2 - Specify RRM requirements for intermediate UE due to A-IoT operation, if any. Combination with existing RRM procedures/requirements is excluded. - Specify RRM requirements for Device(s) in D2T2 if necessary - Use band n8 as an example band. - Specify device (un)availability via Direction 2 in TR 38.769, [RAN2-led, RAN1]. - This objective is applicable only for active Device(s) - Solutions shall be unified for Topology 1 and Topology 2. - This objective begins after RAN#111 (March 2026) - NOTE: Device for outdoor scenarios may be added here, pending RAN#111 decision. Note: Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band.

According to the scope of the above WID, the following three issues shall be discussed in RAN3.

- Specification of signaling for Topology 2

- Specification of UE reader authorization

- Signaling support for inter-node RRC-connected UE Reader Mobility

This paper addressed on the above two issued indicated by bullet 1 and bullet 2.

- Discussion

  - Specify signaling for Topology 2

In R19, we have introduced the following AIOT procedures for Topology 1.

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	A-IoT Session Release (AIoT CN Initiated);

-	A-IoT Session Release Request (node initiated).

In R20, these AIOT procedures are suggested for Topology 2.

Proposal 1: The following AIOT procedures are introduced for Topology 2.

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	A-IoT Session Release (AIoT CN Initiated);

-	A-IoT Session Release Request (node initiated).

More, in TR38.769, for RRC based solution, we also capture the following overall message flows for both Inventory and command procedure.

TR 38.769 6.4.2.1.1 Solution 1: RRC based solution Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled node transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa. Figure 6.4.2.1.1-1: RRC based solution of Topology 2 In this solution, XX interface is NG-C interface. NOTE: Whether "XXAP" is realised by including AIoTF information in NGAP or by carrying a new protocol layer over NGAP, needs further discussion and decision. 6.5.3.1 Candidate procedures for A-IoT Inventory for Topology 2 6.5.3.1.1 RRC based solution Figure 6.5.3.1.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution 1a. The A-IoT CN sends an Inventory request message to the A-IoT enabled node 1b. The A-IoT enabled node allocates and coordinates usage of A-IoT radio resources. 1c/2a RRC communication with the A-IoT enabled UE takes place. NOTE 1: RRC based communication is only depicted schematically, details subject to RAN2. 2. The A-IoT-enabled node sends an Inventory response message to the A-IoT CN. NOTE 2: In step 2, the A-IoT-enabled node may instead send an Inventory failure message to the A-IoT CN indicating that the inventory procedure could not be initiated towards the A-IoT device(s). 3. The A-IoT-enabled node requests the A-IoT-enabled UE(s) to trigger inventory procedure towards the A-IoT device(s). 4a/4b. After receiving inventory result reported from the A-IoT enabled UEs, the A-IoT-enabled node may send one or multiple Inventory reports towards the A-IoT CN including the received inventory result. NOTE 3: Steps 4a/4b may happen in parallel with Step 3 for different A-IoT devices. NOTE 4: Step 4a/4b between A-IoT-enable UE and A-IoT-enabled node subject to RAN2. 6.5.3.2 Candidate procedures for A-IoT Command for Topology 2 6.5.3.2.1 RRC based solution Figure 6.5.3.2-1: Message flow for A-IoT Command in Topology 2 (RRC-based solution) 1. The Inventory procedures are performed in step 1, which are the same as in Figure 6.5.3.1.1-1 in step 1-4. NOTE 1: Step 1 is performed for the "A-IoT Inventory and Command" case, Whether and under which conditions step 1 may be skipped in case of "Command-only" and which consequences this would have for the overall message flow depends on discussions led by SA2, SA3 and RAN2. NOTE 2: Whether and which way steps 2a, 2b and 2c may be part of step 1 is dependent on discussions led by SA2, SA3 and RAN2. 2a. The A-IoT CN sends a Command Request message to the A-IoT enabled node. 2b. The A-IoT enabled node coordinates A-IoT radio resources and allocates A-IoT radio resources to the A-IoT enabled UE accordingly. 2c. The A-IoT enabled node sends a RRC Command Request message to the A-IoT enabled UE. NOTE 3: RRC based communication, i.e., performing the A-IoT Command procedure and allocation of A-IoT radio resources, is only depicted schematically, details subject to RAN2. 3. The A-IoT-enabled UE performs A-IoT command procedures at A-IoT interface towards the A-IoT device over the A-IoT radio interface. 4a. A-IoT enabled UE sends a Command Response message to A-IoT enabled node, if the command result is received from A-IoT device, the A-IoT enabled UE may include the command result in the Command Response message. 4b. The A-IoT enabled node sends a Command Response message to the A-IoT CN, it may include the command result in the Command Response message, if any. NOTE 4: In step 4b, the A-IoT enabled node may instead send a Command Failure message to the A-IoT CN indicating that the command procedure is failed towards the A-IoT device.

In R20, during the normative work, the above progress can be as baseline or as start point to specify the R20 AIOT signalling and procedure for Topology 2.

Proposal 2: The AIOT procedures (Topology 2, RRC based solution) in TR38.769 can be as start point to specify R20 AIOT signalling and procedure for Topology 2.

  - Specification of UE reader authorization

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

In current NGAP/XnAP/F1AP specifications, in order to support V2X services via the PC5 interface, the authorization status of the UE to use the NR sidelink for V2X services has been specified as “V2X Services Authorized”.

NR V2X Services Authorized This IE provides information on the authorization status of the UE to use the NR sidelink for V2X services. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Vehicle UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Vehicle UE. Pedestrian UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Pedestrian UE. LTE V2X Services Authorized This IE provides information on the authorization status of the UE to use the LTE sidelink for V2X services. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Vehicle UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Vehicle UE. Pedestrian UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Pedestrian UE.

In NGAP specification, the authorization status of the UE is sent from AMF to NG-RAN node, via the following messages.

-	INITIAL CONTEXT SETUP REQUEST

-	UE CONTEXT MODIFICATION REQUEST

-	HANDOVER REQUEST

-	PATH SWITCH REQUEST ACKNOWLEDGE

In XnAP specification, the authorization status of the UE is sent from source/old NG-RAN node to the target/new NG-RAN node, via the following messages.

-	HANDOVER REQUEST

-	RETRIEVE UE CONTEXT RESPONSE

In F1AP specification, the authorization status of the UE is sent from node-CU to node-DU, via the following messages.

-	UE CONTEXT SETUP REQUEST

-	UE CONTEXT MODIFICATION REQUEST

Proposal 3: The UE V2X service authorization mechanism can be as reference to specify the UE reader authorization.

- Conclusion

Based on the above analysis, we provide the following procedures.

Proposal 1: The following AIOT procedures are introduced for Topology 2.

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	A-IoT Session Release (AIoT CN Initiated);

-	A-IoT Session Release Request (node initiated).

Proposal 2: The AIOT procedures (Topology 2, RRC based solution) in TR38.769 can be as start point to specify R20 AIOT signalling and procedure for Topology 2.

Proposal 3: The UE V2X service authorization mechanism can be as reference to specify the UE reader authorization.