3GPP TSG-RAN WG3 Meeting #129bis	R3-256559

Prague, Czech Republic, October 13-17, 2025

Agenda Item:	13.3

Source:	ZTE Corporation

Title:	Discussion on RAN-CN Procedures and Signalling for ISAC

Document for:	Discussion

- Introduction

In the SID of ISAC[1], the  objective is given as below.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this contribution, we provide the initial consideration on RAN-CN procedures and signalling for ISAC.

2	Discussion

In TR 23.700-14[2], six key issues related to the support of ISAC in R20 have been identified by SA2, which are given as below.

Key Issue #1: System Architecture to Support Sensing Key Issue #2: Authorization and Revocation to Support Sensing Service Key Issue #3: Sensing Entity and Sensing Function Discovery and (Re-)Selection Key Issue #4: Sensing Data and the Associated Information Collection and Transport Key Issue #5: Sensing Result Exposure Key Issue #6: Configuration of Parameters for Sensing Entities

- Taking the key issues into consideration, the overall RAN-CN procedure for ISAC in R20 can be given as below.

Figure 1: Overall RAN-CN Procedure of ISAC

Step 0. The Sensing Entity (gNB) sends the its sensing capability info to the Sensing Function for capability coordination.

Step 1. The Sensing Service Consumer initiates the Sensing Service Request procedure and Sensing Service Authorization procedure towards the Sensing Function.

Step 2. The Sensing Function triggers the Sensing Service Authorization procedure towards the Sensing Entity.

Step 3. The Sensing Function performs the Discovery or Selection of Sensing Entity.

Step 4. The Sensing Function sends the sensing configuration parameters to the selected Sensing Entity.

Step 5. The Sensing Entity sends the feedback to the request from Sensing Function.

Step 6. The Sensing Entity performs the sensing data measurement based on the received sensing configuration parameters.

Step 7. The Sensing Entity reports the sensing data and associated information to the Sensing Function.

Step 8-9. The Sensing Function determines the final sensing result and exposes the sensing result to the Sensing Service Consumer.

Step 10. The Sensing Function/Sensing Entity may trigger the Sensing Service Revocation procedure to release the sensing service/measurement.

Proposal 1: Agree to capture the overall RAN-CN procedure of ISAC in the TR.

Based on the above procedure, the RAN-CN functions for ISAC can be summarized as below.

- Sensing Capability Coordination: SE (gNB) provides its sensing capability to SF.

For example, the capability not only includes whether SE can perform sensing, but also includes whether the SE can do sensing measurement processing, whether the SE  can do sensing fusion from multiple TRPs’ sensing measurements

- Authorization and Revocation of Sensing Entity: SF triggers the Authorization of SE

- Configuration of Parameter for Sensing Entity: SF provides the sensing configuration parameters to SE

For example. The configuration of parameters from sensing request can include sensing area and potential information of sensing targets, i.e. UAV, such as UAV speed range, UAV size

- Sensing Data and Associated Information Exchange: SE performs the sensing measurement and reports the sensing data with association information to SF

- Sensing Service Release: SF/SE triggers the release/cancellation of the sensing measurement

Proposal 2: RAN3 shall study the RAN-CN signalling for ISAC based on the above functions.

Basically, we think the measurement metrics can be divided into four levels.

Level 1: Raw data

Level 2: Delay, Doppler, angle

Level 3: Sensing results per point

Level 4: Sensing results per object

Regarding the study of Level 1 and Level 2 measurement metrics which are purely physical layer measurements, it should be pending to the discussion and decision in RAN1. While, for the study of Level 3 and Level 4 measurement metrics, the details could be further discussed in RAN3 and coordinated with RAN1 if needed.

- Given the massive volume of sensing results received at the gNB, how to determine the association between the sensing results and the corresponding UAV(s) needs to be further discussed.

As shown in the below figure, the points in different colors represent the sensing data of different UAVs, and it needs to discuss how to report to SF to differentiate these sending data.

Figure 2: Multiple sensing data examples

- If the trajectory of the UAV could be reported, since the trajectories of different UAVs may interact, how to trace the trajectory of a specific UAV without any confusion in SF needs to be further discussed.

Figure 3: Trajectories of different UAVs

- In addition to the sensing results including the channel parameters, whether the gNB can send some other assistance information (e.g. type or size of the UAV) to help the Sensing Function make the final sensing decision needs to be further discussed.

Proposal 3: RAN3 should further study the following issues:

-How to determine the association between the sensing results and the UAV?

-How to report the trajectory of UAV to Sensing Function without any confusion?

-Is there any other assistance information needed from gNB to Sensing Function?

3	Summary

Proposal 1: Agree to capture the overall RAN-CN procedure of ISAC in the TR.

Proposal 2: RAN3 shall study the RAN-CN signalling for ISAC based on the above functions.

Proposal 3: RAN3 should further study the following issues:

- How to determine the association between the sensing results and the UAV?

- How to report the trajectory of UAV to Sensing Function without any confusion?

- - Is there any other assistance information needed from gNB to Sensing Function?

4	Reference

[1] RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, AT&T (Moderator)

- [2] TR 23.700-14 v1.0.0

5	TP for TR 38.765

8		RAN-CN procedures and signalling

Figure 8: Overall RAN-CN Procedure of ISAC

Step 0. The Sensing Entity (gNB) sends the its sensing capability info to the Sensing Function for capability coordination.

Step 1. The Sensing Service Consumer initiates the Sensing Service Request procedure and Sensing Service Authorization procedure towards the Sensing Function.

Step 2. The Sensing Function triggers the Sensing Service Authorization procedure towards the Sensing Entity.

Step 3. The Sensing Function performs the Discovery or Selection of Sensing Entity.

Step 4. The Sensing Function sends the sensing configuration parameters to the selected Sensing Entity.

Step 5. The Sensing Entity sends the feedback to the request from Sensing Function.

Step 6. The Sensing Entity performs the sensing data measurement based on the received sensing configuration parameters.

Step 7. The Sensing Entity reports the sensing data and associated information to the Sensing Function.

Step 8-9. The Sensing Function determines the final sensing result and exposes the sensing result to the Sensing Service Consumer.

Step 10. The Sensing Function/Sensing Entity may trigger the Sensing Service Revocation procedure to release the sensing service/measurement.