3GPP TSG-RAN WG3 #129-bis 	R3-256565

13th – 17th Oct 2025

Prague, Czech Republic

Agenda item:		  12.2.1

Source:	Lekha Wireless Solutions

Title:	Discussion on Multiple-hop UE trajectory

Document for:		Discussion and Decision

1. Introduction

In RAN#109 meeting, the new SI on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3 was approved [1] with the following objectives:

- Objective of SI The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

In this contribution, we would discuss on Multi-hop UE trajectory across gNBs and provide some proposals.

2. Discussion

2.1 Issues in Extending Rel-18 Framework to Multi-hop UE Trajectory

Observation:

In Rel-18, UE trajectory prediction and measured UE trajectory collection were introduced to support AI/ML-based mobility optimization in NG-RAN. The scope was limited to the next one-hop target NG-RAN node, considering complexity and time constraints.

The following aspects were specified in Rel-18 [2]: - Signaling procedures for UE trajectory prediction and measured UE trajectory collection. - Information elements included in predicted UE trajectory and measured UE trajectory. - Configuration of UE trajectory collection.

Multi-hop UE Trajectory:

Building on the Rel-18 framework, multi-hop UE trajectory refers to a list of cells (chronologically ordered) in which the UE will be served in RRC_CONNECTED mode, spanning at least two different gNBs.

- A gNB with a trained AI/ML model may infer a multi-hop UE trajectory prediction and send it to other gNBs owning cells included in the prediction.

- A gNB may also request measured UE trajectories for a UE that span across multiple gNBs.

Key Issues for Multi-hop UE Trajectory

To extend Rel-18 single-hop mechanisms, the following enhancements need to be addressed:

- Prediction Transfer – Mechanism for transferring UE trajectory predictions to multiple target gNBs.

- Configuration Transfer – Mechanism for transferring UE trajectory prediction/collection configuration to multi-hop target gNBs.

- Feedback Handling – Procedures for collecting and forwarding trajectory feedback from multi-hop target gNBs back to the source gNB.Based on the above observations, the following proposal is suggested:

To support multi-hop UE trajectory across gNBs, the Rel-18 single-hop framework can be reused as a baseline, with necessary enhancements for prediction transfer, configuration transfer, and feedback handling.

Proposal 1: The mechanism for transferring predicted UE trajectory information to multiple target gNBs needs to be defined.

Proposal 2: The configuration of UE trajectory prediction/collection should support transfer to multi-hop target gNBs.

Proposal 3: A feedback mechanism from multi-hop target gNBs back to the source gNB should be introduced.

2.2 UE Assistance Information for Multi-hop Trajectory Prediction

In Rel-17 TR 37.817, the following data was identified as required input for mobility optimization [3]:

From UE: - UE location information (e.g., coordinates, serving cell ID, moving velocity), as interpreted by gNB implementation when available. - Radio Measurements: Measurements related to serving and neighbouring cells associated with UE location (e.g., RSRP, RSRQ, SINR). - UE Mobility History Information.

To enable multi-hop trajectory prediction, additional input data may be required. One possible enhancement is to utilize UE destination/route information. For example, if a UE’s route is known (similar to navigation applications), multi-hop trajectory prediction can leverage predefined BSs along the path. The main open issue is how destination/route information is conveyed to the network:

- Option 1: Application-layer exposure via NEF

- Application → NEF API → Operator Core Network.

- NEF forwards “mobility intent” (destination, route, time window, QoS requirements) to NWDAF/MDAS for prediction support.

Option 2: Direct RAN/NAS signaling (future 6G option)

- UE provides Mobility Intent IE (destination/route ID) via RRC/NAS message to serving gNB.

- Serving gNB forwards to other gNBs (via Xn) or to NWDAF for global coordination.

Proposal 4: Consider destination/route information as an additional input for multi-hop UE trajectory prediction.

Proposal 5: Evaluate NEF-based application-layer exposure as a short-term mechanism for delivering mobility intent to the RAN.

Proposal 6: Study the feasibility of direct RAN/NAS signaling of mobility intent as a potential long-term (e.g., 6G) solution.

2.3 Mechanisms for Multi-hop UE Trajectory Prediction Transfer

In Rel-18 the mechanism for single hop trajectory collection is mentioned as:

UE trajectory collection is supported via the Data Collection Reporting initiation procedure, Data Collection Reporting procedure, and Handover Preparation procedure.

- For multi-hop support, an open issue arises on how a source node (e.g., gNB0) should send a multi-hop prediction to neighbor gNBs. Two options can be considered: Hop-by-hop and Point-to-multipoint. In hop-by-hop transfer, one concern is the need for every gNB along the predicted path to maintain UE context and forward trajectory-related feedback back to the originating source gNB. This may result in excessive UE context maintenance overhead.

- Option 1: Hop-by-hop with AI/ML-capable Anchor Nodes

- If a gNB along the path has an AI/ML model, it can act as an anchor node, updating/refining the prediction locally. Feedback and data transfer are localized only until the next AI/ML-capable node, avoiding long backhaul of UE context.

- If a target gNB has an AI/ML model implemented and is capable of performing trajectory prediction/collection, then:

- The AI/ML-capable gNB can update or refine the trajectory prediction based on latest UE measurements.

- Data transfer and feedback forwarding may be restricted only until the next AI/ML-capable gNB, instead of back to the originating source gNB.

- This creates anchor points in the trajectory path where predictions are refreshed and responsibility for maintaining UE context is localized.

- Advantages:

- Reduces the burden of maintaining long UE context chains across multiple gNBs.

- Feedback handling becomes localized, improving scalability.

- Allows AI/ML-capable gNBs to refine predictions dynamically based on updated measurements.

- Proposal 7: Introduce the concept of AI/ML-capable anchor nodes, where multi-hop trajectory prediction/collection responsibility can be localized and refreshed, reducing the need to forward data and feedback to the originating source node.

- Introduce a mechanism for a gNB to indicate to other nodes whether it has implemented an AI/ML model for UE trajectory prediction/collection. This enables neighboring nodes to identify AI/ML-capable anchor nodes and optimize multi-hop trajectory transfer and feedback procedures.

- Proposal 8: Include an IE indicating gNB AI/ML trajectory prediction capability during Xn Setup/Update or NG Setup/Update procedures.

- Mobility Class–based Refinement:

- In some cases, an AI/ML-capable anchor node may be 5+ hops away from the source gNB. Depending on UE mobility class:

- Pedestrian / Low-speed vehicular: Full 5-hop prediction exchange and feedback is unnecessary → may restrict to 2 hops only.

- High-speed vehicular (e.g., train): Longer multi-hop prediction exchange (≥5 hops) is beneficial.

- Proposal 9: Consider mobility class–based limitation of prediction exchange depth (e.g., restrict to 2 hops for low-mobility UEs, extend for high-speed UEs).

- Conclusion

In this contribution, we focused on open issues for Multi hop UE trajectory and provided our view. The following proposals are made:

Issues in Extending Rel-18 Framework to Multi-hop UE Trajectory

Proposal 1: The mechanism for transferring predicted UE trajectory information to multiple target gNBs needs to be defined.

Proposal 2: The configuration of UE trajectory prediction/collection should support transfer to multi-hop target gNBs.

Proposal 3: A feedback mechanism from multi-hop target gNBs back to the source gNB should be introduced.

UE Assistance Information for Multi-hop Trajectory Prediction

Proposal 4: Consider destination/route information as an additional input for multi-hop UE trajectory prediction.

Proposal 5: Evaluate NEF-based application-layer exposure as a short-term mechanism for delivering mobility intent to the RAN.

Proposal 6: Study the feasibility of direct RAN/NAS signaling of mobility intent as a potential long-term (e.g., 6G) solution.

Mechanisms for Multi-hop UE Trajectory Prediction Transfer

- Proposal 7: Introduce the concept of AI/ML-capable anchor nodes, where multi-hop trajectory prediction/collection responsibility can be localized and refreshed, reducing the need to forward data and feedback to the originating source node.

- Proposal 8: Include an IE indicating gNB AI/ML trajectory prediction capability during Xn Setup/Update or NG Setup/Update procedures.

- Proposal 9: Consider mobility class–based limitation of prediction exchange depth (e.g., restrict to 2 hops for low-mobility UEs, extend for high-speed UEs).

- References

- RP-252867, Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3, #RAN 109, Sep 2025.

- TS 38.300, NR and NG-RAN Overall Description; Stage 2, Rel 18 V 18.6.0, 2025.

- TR 37.817-Study on enhancement for Data Collection for NR and EN-DC, Rel 17, V17.0.0, 2022.