3GPP TSG-RAN3 Meeting #129bis                                                              R3-256571

# Prague, Czech Republic, Oct 13 – 17, 2025

Agenda Item:	10.2.1

Source: 	Qualcomm Inc, Charter Communications, T-Mobile USA, Verizon Wireless, KT Corp, Tejas Networks, Fujitsu, Rakuten, NTT DOCOMO, JIO Platforms, Reliance JIO, FiberCop, CEWiT, Telstra

Title: 	6G RAN general principles and requirements

Document for:	Discussion/Agreement

# Introduction

RAN#108 agreed 6G Study Item for RAN WGs and revised in RAN#109 [1]. 6G SI objectives relevant to extract the 6G RAN Architecture are highlighted below. As starting point, we can use these highlighted points to start identifying 6G RAN Architecture Requirements for both 6G TN and NTN networks. Note that some 6G SID objectives not necessary for RAN Architecture discussions are not copied here.

The detailed objectives of the study are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914]

- PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4]

- Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1]

- Aspects of integration with communication services [RAN1]

- higher layer procedures and protocol aspects [RAN2]

- RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

Note: RAN1 identify detailed requirements, if triggered by TSG RAN

Following are initial 6G Requirements for architecture and migration extracted from pCR (RAN#109) for TR 38.914 [3], these requirements will continue to be updated further in future RAN TSG discussions.

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

In this document, we discussed multiple proposals to trigger RAN3 discusisons to identify general requirements for  6G  RAN Architecture and intefaces design.These are not exhaustive 6G RAN Architecture Requirements and which will be evolved further as we make progress of 6G SID. In this document, we did not discuss any specific requirements related to Migration from 5G to 6G and Inter-RAT mobility scenarios.

# 6G RAN General Principles and Requirements

## 6G RAN Deployment Models:

In 5G RAN commercial deployments, both split gNB and non-split gNB RAN deployments are supported. 5G MNOs will continue to support same RAN deployments in 6G with the same RAN deployment models using both split RAN node and non-split RAN node deployment models. In order to continue to support same RAN deployment scenarios for 6G, we believe that 6G RAN shall continue to support same RAN deployment scenarios in case of Indoor Hotspots, Rural, Urban (macro), Dense Urban (macro) and sub-unban (macro).

## Split 6G RAN NF Deployment:

Similar to gNB, 6GR shall support disaggregation of the RAN network functions (NFs) . Different split options between 6G RAN NFs like the one of Central Unit and Distributed Unit of 6G RAN nodes may be possible. Higher layer split between CU and DU as specified in 5G NR can be considered as baseline and any additional optimizations can be further discussed. Lower layer split will not specified in 3GPP and way forward was agreed in RAN#109, RP- RP-252881 [7].

Following are typical example deployment scenarios:

- CU can be deployed in cloud, DU can be deployed at the cell site and RF components at the top of the cell tower.

- CU can be deployed deep in the cloud, DU can be deployed in edge cloud to provide better scheduling latency and also allows to make use of cloud computing power by sharing common pool of hardware resources across multiple DUs and RF components can be deployed at cell site.

- In another deployment, both CU + DU can be deployed together in cloud as NFV and flexible sharing of cloud compute resources and RF components at the cell site.

5G Split gNB architecture allowed operators the flexibility to select from various deployment models based on their specific needs.  It also enables flexible usage of shared or distributed hardware resources across multiple CUs or DUs. Technical analysis of various split options were discussed in 5G TR 38.801[6] and can be considered as baseline for 6G RAN discussions as well.

6G RAN node deployments can be either co-located or non-collocated with 5G gNB deployments can be considered for this scenario.

Figure 1: Split 6G RAN NF deployment

## Non-Split 6G RAN NF Deployment (i.e.,Aggregated):

In this scenario, the RAN NFs   are supported at the 6G RAN node or 6G RAN node may be split into lower layers split of 6G RAN node as agreed in [7].  It is assumed that the 6G RAN node is able to connect to other 6G RAN nodes or 5G gNBs connected to 5GC via appropriate direct/indirect RAN interfaces.

Figure 2: Non-Split 6G RAN NF deployment

6G RAN Architecture shall support both upper layer split and non-split  RAN NF deployment scenarios.

## Heterogeneous 6G RAN Deployment:

In 5G heterogeneous deployments, there exists both macro and small cell deployments. Macro deployments will provide wide area coverage layer using low bands (FR1)/Mid bands (FR3) and small cell deployments in low (FR1) or mid-bands (FR3) or high frequency (FR2) can be used for both indoor hotspots as well as in urban-macro deployment scenarios. In certain cases, macro cells and small cells may also operate in the same frequency band, enabling hotspot microcells within the macro coverage layer to share spectrum resources.  In 6G RAN, same macro, small cell mixed deployments are expected to continue in FR1, FR2 and new FR3 bands. Thus, from 6G RAN Architecture perspective both macro and small cell deployments shall be supported.

Figure 3: Heterogeneous 6G RAN deployment including both Marco + Small cells

6G RAN Architecture shall support both macro and small cell deployment scenarios to enable heterogeneous deployments for both indoor and outdoor deployment scenarios.

## 6G RAN Sharing Deployments:

Like in 5G NR, 6G RAN should support shared RAN deployments to enable multiple 6G core network operators to share same 6G RAN network for efficient utilization of radio spectrum and RAN infrastructure. The Shared 6G RAN could cover large geographical areas, as in the case of national or regional network sharing. In 5G RAN, as we know MOCN (Multi Operator Core Network) and RAN Sharing with  Multiple Cell IDs are supported and same RAN sharing mechanism shall be supported for 6G RAN deployments as baseline RAN sharing mechanism.

Figure 4: Shared 6G RAN deployment

6G RAN shall support RAN sharing mechanisms same as in 5G RAN Sharing (i.e., MOCN and  RAN Sharing with Multiple Cell Id)

## Cloud Native support for 6G RAN:

During 4G and 5G deployment days, cloud network deployments were in initial stages of evolution. Now cloud deployments are matured and 5GC already supported cloud native designs from Rel-15 and many 5G SA deployed 5GC networks are fully deployed in cloud environment with cloud native 3GPP interfaces. When 5G study started in R14 time frame, cloud based technologies were still evolving and so 5G RAN used old legacy 3GPP interfaces without any evolution. 3GPP RAN eco-system is still stuck in old 2G/3G days RAN interfaces and many protocols (ex: SCTP, GTP) are not cloud friendly and did not evolve for many years. This is causing 3GPP MNOs networks falling behind the fast growing cloud networks (deployed by many hyper scalers) and not able to take advantage of many cloud native technologies/solutions for the benefit of 3GPP MNO evolution. Using Cloud Native deployments, MNOs will be able to take advantage of latest cloud native available services/technologies (ex: Containerization, Kubernetes, Dockers, best in class MLOps etc) to improve operational efficiency and ease of  deployment of new services with minimum or no impact to existing services.

Since 6G networks are going to be used for next 15-20 years starting from ~2030, it is time for 3GPP 6G RAN networks to start supporting cloud native interfaces and protocols to enable operators start deploying 6G RAN networks in cloud infrastructure. 6G RAN shall enable network function virtualization to reap the benefits of independent software and hardware evolution. Cloud native RAN deployments will allow MNOs to make use of COTS (Commercial Off-The-Shelf hardware) available in the market and deploy RAN network functions as virtual software functions.

Thus, we propose

6G RAN network functions and interfaces shall allow network function virtualization in cloud native deployments to enable RAN, Core, OAM to be deployed in (multi)-cloud environment.

6G RAN architecture shall allow flexible deployment that enables introduction of new 6G services in flexible manner during life cycle of 6G RAN with minimum or not impacting already deployed interface functions/services.

## Open standard interfaces support for 6G RAN:

The main goal of 3GPP specifications is to specify open standard based interfaces and support true multi-vendor inter-operability between various network nodes. This is of paramount importance for all 6G RAN network node interfaces and between RAN and CN. OAM based configuration shall be minimized and enable 3GPP standard based signalling procedures for various configurations. RAN WGs shall not assume to have common OAM between 6G RAN and Core Network , but they have to be easily integrated.

For all RAN internal interfaces and between RAN-CN interface protocol stacks, each protocol for any interface and application protocols shall be allowed to evolve independently over the life span of 6G and not stuck with a single protocol version defined in initial release of 6G standardization. If new protocol versions are not supported between transmitter and receiver nodes, then it shall always be possible to fallback to basic version of protocol specification. This approach allows each protocol to evolve independently and 3GPP stacks will continue to take advantage of latest available protocol versions.

6G RAN shall support inter-vendor inter operable interfaces between RAN nodes and between RAN-CN.

6G RAN shall allow plug and play mechanism between different RAN network functions.

6G RAN protocol stack shall allow support for evolution of each protocol layer over the 6G lifespan.

## 6G RAN Awareness of Services:

5G introduced features like XR PDU Set and RAN aware QoE features to make RAN aware of application type and application QoS characteristics to allow efficient RAN scheduling to improve the application performance. In 6G, in order to make RAN aware of application type, application characteristics and user experience of application is important for RAN efficient scheduling, optimize network resource allocation to maximize system coverage & capacity and to achieve improved user experience for a given application. Thus, RAN should study necessary mechanisms needed to accomplish the goal of 6G RAN awareness of various services.

6G RAN framework shall allow RAN awareness of various services to enable real time service observability and service performance optimization.

## AI/ML support for 6G RAN:

In 6G era, it is essential to support AI native designs for RAN protocols, architecture and various RAN features to improve signalling efficiency, minimize failure scenarios, efficient network operations, to improve network energy efficiency, reduced UE power consumption, improved system coverage, improved system capacity etc. Thus, 6G RAN shall support native AI/ML framework by considering end to end AI/ML support from both protocols and architecture perspective for various features from day 1.

6G RAN shall make use of native AI/ML frame work for various functions to improve RAN functionality and performance.

## 6G RAN Automatic Configuration and Optimization:

In 4G and 5G, various SON features were introduced in multiple releases to enable automatic self configuration of networks and optimization of networks for various features based on UE provided SON measurement reports. 6G RAN networks shall ensure the path towards autonomous network framework, hence allow support for 6G SON features from day 1 to support self-configuration and optimization of 6G RAN networks is essential.

6G RAN design shall allow self-organization and performance optimization of various features using standardized signaling frame work (ex: using SON like mechanism).

# Summary

Based on above discussion, we have the following proposals for 6G RAN Architecture Requirements.

Proposal 1.	6G RAN Architecture shall support both upper layer split and non-split  RAN NF deployment scenarios.

Proposal 2.	6G RAN Architecture shall support both macro and small cell deployment scenarios to enable heterogeneous deployments for both indoor and outdoor deployment scenarios.

Proposal 3.	6G RAN shall support RAN sharing mechanisms same as in 5G RAN Sharing (i.e., MOCN and  RAN Sharing with Multiple Cell Id)

Proposal 4.	6G RAN network functions and interfaces shall allow network function virtualization in cloud native deployments to enable RAN, Core, OAM to be deployed in (multi)-cloud environment.

Proposal 5.	6G RAN architecture shall allow flexible deployment that enables introduction of new 6G services in flexible manner during life cycle of 6G RAN with minimum or not impacting already deployed interface functions/services.

Proposal 6.	6G RAN shall support inter-vendor inter operable interfaces between RAN nodes and between RAN-CN.

Proposal 7.	6G RAN shall allow plug and play mechanism between different RAN network functions.

Proposal 8.	6G RAN protocol stack shall allow support for evolution of each protocol layer over the 6G lifespan.

Proposal 9.	6G RAN framework shall allow RAN awareness of various services to enable real time service observability and service performance optimization.

Proposal 10.	6G RAN shall make use of native AI/ML frame work for various functions to improve RAN functionality and performance.

Proposal 11.	6G RAN design shall allow self-organization and performance optimization of various features using standardized signaling frame work (ex: using SON like mechanism).

# References

[1] RP-252912: New SID: Study on 6G Radio (RAN#109)

[2] RP-251873:Study on 6G Scenarios and Requirements (RAN#108)

[3] RP-252870: pCR for 38.914 on requirements for architecture and migration for 6G (RAN#109)

[4] RP-252887: pCR for 38.914 on Deployment scenarios for 6G (RAN#109)

[5] RP-252882: TR 38.914 v0.2.0 Study on 6G Scenarios and Requirements (RAN#109)

[6] TR 38.801: Radio access architecture and interfaces (for 5G in Rel-14))

[7] RP-252881: Way forward on fronthaul in 6G (RAN#109)

[8] RP-252873: Summary of 6G Diverse Device Types (including Minimum UE Bandwidth)

# Appendix: for discussion in upcoming meetings

## 6G RAN support for muti-hop network topologies:

During 5G RAN deployments, in order to support coverage extension and reliability, multiple solutions were developed. Example: IAB, WAB, NCR etc. In 6G RAN deployments to support coverage extension, some operators have expressed interest to support simple solutions like Reflective Intelligent Surface (RIS), Network Controlled Repeater (NCR) and Wireless Access Backhaul (WAB) without complex IAB nodes. 6G RAN Architecture shall allow flexibility to introduce simple solutions for coverage extension, support for reliability using multi-hop network node deployments.

- Proposal : 6G RAN Architecture shall support multi-hop network topologies to support resilience and coverage extension.