3GPP TSG-RAN WG3 Meeting #129bis                                                             R3-256574

Prague, Czech Republic, 13 – 17 October 2025

Agenda item: 		10.2.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on 6G RAN logical architecture

Document for:		Discussion

# 1	Introduction

TSG RAN#108 agreed on a new SI on 6G Radio. The SID contains the following objectives in RAN3 scope [1]:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. … - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] … - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML TSG#112 (June/2026): RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split.

TSG RAN#109 further agreed the following requirements for architecture and migration for 6G [2].

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

This document discusses the 6G RAN logical architecture.

# 2	Discussion

The 6G RAN logical architecture can be regarded as an evolution of the 5G RAN logical architecture. As in the 5G logical architecture, the 6G RAN should support a 6G RAN-node. Details of the 6G RAN-node’s functional split are FFS. The terminology of the 6G RAN-node is FFS.

Proposal 1: The 6G RAN to support the 6G RAN-node. FFS on the details of the 6G RAN-node's functional split(s). FFS on the terminology of the 6G RAN-node.

As in 5G, the 6G RAN-node supports an interface with the UE. This interface will be defined by RAN2.

Proposal 2: The 6G RAN-node to support an interface with the UE.

As defined by the SID [1] and TSG RAN agreements [2], the 6G RAN-node should support standalone (SA) operation. Whether the 6G RAN-node can also support non-SA operation is up to further discussion by TSG RAN.

Proposal 3: The 6G RAN-node to support SA operation. Whether the 6G RAN-node supports NSA is up to TSG RAN discussion.

In SA operation, the 6G RAN-node supports an interface with the 6G CN. As in 4G and 5G, the 6G RAN node should also support an interface with peer 6G RAN-nodes. The nature of the 6G RAN-to-CN interface and the inter-6G-RAN-node interface, i.e., whether they can be represented as point-to-point or service-based interfaces, is part of the 6G Study and discussed in other contributions.

Proposal 4: In SA operation, the 6G RAN-node to support an interface with the 6G CN and with peer 6G RAN-nodes. FFS the nature of these interfaces, e.g., whether they are point-to-point or service-based.

Figure 1 shows the RAN logical architecture based on the above proposals.

Figure 1: RAN logical architecture

It can be assumed that 6G RAN supports at least a subset of the functions defined for a SA 5G RAN-node in TS 38.300 and TS 38.401. RAN3 needs to determine, which of these 5G RAN-node functions need to be adopted for 6G RAN on Day 1.

Proposal 5: RAN3 to study, which functions defined for the SA 5G RAN-node need to be adopted for 6G RAN on Day 1.

In 5G, the UE radio capabilities are stored in the CN and passed between RAN nodes. The UE’s context is stored in different locations based on the UE’s RRC connectivity state:

  - For a UE in RRC CONNECTED mode, UE context is stored in the gNB-CU and gNB-DU.

  - For a UE in RRC INACTIVE mode, some of the UE context is stored in the UE’s last gNB-CU, where the UE was RRC-connected last.

  - For a UE in RRC IDLE mode, UE context is released and regenerated when the UE reconnects.

Further, UE logged measurements are stored at the gNB-CU.

For 6G, RAN3 should study whether the 6G RAN should support a RAN-based NF for the storage of UE context, UE radio capabilities, and/or other information, and/or other purpose. This study has interdependence with SA2 and RAN2.

Proposal 6: RAN3 to study whether the 6G RAN should support other RAN-based network functions, e.g., for the storage of UE context, UE radio capabilities, and/or for other information or other purposes. This study has interdependence with SA2 and RAN2.

# 3	Conclusion

This contribution discussed the 6G RAN logical architecture. The following proposals have been made:

Proposal 1: The 6G RAN to support the 6G RAN-node. FFS on the details of the 6G RAN-node's functional split(s). FFS on the terminology of the 6G RAN-node.

Proposal 2: The 6G RAN-node to support an interface with the UE.

Proposal 3: The 6G RAN-node to support SA operation. Whether the 6G RAN-node supports NSA is up to TSG RAN discussion.

Proposal 4: In SA operation, the 6G RAN-node to support an interface with the 6G CN and with peer 6G RAN-nodes. FFS the nature of these interfaces, e.g., whether they are point-to-point or service-based.

Proposal 5: RAN3 to study, which functions defined for the SA 5G RAN-node need to be adopted for 6G RAN on Day 1.

Proposal 6: RAN3 to study whether the 6G RAN should support other RAN-based network functions, e.g., for the storage of UE context, UE radio capabilities, and/or for other information or other purposes. This study has interdependence with SA2 and RAN2.

# 4	References

[1]  RP-251881, 3GPP TSG RAN #108, Prague, Czech Republic, June 2025. Further updates in RP-25912, 3GPP TSG RAN #109, Beijing, China, September 2025.

[2]  RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G, 3GPP TSG RAN #109, Beijing, China, September 2025.