3GPP TSG RAN WG3#129bis	R3-256587

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	13.3

Source: 	China Telecom, BUPT

Title: 	On RAN-CN procedures for supporting gNB-based sensing

Document for:	Discussion and decision

- Introduction

The 3GPP RAN#108 has approved a new SID, “Study on Integrated Sensing And Communication (ISAC) for NR” [1] as part of the Rel-20 5G NR advanced. The detailed focus of the study is as follows:

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

And at RAN1#122, evaluation aspects for ISAC have been discussed and some evaluation parameters have been agreed, the SI progress reached 15%. In this meeting, RAN3 start the discussion on the network architecture, procedures and signalling for gNB-based mono-static sensing.

In this paper, we will focus on the study of RAN-CN procedures to enable gNB-based sensing and fulfil key sensing service requirements.

- Discussion

Although the sensing network architecture between the RAN and 5GC may involve different options, the sensing procedures between them are quite similar across these architectural variants. These procedures generally include the RAN-CN sensing interface management procedures between RAN and CN, as well as the procedures for sensing service session setup, modification, release, and sensing data reporting. All procedure signalling is non-UE associated signalling.

Terms used within the discussion:

Sensing gNB: Indicating a gNB capable of serving as a sensing transmitter and/or as a sensing receiver.

Sensing Function (SF): Indicating the logical function which is involved to support Sensing Service in 5GC.

2.1 RAN-CN Sensing Interface Management Procedures

The RAN-CN sensing interface manage procedures is to manage a connection between RAN and 5GC for exchanging the parameters required for interface connection and data transmission needed for sensing service support.

The RAN-CN sensing interface setup procedures should include the following procedures:

- Sensing interface setup procedure: The procedure is triggered by RAN (Sensing gNB) to establish a connection with 5GC to exchange the parameters required for interface connection and data transmission, such as the user plane address, transport protocols and security information required to the Sensing gNB.

- Interface reset procedure: In the event of a failure in 5GC or RAN, 5GC could trigger the interface reset procedure to initialize or re-initialize the RAN-CN interface connection. This procedure does not affect the configuration data exchanged during the interface setup procedure.

The interface configuration update procedure facilitates the updating of configuration data on the RAN-CN interface to ensure proper interoperability for supporting the sensing service. Since both 5GC and NG-RAN may have the sensing-related configuration change, the procedure should be triggerable by either entity. It should include two following procedures:

- NG-RAN configuration update procedure: the procedure is triggered by RAN (Sensing gNB) to update interface configuration data.

- 5GC configuration update: the procedure is triggered by 5GC (AMF or SF) to update configuration data.

Observation 1: Within the sensing network architecture, the design of the sensing interface management procedures depends on the connection point in the 5GC. If the RAN connects to AMF, the existing NG interface management procedures can be reused, requiring only the extension of messages with new parameters. Conversely, if the RAN connects directly to the SF, an entirely new set of signalling procedures must be designed.

2.2 RAN-CN Sensing Service Session Management Procedures

The RAN-CN Sensing Service Session Management Procedures are responsible for establishing, modifying, and releasing a sensing session. The procedures provide the sensing service requirements, measurement and reporting configurations, and the necessary NG-RAN sensing resources to fulfil them between the 5GC and NG-RAN.

The RAN-CN sensing service management procedures should include at least the following:

- Sensing Service Session Setup: In this procedure, the SF or AMF sends a Sensing Service Session Setup request to the Sensing gNB. This request message should contain the necessary configurations for the Sensing gNB to carry out the sensing operations. It may also include the specific information on how the sensing data/results should be reported, e.g. the periodicity, or the criteria for an event report. Upon receiving the setup request, if the Sensing gNB has the requisite capability and available resources, it shall execute the sensing service configuration and allocate corresponding resources. Then the sensing gNB shall send the Sensing Service Session Setup Response message to 5GC, it may also indicate if the Sensing gNB needs to process the sensing data and generate sensing result or partial result in the report.

- Sensing Service Session Modify: This procedure is triggered by the SF or AMF when it needs to modify the configuration of an established sensing service session.

- Sensing Service Session Modify Indication: The procedure allows the Sensing gNB to request modification of the established sensing service session(s).

- Sensing Service Session Release: This procedure enables the release of an established Sensing Service Session configuration and its associated NG-RAN resources. It is triggered by the SF or AMF.

When a direct connection between Sensing gNB and SF exists, the messages related to the sensing service Session management procedures are exchanged between SF and sensing NG-RAN. If NG-RAN node is connected to AMF, the AMF shall act as a relay to forward the messages between SF and Sensing gNB.

2.3 RAN-CN Sensing Data Report Procedure

The RAN-CN Sensing Data Report Procedure is used to deliver sensing data from the Sensing NG-RAN to the 5GC.

RAN-CN Sensing Data Report: This procedure is triggered by the sensing gNB. Based on the configuration derived from the initial sensing service request, the Sensing gNB performs sensing operations to obtain 3GPP-defined sensing data. It then provides this data to the Sensing Function, typically over a direct connection. Reports can be generated periodically or based on event triggers, as configured by the network.

Observation 2: The Sensing Service Session Management and RAN-CN Sensing Data Report Procedures are novel and require a new design, irrespective of the sensing network architecture (i.e., with or without a direct RAN-CN interface).

Proposal: The following procedures be studied as core requirements for supporting the sensing service: RAN-CN Sensing Interface Management, Sensing Service Session Management, and RAN-CN Sensing Data Reporting.

- Conclusion

Based on the above discussion on RAN-CN procedures for supporting gNB-based sensing, we provide the following observations and proposals:

Observation 1: Within the sensing network architecture, the design of the sensing interface management procedures depends on the connection point in the 5GC. If the RAN connects to AMF, the existing NG interface management procedures can be reused, requiring only the extension of messages with new parameters. Conversely, if the RAN connects directly to the SF, an entirely new set of signalling procedures must be designed.

Observation 2: The Sensing Service Session Management and RAN-CN Sensing Data Report Procedures are novel and require a new design, irrespective of the sensing network architecture (i.e., with or without a direct RAN-CN interface).

Proposal: The following procedures be studied as core requirements for supporting the sensing service: RAN-CN Sensing Interface Management, Sensing Service Session Management, and RAN-CN Sensing Data Reporting.

- Reference

- RP-252819, Study on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, CTC.

- TS 38.300, NR and NG-RAN Overall description; Stage-2 Rel-19, 3GPP.

- TS 23.501, System architecture for the 5G System (5GS); Stage 2 Rel-19, 3GPP.

- TR 23.700-14, Study on Integrated Sensing and Communication; Stage 2 Rel-20, 3GPP.

- R3-256589, draft TR38.765 Study on Integrated Sensing and Communication (ISAC) for NR, CTC, Xiaomi.