3GPP TSG-RAN WG3 Meeting #129bis	R3-256591

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	[TP to BL pCR] Consideration on RAN-CN interface options

Agenda Item:	10.3.2

Source:	Huawei, Jio Platforms

Document for:	Agreement

# Introduction

In the 6G SID [1], the following bullet can be found:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

And it is required for RAN3 to provide interim study results to allow TSGs to make a decision on the RAN-CN interface: P2P vs SBI by June 2026.

In this contribution, we discuss the RAN CN interface options.

# Discussion

In 4G and 5G, both the S1 interface and NG interface are designed as point-to-point interface, and the point-to-point logical interface is feasible even in the absence of a physical direct connection.

The 4G/5G RAN-CN control plane interface seperates the Radio Network Layer and the Transport Network Layer. S1AP and NGAP were introduce as the Radio Network Layer for the respective generation. The transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP.

The 4G/5G RAN-CN user plane interface is based on GTP-U tunnels over UDP/IP, e.g., the NG-U interface provides non guaranteed delivery of PDU Session user plane PDUs.

(a) NG Interface Control Plane 							(b) NG-U protocol structure for PDU/MBS Session

Figure 1: Control plane and user plane of NG interface

This control plane interface and user plane interface design fulfilled the RAN-CN interface requirements in 4G and 5G, and they provide reliable signalling transmission and support large volume of user data transmission.

## RAN-CN control plane interface

## P2P vs SBI

Based on previous discussions in RAN, there are two potential solutions for the RAN-CN control plane interface:

- Solution 1: P2P-based interface

- Solution 2: Service-based interface

NG interface is a P2P interface between the gNB and AMF, and NG flex is supported enabling a gNB to connect with multiple AMFs within the same reigion. With P2P interface design, a clear RAN CN function split is achieved, for example, for the signalling processing of a UE, the gNB only needs to route the relevant signalling to the UE’s serving AMF, which, when necessary, communicates with the SMF/LMF and other CN functions. The gNB does not need to understand the processing sequence and logic among CN functions.

Observation 1: P2P-based interface, ensures clear RAN CN function-split, with the clear boundary the RAN node does not need to understand the processing sequence and logic among CN functions.

Some companies may consider using service-based interface to enable gNBs to communicate directly with the new CN function. It is noteworthy that in the Ambient IoT of Rel-19, the NG interface was also used between gNB and AIOTF. Therefore, if necessary, a direct connection between the 6G RAN and new core network functions can also be achieved through the P2P RAN-CN interface.

Observation 2: P2P-based interface, also allows direct connectivity between the 6G RAN and 6G new CN function, if needed.

Although service-based interface is used among CN functions in 5G, but it is another story when talking about RAN-CN interface. As there are large scale of RAN nodes comparing to the number of core network nodes, RAN's complex and tightly coupled functionalities, along with its large scale, lead to challenges in registration, discovery, and API definition.

As mentioned abover, service-based interface requires increased RAN-CN interactions which degrade signaling performance, while unclear boundaries between RAN and CN result in extensive cross-domain data exchange within APIs. Determining when to perform parallel operations and when to perform sequential operations will significantly increase the RAN processing complexity.

Take mobility as an example, in Xn based handover, once the UE accesses to the target gNB, the target gNB only needs to send a Path Switch Request to the UE’s serving AMF, then the AMF will contact with the SMF(s), the SMF(s) will further contact with UPF(s), but in case service based interface is used between 6G RAN and CN, the gNB will need to understand which CN functions need to contact and in which sequence, adjust the logic when some of the peer CN functions fail the request while some others not, etc.

Observation 3: If service-based interface is used, due to the large scale of RAN nodes, the RAN-CN interactions will be largely increased and will lead to signaling performance deterioration, while unclear boundaries between RAN and CN will lead to extensive cross-domain data exchange within APIs and significantly increase the RAN processing complexity.

Based on above observations, we do not see clear motivation to support service-based interface for 6G new service/functions.

Proposal 1: For the 6G RAN-CN control plane interface, RAN3 to discuss and agree to use P2P-based interface.

## P2P interface options, SCTP vs QUIC

For P2P based interface, there are two potential options:

- Option a: NGAP* over SCTP

- Option b: NGAP* over QUIC

SCTP is the transport layer protocol used in all RAN3 designed control plane interfaces in legacy systems, it provides high reliability and performance for telecommunication system, its key characteristics can be summaried as follows:

- Reliable, In-Sequence Delivery

- Message-Oriented and Connection-Oriented

- Multi-Streaming, reducing head-of-line blocking

- Multihoming, to achieve transport network redundancy

- Four-Way Handshake, to mitigate denial-of-service (DoS) attacks

Observation 4: SCTP is originally developed for transporting telecommunication signaling over IP networks, benefits applications requiring high reliability and performance.

QUIC is a transport layer protocol introduced for modern internet applications, its key characteristics can be summaried as follows:

- Integrated Security, natively incorporates Transport Layer Security (TLS 1.3)

- Fast Connection Establishment, reduces the number of round trips

- Multiplexing and Streams

- QUIC connections are not tied to specific IP addresses or ports, can reduce TNLA handling

Observation 5: QUIC is originally developed for modern internet applications, with faster connection establishment, and its connections are not tied to specific IP addresses or ports.

Proposal 2: For the transport network layer of 6G RAN-CN control plane interface, RAN3 further compare the pros and cons for SCTP and QUIC.

## RAN-CN user plane interface

GTP-U tunnel over UDP/IP is used as the user plane interface solution for many interfaces, e.g., between gNB and UPF, between gNB and MB-UPF, between eNB and MBMS GW, between gNBs, between eNBs, between gNB-CU and gNB-DU, etc. It provides non guaranteed delivery of PDU Session user plane PDUs, and supports large volume of user data transmission, from our understanding, when discussing the RAN-CN user plane interface for 6G, the GTP-U tunnel over UDP/IP should adopted by default.

Proposal 3: For the transport network layer of 6G RAN-CN user plane interface, GTP-U tunnel over UDP/IP is used.

## RAN-CN Data transfer design

The following objective can also be found in the SID:

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

Data transfer and management are hot topics in both RAN2 and SA2, for new services data which may not suitable to be transmited on legacy control plane or user plane due to some claimed reasons (e.g., large-volume, MNO visibility/ controllability, QoS differentiation between different types of data…), a new data plane is proposed by some companies.

In 5G, there are multiple mechanisms for collecting data for different purposes. However, these different types of data are collected and utilized independently using different mechanisms, and cannot be well integrated or combined. There might be overlapping, duplicated, mixed classification of data, which is not efficient to be utilized by the network, and also difficult to be provided as a service friendly to the data consumers. Additionally, for data with large volume, such as AI models, the collection mechanisms may not flexible enough. Looking ahead to future 6G systems, for the collection of different types of data from various sources for different services and applications, maybe there is an opportunity to establish an end-to-end, unified data management framework, to enable joint collection, management and distribution of data within 3GPP system.

From RAN3 perspective, we can futher discuss potential RAN3 impacts over RAN and CN interface to support the new 6G Data Transfer design, by taking SA2/RAN2 progress into account.

Proposal 4: RAN3 futher study potential RAN3 impacts over RAN and CN interface to support the 6G Data Transfer design, by taking SA2/RAN2 progress into account.

# Conclusion

This document discusses the RAN-CN interface options, get the following observations and proposals:

RAN-CN control plane interface

Observation 1: P2P-based interface, ensures clear RAN CN function-split, with the clear boundary the RAN node does not need to understand the processing sequence and logic among CN functions.

Observation 2: P2P-based interface, also allows direct connectivity between the 6G RAN and 6G new CN function, if needed.

Observation 3: If service-based interface is used, due to the large scale of RAN nodes, the RAN-CN interactions will be largely increased and will lead to signaling performance deterioration, while unclear boundaries between RAN and CN will lead to extensive cross-domain data exchange within APIs and significantly increase the RAN processing complexity.

Proposal 1: For the 6G RAN-CN control plane interface, RAN3 to discuss and agree to use P2P-based interface.

Observation 4: SCTP is originally developed for transporting telecommunication signaling over IP networks, benefits applications requiring high reliability and performance.

Observation 5: QUIC is originally developed for modern internet applications, with faster connection establishment, and its connections are not tied to specific IP addresses or ports.

Proposal 2: For the transport network layer of 6G RAN-CN control plane interface, RAN3 further compare the pros and cons for SCTP and QUIC.

RAN-CN user plane interface

Proposal 3: For the transport network layer of 6G RAN-CN user plane interface, GTP-U tunnel over UDP/IP is used.

RAN-CN Data Transfer design

Proposal 4: RAN3 futher study potential RAN3 impacts over RAN and CN interface to support the 6G Data Transfer design, by taking SA2/RAN2 progress into account.

# Reference

[1] RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] RP-2529093 Summary of Discussion on 6G Requirements for architecture and migration, NTT DOCOMO, INC. (Rapporteur)

# Text Proposal for 6G TR (based on R3-255517)

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

# 6	RAN Architecture and Interfaces

Editor’s note: The aim of this section is to describe all aspects related to the Interface between gNB to CN.

## 6.1	General Principles and Requirements

## 6.2 		RAN-CN interface

### 6.2.1	RAN-CN interface options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services and include control plane and user plane aspects considering interim milestone.

#### 6.2.1.x		RAN-CN interface Control Plane

The RAN-CN interface control plane is a point-to-point interface between an RAN node and a CN node.

The following options should be considered in the discussion on RAN-CN interface control plane transport network layer. NGAP* should be understood as a similar protocol of NGAP which is taken as a basis for further discussion.

Option 1: NGAP* over SCTP

Figure 6.2.1.x-1: RAN-CN Interface Control Plane protocol structure – Option 1

SCTP is the transport layer protocol used in all RAN3 designed control plane interfaces in legacy systems, it provides high reliability and performance for telecommunication system, its key characteristics can be summaried as follows:

-	Reliable, In-Sequence Delivery

-	Message-Oriented and Connection-Oriented

-	Multi-Streaming, reducing head-of-line blocking

-	Multihoming, to achieve transport network redundancy

-	Four-Way Handshake, to mitigate denial-of-service (DoS) attacks

Option 2: NGAP* over QUIC

Figure 6.2.1.x-2: RAN-CN Interface Control Plane protocol structure – Option 2

QUIC is a transport layer protocol introduced for modern internet applications, its key characteristics can be summaried as follows:

-	Integrated Security, natively incorporates Transport Layer Security (TLS 1.3)

-	Fast Connection Establishment, reduces the number of round trips

-	Multiplexing and Streams

-	Connections not tied to specific IP addresses or ports, can reduce TNLA handling

#### 6.2.1.y		RAN-CN Interface User Plane

The RAN-CN interface user plane protocol stack is shown on Figure 6.2.1.y-1. The transport network layer is built on IP transport and GTP-U is used on top of UDP/IP to carry the user plane PDUs between the RAN node and the CN node.

Figure 6.2.1.y-1: RAN-CN Interface User Plane protocol structure

### 6.2.2	Evaluation of RAN-CN interface options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 5.2.1

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>