3GPP TSG-RAN WG3 Meeting #129bis	R3-256600

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:	11.2.2

Source:	Nokia

Title:	(TP for SON BL CR for TS 38.300) MRO Enhancements for Conditional Intra-CU LTM

Document for:	Approval

# 1	Introduction

This is an initial contribution related to MRO enhancement for Conditional intra-CU LTM in Rel.20.

# 2	Discussion

## 2.1 MRO for Conditional intra-CU LTM

### 2.1.1 Agreements achieved in RAN Plenary

The agreements achieved in RAN #108 [1]:

The objective of NR_SON_MDT_Ph5-Core WI is to specify data collection enhancement in NR standalone for SON/MDT purpose. The specific objectives of this work item are:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

If needed, co-operate with RAN1, SA5.

## 2.1.2 Discussion

In Fig. 1 an extraction from the 3GPP TS 38.401 showing the inter DU C-LTM is depicted.

Figure 1: Conditional intra-CU LTM (Inter-gNB-DU)

The source cell of source DU sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations including TA validity timer, and the corresponding execution conditions (step 10 – 12). Source cell and each candidate cell provides its own execution condition for conditional LTM. With a NW triggered UL synchronization, the UE performs early TA acquisition (step 13 – 15). The source gNB-DU sends a new MAC CE to the UE which may include TA value of candidate cells in step 16. The main difference compared to Rel. 18 LTM is that there is no Cell Switch Command from source DU. Instead, the executing the LTM cell switch to the given candidate cell is triggered via fulfilment of the given execution conditions communicated to UE in step 10.

The conditional intra-CU LTM may be executed also intra DU scenario. Detailes may be found in TS 38.401.

As part of the NR_SON_MDT_Ph4-Core WI in Rel. 19, MRO for Rel. 18 LTM was provided. The focus was related to enhancement of the RLF report with LTM relevant information. Apart of that RAN3 agreed network-based solution for so called case 2 “RACH less LTM cell switch failure due to wrong beam” and case 3 “LTM failure due to outdated TA”.

Due to fact that for conditional LTM the beam selection is done on UE side and not decided by source DU the case 2 and related network-based monitoring may not be applicable/needed for conditional LTM.

Observation 1: The network-based concept for case 2 may not be applicable/needed for conditional LTM.

Proposal 1: RAN3 agrees to analyse the applicability and needed of Rel. 19 network-based solution for case 2 for conditional LTM.

Regarding the case 3 as Early TA acquisition procedure is applicable also for conditional LTM, the Rel. 19 agreed network-based solution may be beneficial also for conditional LTM. There are however some differences like source DU is not aware of the target cell selected by UE. Also, Conditional LTM may co-exist with MIMO multiple TAs in the case where the target DU has multi-TRP operations. In this case, the TAs are identified by a TAG ID (TA Group ID) and each TA may be configured with a separate TA validity timer. So, when a failure occurs due to invalid/outdated TA value, the network doesn’t know which TA was the culprit.

Observation 2: The network-based concept for case 3 may be not applicable for conditional LTM.

Proposal 2: RAN3 agrees to analyse the applicability of Rel. 19 network-based solution or proposing UE based solution for case 3 for conditional LTM.

For Rel. 18 LTM it was agreed in Rel. 19 to re-use the MRO connection failures defined for standard HO and CHO via adding appropriate text into TS 38.300, chapter 15.5.2.2.2. Similar approach seems to be applicable also for intra-CU conditional LTM.

Proposal 3: RAN3 agrees to re-use the MRO connection failures defined for standard HO and CHO also for conditional intra-CU LTM via adding appropriate text into TS 38.300, chapter 15.5.2.2.2 as proposed in Annex 1.

As part of the NR_SON_MDT_Ph4-Core WI in Rel. the focus was also related to enhancement of the SHR report with LTM relevant information. Apart of that RAN3 agreed network-based solution for so called case 1 “BFR shortly after successful LTM cell switch to the wrong beam”.  Due to fact that for conditional LTM the beam selection is done on UE side and not decided by source DU the case 1 and related network-based monitoring may not be applicable/needed for conditional LTM.

Observation 3: The network-based concept for case 1 may not be applicable/needed for conditional LTM.

Proposal 4: RAN3 agrees to analyse the applicability and needed of Rel. 19 network-based solution for case 1 for conditional LTM.

With introduction of the Conditional Intra-CU LTM even the NW triggered UL synchronization when the UE performs early TA acquisition is s till applicable, the TA value is not sent to UE via Cell Switch Command.  Instead, the TA value if provided by network may be used later after the execution conditions for the given candidate target cell are fulfilled. The validity of the TA value is however limited by TA validity timer.  Too short setting of the TA timer value may lead to RACH based LTM cell switch due to TA timer expiry at UE side instead of RACH less despite the TA value received in RAR message is the same as the one previously received from network. On the other hand, inappropriately too long setting of the TA timer value to situation when the TA value is no longer valid but due to still running TA timer the RACH less LTM switch fails, and UE consequently successfully recovers/re-establishes to the same cell with TA value received in RAR message different to one previously received from network.

Observation 4: Proper setting of the TA timer may be crucial for conditional intra-CU LTM.

Proposal 5: RAN3 agrees to analyse the possible optimization options for tuning the TA timers for conditional intra-CU LTM.

# 3	Conclusions

We have the following observations and proposals:

Observation 1: The network-based concept for case 2 may not be applicable/needed for conditional LTM.

Proposal 1: RAN3 agrees to analyse the applicability and needed of Rel. 19 network-based solution for case 2 for conditional LTM.

Observation 2: The network-based concept for case 3 may be not applicable for conditional LTM.

Proposal 2: RAN3 agrees to analyse the applicability of Rel. 19 network-based solution or proposing UE based solution for case 3 for conditional LTM.

Proposal 3: RAN3 agrees to re-use the MRO connection failures defined for standard HO and CHO also for conditional intra-CU LTM via adding appropriate text into TS 38.300, chapter 15.5.2.2.2 as proposed in Annex 1.

Observation 3: The network-based concept for case 1 may not be applicable/needed for conditional LTM.

Proposal 4: RAN3 agrees to analyse the applicability and needed of Rel. 19 network-based solution for case 1 for conditional LTM.

Observation 4: Proper setting of the TA timer may be crucial for conditional intra-CU LTM.

Proposal 5: RAN3 agrees to analyse the possible optimization options for tuning the TA timers for conditional intra-CU LTM.

# References

- RP-251869, RAN #108

# Annex 1: Text proposal to TS 38.300 (MRO for Conditional Intra-CU LTM)

First change, ommited text not changed

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

In case of Conditional Intra-CU LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late Conditional Intra-CU LTM cell switch, Too Early Conditional Intra-CU LTM cell switch and Conditional Intra-CU LTM cell switch to Wrong Cell.

In case of CHO with candidate SCG(s), the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO with candidate SCG(s), Too Early CHO with candidate SCG and CHO Execution to Wrong Cell.

Next change, ommited text not changed