3GPP TSG-RAN3 Meeting #129-bis	R3-256611

Prague, Czech Republic, 13 – 17 Oct 2025

Agenda Item:	11.2.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on MRO enhancements for Intra-CU conditional LTM

Document for:	Discussion/Decision

# 1 Introduction

In this contribution, we discuss the details on supporting MRO enhancements for Intra-CU conditional LTM to support the following objective in WID of Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5 (RP-252560):

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

If needed, co-operate with RAN1, SA5.

# 2 Discussion

## 2.1 CLTM failure scenarios

Conditional LTM (CLTM) is where the UE is configured with one or more candidate LTM cells and one or more L1-based or L3-based LTM cell switch execution conditions. UE performs either RACH-less or RACH-based LTM cell switch to one of these candidate LTM cells when the CLTM trigger conditions are met. If the trigger conditions are not configured appropriately, UE CLTM may fail due to either Too Late LTM cell switch, Too Early LTM cell switch or LTM cell switch to Wrong Cell, similar to LTM failure scenarios defined in Rel-19.

Proposal 1: LTM failure scenarios defined in Rel-19 (Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell) should now be extended to cover CLTM as well (both RACH-based and RACH-less).

CLTM failure can happen sometimes if the LTM execution conditions (either L3-based or L1-based or both) are not configured properly.

Observation 1: In some cases, CLTM failure can happen due to improperly configured LTM execution (either L3 or L1 or both) conditions. It is beneficial to provide information to the RAN to detect the conditions that are triggered in both successful and failure cases.

Proposal 2: MRO for CLTM should support enhancements to optimize LTM execution (both L3 and L1) conditions configured for initial LTM and subsequent LTM.

## 2.2 CLTM candidate cell TA timer optimization

The gNB configures the ltm-Candidate-TimeAlignmentTimer (referred to as the CLTM TA Timer) via RRC signaling to the UE, which determines how long the UE considers the associated CLTM candidate cells to be uplink time-aligned. Upon receiving the Time Alignment (TA) value for these candidate cells through the LTM Candidate TA Command MAC CE, the UE stores the TA value and starts the CLTM TA Timer. When a CLTM cell switch is triggered for one of the candidate cells, the UE applies the stored TA value to the corresponding pTAG and starts or restarts the timeAlignmentTimer for that pTAG using the remaining duration of the CLTM TA Timer.

Since the CLTM TA Timer governs how long a UE considers candidate cells to be uplink time-aligned, its configuration directly impacts the success of CLTM. MRO enhancements for CLTM should consider optimizing CLTM TA timer to prevent CLTM failures and improving overall cell switch performance in both RACH-based and RACH-less CLTM scenarios.

Observation 2: gNB configures ltm-Candidate-TimeAlignmentTimer (referred to as CLTM TA Timer for simplicity) via RRC to the UE, which controls how long the UE considers the CLTM candidate cells associated with this timer to be uplink time aligned

Observation 3: When a CLTM cell switch procedure is triggered for a CLTM candidate cell, the UE 1)applies the stored TA value associated with the CLTM candidate cell for the pTAG, 2) start or restart the timeAlignmentTimer associated with the pTAG with the length of the remaining time till expiry for the ltm-Candidate-TimeAlignmentTimer.

Proposal 3: MRO for CLTM should handle how to optimize CLTM TA timer

## 2.2 RACH-less CLTM failure scenarios

To support successful execution of RACH-less CLTM, MRO enhancements for detection of the below failure scenarios related to timer expiry should be considered to optimize the CLTM TA timer.

- If the CLTM TA timer expires before the CLTM conditions are met, the RACH-less CLTM cell switch is never initiated and results in missed opportunity of RACH-less CLTM execution.

- If the CLTM TA timer expires while RACH-less CLTM is ongoing (with T304 still running), the UE can “fallback” from RACH-less CLTM to RACH-based CLTM without waiting for T304 expiry. This results in  RACH-less CLTM failure.

- If T304 expires during RACH-less CLTM execution, UE declares RLF and performs RACH-based CLTM recovery (if configured) or performs re-establishment resulting in RACH-less CLTM failure.

- In RACH-less CLTM execution, a successful LTM cell switch occurring very close to the expiry of the CLTM TA timer can result in a minimal TA duration being applied to the pTAG which may result in misalignment of early UL sync.

Proposal 4: MRO for RACH-less CLTM failure should support detection of CLTM TA timer expiry before CLTM conditions are satisfied (RACH-less CLTM was not initiated at all).

Proposal 5: MRO for RACH-less CLTM failure should support detection of CLTM TA timer expires when RACH-less CLTM is ongoing (T304 still running).

Proposal 6: MRO for RACH-less CLTM failure should support detection of T304 expiry when RACH-less CLTM is ongoing (RACH-less CLTM execution failure).

Proposal 7: MRO for RACH-less CLTM failure should support detection of Successful RACH-less CLTM execution, but very close to “CLTM TA timer expiry”, thereby TA timer applied for pTAG is too small.

Additionally, there may be cases where the gNB fails to send a PDCCH order for early UL sync, possibly due to incorrect measurement configurations or implementation issues, even though the UE has reported measurements for the candidate cells. In such scenarios, the UE does not receive a TA value, and the CLTM TA timer is never started. Consequently, when CLTM execution conditions are met, the UE will perform RACH-based CLTM. Supporting detection of this scenario is necessary to minimize RACH-less CLTM execution failures.

Proposal 8: MRO for RACH-less CLTM failure should support detection of RACH-based CLTM execution because early UL sync was not performed towards the candidate cell.

# 3 Summary

Below are the observations and proposals discussed in this document.

Proposal 1: LTM failure scenarios defined in Rel-19 (Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell) should now be extended to cover CLTM as well (both RACH-based and RACH-less).

Observation 1: In some cases, CLTM failure can happen due to improperly configured LTM execution (either L3 or L1 or both) conditions. It is beneficial to provide information to the RAN to detect the conditions that are triggered in both successful and failure cases.

Proposal 2: MRO for CLTM should support enhancements to optimize LTM execution (both L3 and L1) conditions configured for initial LTM and subsequent LTM.

Observation 2: gNB configures ltm-Candidate-TimeAlignmentTimer (referred to as CLTM TA Timer for simplicity) via RRC to the UE, which controls how long the UE considers the CLTM candidate cells associated with this timer to be uplink time aligned

Observation 3: When a CLTM cell switch procedure is triggered for a CLTM candidate cell, the UE 1)applies the stored TA value associated with the CLTM candidate cell for the pTAG, 2) start or restart the timeAlignmentTimer associated with the pTAG with the length of the remaining time till expiry for the ltm-Candidate-TimeAlignmentTimer.

Proposal 3: MRO for CLTM should handle how to optimize CLTM TA timer

Proposal 4: MRO for RACH-less CLTM failure should support detection of CLTM TA timer expiry before CLTM conditions are satisfied (RACH-less CLTM was not initiated at all).

Proposal 5: MRO for RACH-less CLTM failure should support detection of CLTM TA timer expires when RACH-less CLTM is ongoing (T304 still running).

Proposal 6: MRO for RACH-less CLTM failure should support detection of T304 expiry when RACH-less CLTM is ongoing (RACH-less CLTM execution failure).

Proposal 7: MRO for RACH-less CLTM failure should support detection of Successful RACH-less CLTM execution, but very close to “CLTM TA timer expiry”, thereby TA timer applied for pTAG is too small.

Proposal 8: MRO for RACH-less CLTM failure should support detection of RACH-based CLTM execution because early UL sync was not performed towards the candidate cell.