3GPP TSG-RAN3 Meeting #129bis	R3-256612

Prague, Czech Republic, Oct 13 - 17, 2025

Agenda Item:	12.2.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on Inter-CU LTM

Document for:	Discussion/Decision

# Introduction

TSG RAN#109 approved SID on NG-RAN AI/ML use cases in RP-252867 [1]. The SID contains the following objectives for AI/ML use case study.

RP-252867 The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

In this paper, we discuss our views on AI/ML based Inter-CU LTM optimization.

# Discussion

In the Handover enhancements sub agenda for AI/ML based mobility optimization, AI/ML based Inter-CU LTM is the main topic to be discussed. In addition to Intra-CU LTM, AI/ML based Inter-CU LTM optimization is essential to support for any useful commercial deployements in operator networks. Additionally, the procedure for Intra-CU LTM and Inter-CU LTM is almost the same. Intra-CU LTM has signaling procedures over F1 interface whereas Inter-CU LTM has the same signaling procedures over Xn. Normative specification of both Intra-CU LTM and inter-CU LTM is already completed in Release 18 and 19 respectively.

- Intra-CU LTM and Inter-CU LTM have similar procedures for LTM Handover preparation and execution over F1 and Xn interfaces respectively.

- Useful commercial deployment of AI/ML optimization of LTM feature will require both AI/ML based Inter-CU LTM and Intra-CU LTM.

The AI/ML optimizations studied for Intra-CU LTM over F1 interface can easily be applied to Inter-CU LTM over Xn Interface, as the LTM call flow procedures are identical between both the flavours. This will reduce additional effort to study a new feature for AI/ML based optimization and also will help in deeper analysis of overall LTM optimization.

- Most of the AI/ML based optimization discussed and studied for Intra-CU LTM over F1 interface will be applicable for Inter-CU LTM over Xn interface and only delta changes needed to study AI/ML based Inter-CU LTM.

Hence we propose RAN3 to study AI/ML based Inter-CU LTM in Handover enhancements and enhancements studied for AI/ML based Intra-CU LTM can be extended to Inter-CU LTM as well.

- RAN3 to study AI/ML based optimization for Inter-CU LTM in Handover Enhancements, as most of the AI/ML optimizations studied for Intra-CU LTM over F1 interface can be extended to Inter-CU LTM over Xn Interface.

Similar to Intra CU LTM, Inter-CU LTM can also be based on L1 or L3 Measurement Report. Hence the study of AI/ML based Inter-CU LTM should consider both L1 and L3 MR based optimization. Conditional Inter-CU LTM should also be considered for the study.

- Study on AI/ML based Inter-CU LTM should consider –

  - L1 and L3 Measurement Report based Inter-CU LTM

  - Conditional Inter-CU LTM

Similar to Release 19 AI/ML use cases, the Release 20 Inter CU LTM should also consider both split and non-split architecture for study. Based on the use case and the feature under discussion, the AI/ML inference should be flexibly located in gNB-CU or gNB-DU.

- Both split and non-split architecture should be considered for AI/ML based Inter CU LTM.

- Based on the AI/ML based Inter CU LTM use case features, inference should be flexibly located in gNB-CU or gNB-DU.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Intra-CU LTM and Inter-CU LTM have similar procedures for LTM Handover preparation and execution over F1 and Xn interfaces respectively.

Observation 2.	Useful commercial deployment of AI/ML optimization of LTM feature will require both AI/ML based Inter-CU LTM and Intra-CU LTM.

Observation 3.	Most of the AI/ML based optimization discussed and studied for Intra-CU LTM over F1 interface will be applicable for Inter-CU LTM over Xn interface and only delta changes needed to study AI/ML based Inter-CU LTM.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to study AI/ML based optimization for Inter-CU LTM in Handover Enhancements, as most of the AI/ML optimizations studied for Intra-CU LTM over F1 interface can be extended to Inter-CU LTM over Xn Interface.

Proposal 2:	Study on AI/ML based Inter-CU LTM should consider –

a.	L1 and L3 Measurement Report based Inter-CU LTM

b.	Conditional Inter-CU LTM

Proposal 3:	Both split and non-split architecture should be considered for AI/ML based Inter CU LTM.

Proposal 4:	Based on the AI/ML based Inter CU LTM use case features, inference should be flexibly located in gNB-CU or gNB-DU.

4 References

[1] RP-252867, Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3