3GPP TSG-RAN3 Meeting #129bis	R3-256613

Prague, Czech Republic, Oct 13 - 17, 2025

Agenda Item:	10.5.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on 6G AI/ML Use Cases

Document for:	Discussion/Decision

# Introduction

TSG RAN#109 approved on updated SI on 6G Radio in RP-252912 [1]. The SID contains the following objectives for AI/ML study in RAN3 scope.

RP-252912 - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services

In this paper, we discuss our views on AI/ML use cases and general principles for 6G.

# Discussion

## General Principles for AI/ML Use Cases

Native AI/ML support in 6G RAN features is  one of the key objectives of 6G. AI/ML use cases started in the later release of 5G, and not many use cases were addressed in 5G for AI based optimization. 6G Native AI/ML should enable optimization of the Network and UE performance also considering optimized resource consumption and signalling.  6G AI/ML use cases should maintain balance between performance gains vs added complexity and with the rapid advancements of AI/ML algorithms/models, 6G RAN is expected to see considerable improved performance from day 1.

6G RAN AI/ML use cases should aim for optimization of Network and UE performance, reduction of resources consumption and/or reduction of signaling.

5G AI/ML functional framework consisted of Data collection, AI/ML Training, AI/ML Inference and AI/ML feedback. For any AI/ML use case input data is the key for Model Training. Varied data collected over an extended area and over an extended period of time will enable training AI model for different scenarios. Initial AI/ML model training can be non real-time and does not need data collection in real-time. But offline training models should be allowed to be fine tuned in real time to improve model performance by using real-time data as well.

AI/ML Training  can apply to both real-time and non-real-time use cases hence training data may need to be collected both in real-time (streaming similar to SON) and non-real-time (batch similar to MDT) procedures.

5G AI/ML framework considered AI/ML Training located in RAN or OAM for 5G AI/ML use cases. Initial AI/ML Training is not real-time and needs varied inputs from different sources. It is beneficial for Model Training to happen in a centralized location for offline model training which is trusted and can enforce user privacy protection of the collected data, rather than individual RAN nodes.

In 5G, NG-RAN AI/ML framework, AI/ML training is located in RAN or OAM. It is beneficial for Model Training to happen in centralized location rather than in individual RAN nodes.

For 6G AI/ML use cases, initial model training should be located in a centralized Node.

Both initial Model training and fine tuning should occur in a location that is trusted and can enforce user privacy protection.

For 5G AI/ML use cases, location of AI/ML inference was restricted to NG-RAN for non-split architecture and gNB-CU for split architecture. Though certain use cases like AI/ML based CCO necessitated the AI/ML inference to be located in the gNB-DU for non-split architecture, it was not agreed citing AI/ML functional framework doesn’t support this. For RAN1/2 use cases like BM and CSI compression, it is logical run inference in 5G DU but this was not discussed and not supported, thus causing ineffective usage of AI/ML models on network side. However, in 6G, the AI/ML functional framework should be flexible to accommodate different use cases and allow location on inference in any RAN NF based on the use case. Similar to 5G, the AI/ML models and algorithm should be left to implementation and should be out of the 3GPP  scope.

In 5G, the location of AI/ML inference was restricted to only gNB-CU in a split architecture, irrespective of the use cases supported.

6G RAN AI/ML use cases should support flexible deployment of AI/ML inference in any RAN network function depending on the AI/ML use case.

For 6G AI/ML use cases, AI/ML Models and algorithms should be out of scope for 3GPP.

For 5G AI/ML use cases, input data for training and feedback was restricted among RAN nodes only. The inference output was only exchanged between the RAN nodes. RAN3 AI/ML started earlier than RAN1 and RAN2 AI/ML use cases. Hence collaboration of input and output for AI/ML use cases across RAN1/2/3 was not possible in 5G. For AI/ML use cases like Mobility, it is beneficial to get input data from UE and also provide inference to UE.

In 5G, input data for AI/ML use cases and the inference output was restricted to be supported only between the RAN nodes.

6G should support input data for RAN AI/ML use cases from all sources including UE, RAN, CN and 3rd party sources.

## AI/ML Use Cases

5G NG-RAN AI/ML WI studied and specified the below use cases -

- AI/ML based Load Balancing

- AI/ML based Mobility

- AI/ML based Energy Savings

- AI/ML based Slicing

- AI/ML based CCO

These use cases are applicable across G’s and can be considered for 6G AI/ML use cases and can be evaluated further.

5G NG-RAN AI/ML use cases addressed Load Balancing, Mobility, Energy Savings, Slicing and CCO.

The 5G NG-RAN AI/ML use cases should be evaluated as the baseline for 6G RAN AI/ML use case study.

AI/ML based Energy Savings was studied in 5G, but due to lack of consensus, only exchange of Energy cost among the RAN nodes was specified without establishing clear benefits. AI/ML based Network Energy Savings was not addressed in 5G. For 6G, it is necessary to study features which enable Network Energy Savings like NES based mobility, NES based SIB broadcast etc.

5G NG-RAN AI/ML Energy Savings use case only addressed exchange of Energy Cost among RAN nodes without any AI/ML based optimization and without any performance benefits.

5G Network Energy Savings features should be evaluated as the baseline for 6G AI/ML use cases for Energy Savings (pending RAN1/2 6G network energy saving discussions for day1 deployments).

5G Air Interface AI/ML use cases were studied and specified in RAN1/2 working groups without RAN3 involvement due to lack of time. Hence Network impacts of the AI/ML Air Interface use cases were not addressed in 5G due to time constraints. In 6G AI/ML use cases, we should look at end to end optimization between UE and Network and not just RAN nodes. 6G should enable study of Network side impact for 6G Air Interface use cases.

5G Air Interface AI/ML use cases addressed Mobility, Beam Management, CSI-RS optimization and Positioning.

RAN3 should study the network side impacts of 6G use cases that will be agreed by RAN1 and RAN2.

Since AI/ML features started in the later release of 5G, not many use cases were studied for AI/ML based optimization due to time constraints in each release. Few use cases as mentioned above were studied in 5G. In 6G, considering AI/ML as one of the key objectives, all applicable 6G RAN features should be AI/ML optimized to enable proactive end to end system performance optimization..

In 5G not all RAN features supported AI/ML based optimization. Only few use cases were selected for AI/ML based optimization.

AI/ML based optimization for all applicable 6G RAN features and use cases should be studied.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	AI/ML Training  can apply to both real-time and non-real-time use cases hence training data may need to be collected both in real-time (streaming similar to SON) and non-real-time (batch similar to MDT) procedures.

Observation 2.	In 5G, NG-RAN AI/ML framework, AI/ML training is located in RAN or OAM. It is beneficial for Model Training to happen in centralized location rather than in individual RAN nodes.

Observation 3.	In 5G, the location of AI/ML inference was restricted to only gNB-CU in a split architecture, irrespective of the use cases supported.

Observation 4.	In 5G, input data for AI/ML use cases and the inference output was restricted to be supported only between the RAN nodes.

Observation 5.	5G NG-RAN AI/ML use cases addressed Load Balancing, Mobility, Energy Savings, Slicing and CCO.

Observation 6.	5G NG-RAN AI/ML Energy Savings use case only addressed exchange of Energy Cost among RAN nodes without any AI/ML based optimization and without any performance benefits.

Observation 7.	5G Air Interface AI/ML use cases addressed Mobility, Beam Management, CSI-RS optimization and Positioning.

Observation 8.	In 5G not all RAN features supported AI/ML based optimization. Only few use cases were selected for AI/ML based optimization.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	6G RAN AI/ML use cases should aim for optimization of Network and UE performance, reduction of resources consumption and/or reduction of signaling.

Proposal 2:	For 6G AI/ML use cases, initial model training should be located in a centralized Node.

Proposal 3:	Both initial Model training and fine tuning should occur in a location that is trusted and can enforce user privacy protection.

Proposal 4:	6G RAN AI/ML use cases should support flexible deployment of AI/ML inference in any RAN network function depending on the AI/ML use case.

Proposal 5:	For 6G AI/ML use cases, AI/ML Models and algorithms should be out of scope for 3GPP.

Proposal 6:	6G should support input data for RAN AI/ML use cases from all sources including UE, RAN, CN and 3rd party sources.

Proposal 7:	The 5G NG-RAN AI/ML use cases should be evaluated as the baseline for 6G RAN AI/ML use case study.

Proposal 8:	5G Network Energy Savings features should be evaluated as the baseline for 6G AI/ML use cases for Energy Savings (pending RAN1/2 6G network energy saving discussions for day1 deployments).

Proposal 9:	RAN3 should study the network side impacts of 6G use cases that will be agreed by RAN1 and RAN2.

Proposal 10:	AI/ML based optimization for all applicable 6G RAN features and use cases should be studied.

4 References

[1] RP-252912, Revised SID: Study on 6G Radio