3GPP TSG-RAN WG3 Meeting #129-bis	R3-256615

Prague, CZ, 13th – 17th October, 2025

Agenda Item:	10.2.1

Source:	CATT

Title:	Discussion on general principles and requirements for 6G RAN

Document for:	Decision

1	Introduction

In the 6G WID [1], RAN3 is responsible for the Radio Access Network Architecture, interface protocols and procedures, including the Overall RAN architecture aspects, as highlighted below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we initially analyze key requirements and basic principles for 6G RAN from RAN3 perspective, and subsequently, based on these requirements, we further provide specific proposals for 6G RAN design targets.

2	Discussion

2.1	Overall Principle and Requirement

Requirement captured in TR38.914 on architecture

6G requirements for architecture and migration has been discussed in the RAN level study item, and corresponding agreements have been captured in the TR 38.914 [2], highlighted below:

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

Since it has been discussed and agreed in RAN plenary meeting, RAN3 should confirm the requirements above, and then, use it to guide the further study in RAN3.

Proposal 1: RAN3 confirms the RAN architecture requirements in TR 38.914 and capture them in our own TR to guide further study.

Extending 5G architecture requirement to 6G

In addition to the requirements agreed in RAN#109 meeting, we believe some other requirements for 5G architecture should also extend to 6G.

Deployment flexibility:

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable low latency services, etc.

Just as in 5G, the need to support diverse services such as low-latency communications means that deployment flexibility will continue to be an important requirement for 6G.

RAN internal split

-	The RAN architecture shall allow for CU-DU splitting.

This is a controversial issue with a checkpoint scheduled for June 2026 to determine whether CU/DU split support will be retained. From our point of view, CU/DU split should continue to be supported in 6G as the benefits of introducing split architecture discussed in 5G is still valid.

Support of Virtualization deployment

-	The RAN architecture shall allow deployments using Network Function Virtualization.

We think there is no doubt that the RAN architecture should allow various deployments which including using Network Function Virtualization.

Independ evolvement of RAN and CN

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

This point was discussed during the RAN plenary meeting but was ultimately omitted from the final version. We believe the intention was to ensure the independent evolution of the RAN and Core Network, allowing them to interoperate seamlessly across different release in the same generations. This is a fundamental architectural requirement and should be included.

Proposal 2: Capture the abovementioned architecture requirements in 5G into 6G RAN3 TR

Services supported in 6G

In 5G,on support of service, there are following requirements：

-	The design of the RAN architecture shall allow the deployment of new services rapidly and efficiently.

-	The design of the RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

Similarly,there would also be requirement on architecture for services to be supported in 6G,which is a key topic of discussion at the last RAN plenary meeting. TR 38.914 captures several candidate services, which include not only traditional ones like Mobile Broadband, Immersive Communication, Massive IoT, and Voice, but also new offerings such as Sensing, AI, and regulatory services.

For the traditional services, we have not identified any new architectural requirements specific to 6G till now. Therefore, supporting these existing services can be included as a baseline requirement for subsequent consideration.

-	The design of the RAN architecture shall allow the support of existing services (e.g. Mobile Broadband, Immersive Communication, massive IoT, Vioce).

However, the new emerging services, such as AI/ML and ISAC, exhibit characteristics distinct from traditional ones. At least two key differences have been identified:

- The AMF may not need to be involved in control-related procedures.

- These services will require massive data exchange across all network entities,e.g between UE and RAN,RAN and CN,two NG-RAN nodes.

To facilitate the deployment of these two new services and  accommodating future emerging ones, the following point should be considered as a requirement for the RAN architecture.

- The design of the RAN architecture shall support  the deployment of the new services (e.g. AI/ML, sensing),rapidly and efficiently.

-    Support efficient signalling exchange: enable direct signalling exchange between network entities while guarantee elastic scalability as well.

-	Support efficient data transfer: Enable direct data transfer for new services (e.g. AI/ML, sensing) across all network entities (UE, RAN node, CN).

Proposal 3：Capture the aforementioned architecture requirements on the services supported in 6G into the RAN3 TR.

2.2 Considerations on 6G RAN Design Targets

To meet the extensive requirements mentioned above, the design of the 6G RAN architecture shall minimize the constraints from each requirement as much as possible. This can be achieved by reducing the inter-dependency between network functions. Therefore, during the initial phase of architecture design, priority should be given to evaluating approaches to separating these functions in order to meet specific design targets.

Proposal 4：During the initial phase of architecture design, it should be prioritized to consider separating network function to meet specific design targets.

Given that legacy services imposes different loads on the user plane function and control plane, especially  in eMBB scenarios with high data rate requirement, independently deployment of control plane and user plane function has benefits. Therefore, even in 6G, it is necessary to study the functional architecture of 6G RAN from the perspective of separating user plane and control plane functions.

Proposal 4.1：The design of 6G RAN architecture should support separating user planes and control plane functions.

The 6G RAN needs to support flexible deployment and elastic scaling of network functions. Specifically, the architecture design should support both centralized and distributed network deployment modes. For example, for non-delay-sensitive services, it is beneficial for optimizing the allocation effect of system resources to separate the deployment of real-time functions and near-real-time functions. Therefore, for the flexible deployment of various 6G services, it is necessary to study 6G RAN architecture from the perspective of separating real-time functions and near-real-time functions.

Proposal 4.2：To support the flexible deployment of various service types, RAN3 should be conducted from the perspective of separating real-time functions and near-real-time functions.

As mentioned in section 2.1, the new service types, e.g. AI/ML, sensing, there are significant differences from the characteristics of legacy services in terms of transport path, data processing logic, distributing way and data type, etc. Therefore, from the perspective of architecture design, it is necessary to consider the simplicity, elasticity, scalability of the system architecture to adapt various communication modes.

Proposal 4.3: To support new service scenarios (e.g., AI, Sensing, etc.), separating legacy service functions and new service functions should be considered.

NTN is an important component of the 6G network to support ubiquitous connectivity and services to the UEs as a good supplement of the terrestrial network. And it’s agreed that the 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. Currently, it’s agreed that RAN architecture should support both terrestrial network and non-terrestrial network. From RAN3 point of view, we expect to have a unified architecture for both TN and NTN.

We understand that NR NTN designing could be taken as the baseline,to make the NTN deployment more flexible and relible, we can further consider the following bullets in our study:

- support of transparent payload and regenerative payload with full/partial base station on board;

In 5G, they’ve been studied in NR Rel-16, see TR38.821, the transparent payload is supported in normative phase in Rel-17 and Rel-18, the Regenerative Payload with full gNB on-board is supported since Rel-19.

- Identify any necessary enhancement is needed for the interface management, mobility(intra NTN, TN/NTN, multi-orbits coordination) and Paging.

- Other new capabilities/services to be enabled in NTN is pending to the other WGs, e.g. support of TDD, support of Positioning, etc. No immediate RAN3 impact is foreseen.

Proposal 5: A unified RAN architecture is expected for TN and NTN, and the NR NTN design could be taken as baseline for 6G NTN.

Proposal 6: We should study if any  necessary enhancement to be persuded for 6G NTN, e.g. support of more deployment options, support of multi-orbits coordination, potential mobility enhancement, etc.

3	Conclusion

Base on the discussion above, we provide the following proposals:

Proposal 1: RAN3 confirms the RAN architecture requirements in TR 38.914 and capture them in our own TR to guide further study.

Proposal 2: Capture the following architecture requirements in 5G into 6G RAN3 TR.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable low latency services, etc.

-	The RAN architecture shall allow for CU-DU splitting.

-	The RAN architecture shall allow deployments using Network Function Virtualization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

Proposal 3: Capture the architecture requirements on the services supported in 6G into the RAN3 TR.

-	The design of the RAN architecture shall allow the support of existing services (e.g. Mobile Broadband, Immersive Communication, massive IoT, Vioce).

-	The design of the RAN architecture shall support the deployment of the new services (e.g. AI/ML, sensing) rapidly and efficiently.

-    Support efficient signalling exchange: Enable direct signalling exchange between network entities while guarantee elastic scalability as well

-	Support Efficient Data Transfer: Enable direct data transfer for new services (e.g., AI/ML, sensing) across all network entities (UE, RAN node, CN)

Proposal 4: During the initial phase of architecture design, it should be prioritized to consider separating network function to meet specific design targets.

- Proposal 4.1: The design of 6G RAN architecture should support separating user planes and control plane functions.

- Proposal 4.2: To support the flexible deployment of various service types, RAN3 should be conducted from the perspective of separating real-time functions and near-real-time functions.

- Proposal 4.3: To support new service scenarios (e.g., AI, Sensing, etc.), separating legacy service functions and new service functions should be considered.

Proposal 5: A unified RAN architecture is expected for TN and NTN, and the NR NTN design could be taken as baseline for 6G NTN.

Proposal 6: We should study if any  necessary enhancement to be persuded for 6G NTN, e.g. support of more deployment options, support of multi-orbits coordination, potential mobility enhancement, etc.

4	Reference

- RP-251881 New SID: Study on 6G Radio

- TR 38.914 Study on 6G Scenarios and Requirements  v0.1.0