3GPP TSG-RAN WG3 Meeting #129-bis	R3-256616

Prague, CZ, 13th – 17th October, 2025

Agenda Item:	10.2.2

Source:	CATT

Title:	Discussion on 6G RAN Logical Architecture

Document for:	Decision

1	Introduction

In the 6G WID [1], RAN3 is responsible for the Radio Access Network Architecture, interface protocols and procedures, including the Overall RAN architecture aspects, as highlighted below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we initially analyze a potential 6G RAN Overall Architecture direction from RAN3 perspective, and subsequently, based on this direction, we further provide some specific proposals for architecture and interface design.

2  Discussion

2.1  Overall Architecture

The 6G RAN consists of the following logical nodes:

-	6G RAN nodes providing the 6G U-plane, C-plane and D-plane protocol terminations towards the UE;

The logical nodes in 6G RAN are interconnected with each other by means of the Xn+ interface.

For legacy service, the logical nodes in 6G RAN are connected to the 6GC NFs by means of the NG+ interface. The NG+interface supports a many-to-many relation between 6GC NFs and the logical nodes in 6G RAN.

For new service (e.g, AI, Sensing, etc), the logical nodes in 6G RAN within the Control Plane enables the 6GC NFs to access their services by means of a service-based interface N6gr.

For new service (e.g, AI, Sensing, etc), the logical nodes in 6G RAN are directly connected to the corresponding NFs in 6GC by means of the Ndp interface of D-plane.

The 6G RAN architecture is illustrated in following figure.

Figure 1: 6G RAN overall architecture

Proposal 1:  Capture the aforementioned architectural diagrams and relevant descriptions in the RAN3 TR.

2.2  Considerations of Architecture and Interface

One of the key principles in the design of 6G RAN is elastic scalability, which means the system architecture should not only meet current service requirements but also adapt to the expansion of new services in the future. From 5G to 6G, typical service scenarios have significant changes: new types of services such as AI, sensing, and computing management have been introduced. However, if RAN3 continues to adopt a design concept similar to that of the 5G architecture, i.e, introducing a new service feature, e.g, positioning, AI, by a "patching" method, it will bring many long-term adverse impacts on the entire architecture.

Firstly, adopting the 5G architecture design concept will increase the complexity of direct communication between different network functions.For example,when designing the NRPPa protocol for positioning in 5G, the idea of relaying by AMF increases the information transmission delayand create strong binding relationships between different network functions. In contrast, if the issue of elastic scalability is considered at the initial stage of system design, direct communication can be established between network functions, which will undoubtedly simplify the complexity and reduce the redundancy of the entire RAN design.

Secondly, during the evolution of the 5G architecture, there have been many technical discussions on whether certain functional entities can be deployed on the RAN side or in edge network. However, if  elastic scalability is taken into account at the initial stage of system design, the specific function deployment will be purely practical deployment issues rather than architectural requirements, which further enhances the flexibility and adaptability of network functions.

Thirdly, the initial design of the 6G network only needs to address well-known service requirements in the industry, like AI and Sensing. However, with the rapid development of various applications, more new service scenarios will emerge, these new requirements may subsequently necessitate retroactive modifications to the original design, which undoubtedly brings uncertainty to the future evolution of the architecture.

Based on the above discussion for elastic scalability, we provide the following architecture proposals for 6G RAN.

Proposal 2:  Direct communication should be supported between 6G RAN nodes and newly defined network functions, so as to avoid complex and redundant transport paths.

Proposal 3: A service-oriented communication architecture should be designed between 6G RAN nodes and newly defined network functions to decouple network functions from transport layer, and making it easier to adapt to various flexible deployments.

Proposal 4: 6G RAN nodes and 6GC network functions should form an unified service-oriented network architecture to quickly adapting to various new service scenarios in the future.

For the 6G RAN-CN interface, in aspect of new services, we propose the definition of new interface type, and the new interface type should be cloud native:

- The control plane interface should support HTTP service-based protocols. Since SBA is built on protocols (HTTP) and interactions (API-based) commonly used in cloud deployment, and adapts to the needs of operator, it offers extensive application scenarios. Moreover, it has been well applied in 5GC, which can ensure that the expansion for new services will not affect the original network architecture

- The application protocol of the control plane interface should be decoupled from the transport layer, unlike the legacy N2 interface which has association with SCTP protocol. By using an SBA-based interface, resource-based granularity can be achieved, where service operations and created resources can be addressed via URIs

- The data transmission mechanism between the 6G RAN and the 6GC for the new services should be designed efficiently, taking into account the specific characteristics of the service e.g. the possibility of termination in RAN and CN. A direct data connection should be established between the RAN and the corresponding CN function, rather than routing traffic through the UPF.

Proposal 5: For the 6G RAN-CN interface, in aspect of new services, the new interface type should be cloud native.

Proposal 6: The control plane interface for new services should support HTTP service-based protocols, and be decoupled from the transport layer.

Proposal 7：Establish direct data connection between 6G RAN nodes and corresponding functions in 6GC.

For the new type interface, the applicable functions in 6G include Sensing, AI, Positioning, RAN capability exposure, etc. The main considerations are as follows:

- The information transmission of AI and sensing functions between 6G RAN and 6GC can be realized through direct communication via the new type interface. This avoids relaying, function binding, and enables flexible deployment of network functions;

- The positioning service in 5G requires support the relaying of NRPPa signalling via AMF. However, this design concept can better leverage the new type interfaces of 6G, which inherently possess the characteristics such as direct communication, elasticity, and service-oriented;

- RAN capability exposure allows real-time interaction of information such as RAN performance data, scheduling strategies, and resource status with external third parties or the management functions in the CN or OAM domain. By the use of the new type interfaces, direct and convenient information transmission, as well as unified design with CN functions, can be achieved.

Proposal 8: For the new type interface, the applicable functions in 6G include Sensing, AI, Positioning, RAN capability exposure, etc.

For the 6G RAN-CN interface, in aspect of legacy services, we propose supporting a P2P interface similar to that in 5G. The main considerations for this design are as follows:

- In the design of previous generations of network architectures, the splitting of network functions between RAN and CN has been relatively clear for legacy services. Therefore, there is no need for 6G to invest more standardization efforts in this aspect

- For legacy services, backward compatibility of network evolution can be ensured. Since it allows operators to continue using the transport network infrastructure they have invested in earlier, it can further reducing the overall investment cost of 6G

Proposal 9: For the 6G RAN-CN interface, in aspect of legacy services, to support a P2P interface similar to that in 5G.

For the user plane interfaces, the GTP-U protocol shall continue to be used for the user plane interfaces between 6G RAN and 6G UPF, as well as between 6G RAN nodes. Given its demonstrated efficiency and reliability, continuing with GTP-U ensures a performant and stable foundation for data transport in the new generation.

Proposal 10：For the user plane interfaces, the design of 5G shall continue to be adopted for the interfaces between 6G RAN and 6G UPF, as well as between 6G RAN nodes.

3	Conclusion

Base on the discussion above, we provide the following proposals:

Proposal 1:  Capture the aforementioned architectural diagrams and relevant descriptions in the RAN3 TR.

Proposal 2:  Direct communication should be supported between 6G RAN nodes and newly defined network functions, so as to avoid complex and redundant transport paths.

Proposal 3: A service-oriented communication architecture should be designed between 6G RAN nodes and newly defined network functions to decouple network functions from transport layer, and making it easier to adapt to various flexible deployments.

Proposal 4: 6G RAN nodes and 6GC network functions should form an unified service-oriented network architecture to quickly adapting to various new service scenarios in the future.

Proposal 5: For the 6G RAN-CN interface, in aspect of new services, the new interface type should be cloud native.

Proposal 6: The control plane interface for new services should support HTTP service-based protocols, and be decoupled from the transport layer.

Proposal 7：Establish direct data connection between 6G RAN nodes and corresponding functions in 6GC.

Proposal 8: For the new type interface, the applicable functions in 6G include Sensing, AI, Positioning, RAN capability exposure, etc.

Proposal 9: For the 6G RAN-CN interface, in aspect of legacy services, to support a P2P interface similar to that in 5G.

Proposal 10：For the user plane interfaces, the design of 5G shall continue to be adopted for the interfaces between 6G RAN and 6G UPF, as well as between 6G RAN nodes.

4	Reference

- RP-251881 New SID: Study on 6G Radio

- R3-256615  Discussion on general principles and requirements for 6G RAN,  CATT