3GPP RAN WG3 Meeting #129-bis	R3-256619

Prague, Czechia, 13th – 17th October 2025

Source:	CATT

Title:	Consideration on AI/ML use cases

Agenda Item:	10.5.1

Document for:	Discussion and decision

- Introduction

This document shows our understanding on AI/ML use cases.

- Discussion

  - AI for network: RAN3 use cases

RAN3 has been enhancing NG-RAN to support many RAN3 uses cases. It is reasonable to consider them as well in 6G.

One of the uses cases is UE “mobility”—during the discussion its real meaning seems to be using UE-associated predictions, especially trajectory prediction, to optimise UE-associated configuration including handover decision. RAN2 is now considering a similar use case. Although RAN2 mainly focus on UE-sided model, network-sided model is also within their scope. For example, RAN2 may also study data collection from UE to RAN in order to train and use network-sided models. We support this use case in 6G, but some coordination or mutually alignment with RAN2 may be beneficial.

Proposal 1: Support the use case for UE mobility, or more precisely, using UE-associated predictions (e.g. UE trajectory prediction) to optimise UE-associated configuration. Coordination or mutually alignment with RAN2 may be beneficial, e.g. for a unified solution on network-sided AI/ML model predicting handovers and its inference inputs from the UE.

The remaining use cases are all for non-UE-associated adjustment. There are two typical logics for these use cases:

- Predicting changes over time (e.g. load change, coverage or capacity problem), and using such prediction to adjust RAN beforehand.

- Predicting the impact on KPI / performance of a candidate adjustment (e.g. change on mobility threshold, energy saving action, coverage status change), and using such prediction to decide whether to adopt the adjustment or which adjustment to adopt.

We think both logics should be considered and evaluated.

Proposal 2: Support non-UE related RAN adjustments, e.g. load balancing, energy saving.

Proposal 3: For non-UE related RAN adjustments, support both predicting change over time (e.g. load change over time) and predicting change on performance as result of a candidate action (e.g. energy saving action).

And for the latter type of logic, change on non-UE configuration may cause some inter-node handovers. Considering that inference inputs includes much UE-associated information, and such UE-associated information are far easier to get at the source side, we recommend considering only deploying AI/ML models at the source side if deployed in RAN.

Figure 1: Work flow that source predicts the result of non-UE-associated adjustment

Proposal 4: For predicting change on performance upon action and if AI/ML models are deployed in RAN, considering only deploying AI/ML models at the source side of handovers caused by the candidate action subject of prediction.

  - AI for network: support of use cases in other WGs

Although discussions on use cases in other working groups are still ongoing, we believe it is inevitable for RAN3 to participate in the related work.

Taking the discussion of AI-PHY in 5G as an example: for model transfer/delivery, RAN1 has explored the sharing of datasets or model parameters between the network and the UE or UE-side training entities in the context of two-sided models. RAN1 has proposed two alternatives for this sharing process—OTA and non-OTA solutions as below:

Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) Alternative 2 (OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server) NOTE: The data transfer (up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

Regarding the non-OTA solution, if the gNB acts as the network-side data collection entity, one option discussed in RAN2 involves transferring data from the gNB to the OAM/CN, and then to the UE-side training entity. RAN2 sent a LS to RAN3 to assess the solution's feasibility [1]. The feedback from RAN3 indicated that “The exchange of AI/ML-related data between the CN and gNB is not supported in R19, and no evaluation on the feasibility in Rel-19 has been done” [2].

This situation is largely due to the mid-cycle introduction of AI/ML functions into the 5G evolution, necessitating that developments conform to the existing architecture and interfaces. It is therefore a challenge to facilitate the required AI/ML data delivery between the CN and the gNB. Conversely, for 6G, foundational support for AI/ML is a primary design objective from Day 1. It is consequently imperative that the architectural and interface considerations for AI/ML data are incorporated into the initial design.

Furthermore, a LS received from SA2[3] concerning data-related work indicated the necessity to support data related work as below:

TSG SA asks SA2 and SA5 to progress and collaborate (including with RAN2 and RAN3 where applicable) on 6G data related work tasks and taking the 5G working scope as the starting point.

SA2 and SA5 should coordinate with SA3 for the data security and privacy requirements.

Regular coordination and collaboration among the WGs are expected, including checks on the progress of each group for 6G data related work tasks in March 2026 (TSG SA#111) and/or June 2026 (TSG#112), to avoid the duplicate design for the same functionality and to develop system wide consistent solutions.

Based on the requirement idenfied in 5G and also the discussion in SA on 6G, we propose that RAN3 discuss support of AI related data transfer between RAN and CN from Day 1

Proposal 5: Discuss AI relatd data transfer from Day 1.

To address the requirements for AI/ML data transfer between RAN and CN in 5G, at least the following key characteristics have been identified:

- Data transmission is terminated in RAN and CN.

- The data volume is extreamly high.

Given these characteristics, the existing NG interface's control plane and user plane are unsuitable for this task. The control plane is not designed for large data volumes, while the user plane is terminated at the CN and UE, not facilitating direct RAN-CN transfer.

Therefore, we propose exploring a new data transfer mechanism between RAN and CN, distinct from the current control and user plane solutions.

Proposal 6: In light of the specific characteristics of AI/ML data, we recommend developing a new data transfer mechanism between RAN and CN.

Additionally, RAN3 has received another LS from RAN2 [4] regarding network-side data collection, which includes information for RAN3's consideration.

- The list of logging configurations is provided as a new list configurations within CSI-MeasConfig.

- the data logging procedure based on receiving measurement quantities from lower layer  is captured in RRC spec.

- The L3 event (i.e. Event A1 or Event A2) configuration is contained in the logging configuration to support the event based data logging.

- The logged data is reported via UEInformationRequest/UEInformationResponse.

- The UE can send a UAI that indicates:

- Data is available

- Reason for trigger (full buffer, threshold)

- Low power indication

- The NW side data collection configuration should be released if UE reports the lower power indication to the NW via UAI.

For the use case mentioned above, if we consider the AI training/inference entity is located in source gNB, it create a requirement to deliver large amounts of data from a target node to the source. Furthermore, within the RAN3-led work item on AI/ML for NG-RAN, we have identified use cases that necessitate the exchange of AI-related data between NG-RAN nodes, which are supported via control plane based solution.

While a control-plane-based solution was specified in 5G, considering the variety of use cases in other WGs in 5G and also the emerging scenarios in 6G, a new, dedicated mechanism for exchanging AI data would be more future-proof for 6G.

Proposal 7: Similar to the case for the RAN-CN interface, introduce a new data transfer mechanism between RAN nodes.

It would be beneficial to establish a unified framework for data transfer, applicable to both the CN-RAN interface and the interface between neighboring RAN nodes, in order to simplify the overall design.

Proposal 8: Have a unified design for data transfer between CN and RAN node and between RAN nodes.

- Conclusion

Proposal 1: Support the use case for UE mobility, or more precisely, using UE-associated predictions (e.g. UE trajectory prediction) to optimise UE-associated configuration. Coordination or mutually alignment with RAN2 may be beneficial, e.g. for a unified solution on network-sided AI/ML model predicting handovers and its inference inputs from the UE.

Proposal 2: Support non-UE related RAN adjustments, e.g. load balancing, energy saving.

Proposal 3: For non-UE related RAN adjustments, support both predicting change over time (e.g. load change over time) and predicting change on performance as result of a candidate action (e.g. energy saving action).

Proposal 4: For predicting change on performance upon action and if AI/ML models are deployed in RAN, considering only deploying AI/ML models at the source side of handovers caused by the candidate action subject of prediction.

Proposal 5: Discuss AI relatd data transfer from Day 1.

Proposal 6: In light of the specific characteristics of AI/ML data, we recommend developing a new data transfer mechanism between RAN and CN.

Proposal 7: Similar to the case for the RAN-CN interface, introduce a new data transfer mechanism between RAN nodes.

Proposal 8: Have a unified design for data transfer between CN and RAN node and between RAN nodes.

- Reference

[1] R3-253010 Reply LS on signalling feasibility of dataset and parameter sharing Xiaomi Ericsson

[2] R3-253961 Reply LS on signalling feasibility of dataset and parameter sharing Xiaomi

[3] R3-256533 LS on Guidance on 6G data related work tasks China Moible

[4]R3-256511 LS to RAN1 and RAN3 on NW side data collection ZTE