3GPP TSG-RAN WG3 Meeting #129-bis	R3-256630

Prague, Czechia, 13 – 17 October 2025

Agenda item:	10.4

Source:	Nokia

Title:	6G RAN internal functional split and interfaces

Document for:	Discussion

# 1	Introduction

At RAN#108, the new SID on 6G RAN was agreed in [1].

The main RAN3 led objective to study is listed below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

As part of point c), RAN3 was tasked with a first milestone to achieve:

4.1.1	Interim Milestone

Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810].

TSG#112 (June/2026):

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

This paper addresses the objective (c) above and describes our view on how we can proceed with the above RAN3 task and also highlights the main challenges with 5G RAN internal split and interfaces.

# 2	Discussion

Given that RAN3 has a checkpoint to provide detailed technical analysis to allow TSGs to make a decision on RAN internal interfaces (CU-DU split and CP-UP split), it is imperative that we discuss the Higher Layer Split (HLS) in a systematic manner. The split consists of two possible interfaces:

- CU-DU (in 5G based on F1AP specification)

- CP-UP (in 5G based on E1AP specification)

In our view, discussion on the 2nd one, i.e. the CP-UP split, should be postponed until first decisions are made on the 1st interface, because the CP-UP split heavily depends on whether/how to support CU-DU split. Therefore, our proposal is to start discussing CU-DU split first, and to wait with the discussion on whether/how to support CP-UP split until some progress is made for CU-DU split.

Proposal 1: RAN3 to wait with the study on the CP-UP split (i.e. E1 interface) until some progress is made for CU-DU split (i.e. F1 interface).

In 5G, the CU-DU split (in 5G, the F1 interface) was introduced to enable flexible and possibly multi-vendor disaggregated RAN deployments. Figure 1 shows the 5G disaggregated architecture and interfaces between CU-CP and DU as well as CU-CP and CU-UP where the HLS is between PDCP and RLC layer of the protocol stack. However, until now, there have been very few commercial disaggregated multi-vendor deployments, which hints that the 5G HLS is not designed optimally, especially for control plane. Therefore, in this paper, we propose that 6G should have a fresh look into the HLS as it is specified for 5G RAN considering the facts on why there is not much commercial traction for multi-vendor HLS.

Observation 1: Multi-vendor disaggregated gNB deployment has not become the mainstream deployment in 5G networks.

Figure 1: 5G Disaggregation in 3GPP TS 38.401

One of the reasons why CU-DU split has not become truly multivendor interface is due to the way 3GPP has standardized the interface in 5G which makes it a very complex interface. At the same time, the performance benefits of having a multivendor disaggregated RAN are not high which does not provide enough justification for multivendor deployments given the additional integration and inter-operable testing complexities. Additionally, the control plane performance degrades as compared to that of the monolithic RAN. Specifically, in 5G CP HLS, the RRC is terminated in the CU, but the DU owns the radio resources and their management. For example, the CU is responsible for cell selection (admission) and establishment of the RRC connection for the User Equipment (UE). The same holds for cell/gNB changes during handovers. However, the DU manages the cell resources. This results in the fact that during RRC setup, a UE context must be setup both in the CU and the DU, and the UE context data must be aligned/coordinated at both nodes. As a result, many control plane procedures (e.g. UE initial access procedure and handover procedure) contain significantly more messages as compared to the monolithic RAN. Similarly, functional split in RRM configuration management also leads to complicated and suboptimal UE capabilities coordination between CU and DU. Such suboptimal UE capability management at the network leads to suboptimal RRM configuration which affects the overall system performance. In many deployments, propriety fixes were done to avoid the performance loss.

Observation 2: Functional split in 5G between CU and DU is not optimal because the CU acts as the RRC anchor, while most of the RRC resources are owned by DU. This leads to significant increase in control signalling and complex procedures compared to monolithic RAN.

Observation 3: Functional split in RRM configuration management leads to complicated and suboptimal UE capabilities coordination between CU and DU.

In the following, we describe the challenges that appear because of functional split between CU and DU in 5G.

- Challenge # 1: CP Latency

As a result of 5G HLS, the F1-C signalling related to the RRC coordination results in significant control plane latency for the UE(s). For example, it is observed that RRC procedures become slower in disaggregated RAN deployments as compared to monolithic RAN deployments. One may argue that this is not the major challenge since CP procedures are not that frequent. However, in our view, procedures like UE initial access and RRC reestablishment are the most frequent CP procedures, and they can impact the call setup time and service continuity. In addition, for the support of low latency services in edge clouds, optimized RRC is one important factor. Therefore, these CP procedures should be as latency optimized as possible.

Observation 4: CP latency of disaggregated RAN deployment is higher than monolithic RAN deployment.

- Challenge # 2: Unrealized pooling gains

One of the real motivations to do the CU-DU split is the centralization of CU at the (far-)edge sites which could provide significant pooling gains both in UP as well as CP. However, CP pooling gains get significantly impacted due to increased processing needs of F1 messages. Since, the existing 5G F1-C results in lots of ping-pong between CU and DU, it offsets the pooling gains one can expect due to centralization of CU.

Observation 5: Increased CP overhead over F1-C significantly decreases the pooling gains.

- Challenge # 3: RRC optimizations

As mentioned above, the current 5G HLS resulted in a split in RRC where some parameters are handled by CU and some are handled by DU. With such split, it becomes even harder to handle possible optimizations of the RRC procedures that may be introduced in RAN2 (which tend to get more and more complex in every release and generation due to large number of supported devices and features). Having an architecture that can flexibly adapt to such optimizations in 6G RRC is instrumental to make the whole system future proof and implementation friendly, not only from network’s but also from device’s perspective. Therefore, possible split between CU and DU parameters must avoid splitting the RRC handling as much as possible. Currently, 5G HLS hinders such optimizations.

Observation 6: 5G HLS complicates handling of possible RRC optimizations introduced in RAN2, which are instrumental for future proofness and supporting various device types/ UE capabilities.

- Challenge # 4: Implementation friendliness

By the time 6G networks will be commercialized, we expect that still both classic and cloud RAN deployments will coexist. This means that in order to cater for multiple services and their requirements (e.g. latency etc.) as well as reduce the total cost of ownership (TCO) for the 6G network, we will have to support all possible deployments where DU/CU may be present at the cell site in some cases but are centralized in the other cases. Therefore, we have to design the RAN architecture in such a way that it not only provides similar performance in all different deployment scenarios but also allows implementation friendliness. However, the way current 5G HLS is designed, it does not allow easy implementation to cater for different deployments.

Observation 7: Current 5G HLS does not allow implementation friendly design.

- Challenge # 6: Standardization complexity

Current 5G HLS went through multiple releases in 5G and, for every 5G feature, HLS procedures and optimizations were supported as an afterthought in the later release. Such procedures resulted in complex and sometimes incomplete F1 specifications. If specification is not ideal or complete, then network vendors may make their own "corrections" or additions to procedures/functionalities to make their networks operate properly. As a result, if different vendors have different implementations of CU and DU functionalities, then multi-vendor CU-DU operation becomes very difficult, which, in turn, jeopardises the main purpose of defining the interface (see above).

Observation 8: In 5G, feature support with 5G disaggregated RAN is always done as an after-thought which results in either complex or incomplete specifications.

Furthermore, due to unnecessary complexity of RRC protocol and because of F1 originated CP latency, it is not able to handle its main task of controlling radio resources. In order to do the faster control of radio resources, a lot of control information is added in the MAC headers which resulted it additional MAC layer complexities. Going forward, in 6G, 3GPP should avoid such interlinks of functionalities and keep the design simple where right protocol is used for the right purpose.

Observation 9: In 5G, due to unnecessary CP latency and complexity of RRC protocols, a lot of control information is added to MAC CEs which unnecessarily complicates the MAC layer.

- Challenge # 7: No standardized resiliency

Current 5G HLS does not support any standardized resiliency solution. The cardinalities are defined such that CU-CP becomes a single point of failure (e.g., a single CU connection per DU). Furthermore, true centralization gain of CUs comes when it is connected to multiple DUs and support large number of UEs. Therefore, it becomes very important that 3GPP defines an architecture where CU does not become a single point of failure to avoid service disruptions.

Observation 10: In 5G, CU may become a single point of failure causing service disruptions especially in centralized RAN deployments.

Based on the above challenges, we suggest revisiting in 6G the higher layer split and discuss whether and how CU-DU functional split should be supported and specified in 3GPP, respectively.

Proposal 2: RAN3 to study whether and how higher layer split is supported for 6G.

# 3	Conclusion

In this contribution, first, we propose to separate the discussions on CU-DU and CP-UP splits:

Proposal 1: RAN3 to wait with the study on the CP-UP split (i.e. E1 interface) until some progress is made for CU-DU split (i.e. F1 interface).

Later, we made the following observations on the CU-DU split:

Observation 1: Multi-vendor disaggregated gNB deployment has not become the mainstream deployment in 5G networks.

Observation 2: Functional split in 5G between CU and DU is not optimal because the CU acts as the RRC anchor, while most of the RRC resources are owned by DU. This leads to significant increase in control signalling and complex procedures compared to monolithic RAN.

Observation 3: Functional split in RRM configuration management leads to complicated and suboptimal UE capabilities coordination between CU and DU.

Observation 4: CP latency of disaggregated RAN deployment is higher than monolithic RAN deployment.

Observation 5: Increased CP overhead over F1-C significantly decreases the pooling gains.

Observation 6: 5G HLS complicates handling of possible RRC optimizations introduced in RAN2, which are instrumental for future proofness and supporting various device types/ UE capabilities.

Observation 7: Current 5G HLS does not allow implementation friendly design.

Observation 8: In 5G, feature support with 5G disaggregated RAN is always done as an after-thought which results in either complex or incomplete specifications.

Observation 9: In 5G, due to unnecessary CP latency and complexity of RRC protocols, a lot of control information is added to MAC CEs which unnecessarily complicates the MAC layer.

Observation 10: In 5G, CU may become a single point of failure causing service disruptions especially in centralized RAN deployments.

Based on the above observations, we made the following proposal:

Proposal 2: RAN3 to study whether and how higher layer split is supported for 6G.

# 4	References

- RP-252912, Revised SID: Study on 6G Radio.