3GPP TSG-RAN WG3 #129-bis                                                 R3-256631

Prague, CZ, 13th – 17th October, 2025

Agenda Item:	9.2.6

Source:	CATT, Nokia, Nokia Shanghai Bell, ZTE Corporation, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Title:	Further discussion on NG Suspend/Resume

Document for:	Discussion & Decision

- Introduction

In the previous RAN3 meetings, we discuss whether and how to support NG/S1 suspend/resume for regenerative payload. In the last RAN3 meeting, there’s a common understanding that short NG/S1 interruption, causing e.g. packet loss, that may be predictable or unpredictable, e.g. hard FLSO to be further discussed in Q4, as highlighted below:

Remove text from BL CRs related to suspend/resume short NG/S1 interruption, causing e.g. packet loss, that may be predictable or unpredictable, e.g. hard FLSO to be further discussed in Q4

In this contribution, we will continue discuss the remaining issues on NG/S1 suspend/resume. On top of the discussion, we will provide the observations and proposals accordingly.

- Discussion

To explain the issue and solution clearly, the scenario of hard feeder link switch for regeneration payload is illustrate with the figure below.

Figure1. Hard feeder link switch for regeneration payload

As shown in the figure above, before time T1, the onboard gNB is connected with the old NTN-GW. The gNB is about to initiate the hard feeder link switch procedure at T1, which means, the on-board gNB will disconnect with the NTN-GW 1 from T1, and start to establish the feeder link towards the target NTN-GW, i.e. NTN-GW 2 in this example.

At time T2, the feeder link switch is taking place and the gNB is not connected with any NTN-GW.

At time T3, the new feeder link is ready, the hard feeder link switch procedure is completed. There’s a “GAP” between T1 to T3 without any available feeder link between the on-board gNB and NTN-GW. Thus, NG connection is interrupted in this duration.

Observation 1: Interruption time is unavoidable for hard feeder link switch, between the break of old feeder link and the completion of the new feeder link establishment.

Due to variations in satellite capabilities, antenna types (phased array/mechanical), link quality, and orbital characteristics, the interruption time is uncertain and unpredictable, typically ranging from milliseconds to seconds. The duration of the interruption of different hard feeder link switch may not be well pre-planned.

Observation 2: Due to variations in satellite capabilities, antenna types (phased array/mechanical), link quality, and orbital characteristics, the interruption time is uncertain and unpredictable, typically ranging from milliseconds to seconds.

We understand that, the 5GC could be configured with the initiation time or condition of the feeder link switch in advance for each of the NGSO satellite by pre configuration or OAM, it remains unable to accurately know the exact completion time of each feeder link switch.

Observation 3: Even though the 5GC could be configured with the initiation time of hard feeder link switch in advance by pre configuration or OAM, it remains unable to accurately know the exact completion time of hard feeder link switch.

During the hard feeder link switch, with the AMF remaining unchanged, the UE within the satellite service range remains unaware of the handover. After time T1, when the old power feed link disconnects, the 5GC needs to decide whether to immediately transmit DL packets to the new NTN-GW or after a certain delay (e.g., 100 milliseconds). If the 5GC immediately sends DL packets to the new NTN-GW upon disconnection of the old feeder link, while the new feeder link has not yet been established, the new NTN-GW is required to buffer the DL packets. However, in satellite communications, the Gateway primarily functions as a forwarding module and is not designed for data buffering. Consequently, when the new Gateway receives the DL packets before the new feeder link is properly established, it may encounter operational confusion. As the data processing module, the UPF is more suitable for buffering data. But if the 5GC's buffering duration is too short (e.g., 100 milliseconds after the old feeder link disconnection), the new feeder link may not yet be established. Conversely, if the buffering duration is too long (e.g., 1-2 seconds), the UPF may need to buffer more data, potentially delaying transmission to the gNB/UE. So the 5GC needs to know when the new feeder link will be established in order to promptly transmit data to the new Gateway. Therefore, in case of NG Connection interruption due to hard feeder link switch over, 5GC needs to know the start and completion time of the feeder hard handover.

Observation 4: In case of NG Connection interruption due to hard feeder link switch over, 5GC needs to know the start and completion time of the hard feeder link switch to timely transmit data to the new NTN-GW.

To solve the above problem, it is beneficial to introduce the new indicator to suspend or resume NG connection. Specifically, the gNB clearly informs the 5GC before the hard feeder link switch, and then the 5GC can suspend the NG connection and buffer the DL packet. And when the new feeder link is activated, the gNB could notify the 5GC to resume NG connection and continue the DL NG-C/U packets transmission via the new NTN-GW.

As above, for hard feeder link switch case, it is beneficial to introduce the new indicator in RAN Configuration Update to suspend or resume NG connection.

Proposal 1: For hard feeder link switch case, it is beneficial to introduce the new indicator in RAN Configuration Update to suspend or resume NG connection to solve packet loss.

Corresponding stage 2 CR for TS 38.300 is provided in [2]. For the stage 3, introduce the “NG Connection Control” IE in the RAN Configuration Update message for hard feeder link switch. Corresponding stage 3 CR for TS 38.413 is provided in [3].

RAN3 is requested to discuss and agree the CRs on support of NG Suspend/Resume for NR NTN.

Proposal 2: Discuss and agree the CRs on support of NG Suspend/Resume for NR NTN.

Similarly, for IoT NTN, S1 Suspend/Resume should also be supported. The same design as NR NTN could be applied. Corresponding CRs to the stage 2 and stage 3 are provided in [4][5]. RAN3 is requested to discuss and agree corresponding CRs.

Proposal 3: Discuss and agree the TPs on support of NG Suspend/Resume for IoT NTN.

As the solution above introduced the new indicators into the RAN Configuration Update, and 5GC is expected to do corresponding suspend/resume for the DL signaling and data transmission according to the indicators. It’s better to send the LS to SA2, to inform our solution, and let them take it into account for their work. The draft LS is provided in [6].

Proposal 4: It is proposed to send LS to SA2 on support of NG/S1 suspend and resume, the draft LS is provided in [6].

- Conclusion

Based on the discussion in section 2, we have the following observations and proposals:

Observation 1: Interruption time is unavoidable for hard feeder link switch, between the break of old feeder link and the completion of the new feeder link establishment.

Observation 2: Due to variations in satellite capabilities, antenna types (phased array/mechanical), link quality, and orbital characteristics, the interruption time is uncertain and unpredictable, typically ranging from milliseconds to seconds.

Observation 3: Even though the 5GC could be configured with the initiation time of hard feeder link switch in advance by pre configuration or OAM, it remains unable to accurately know the exact completion time of hard feeder link switch.

Observation 4: In case of NG Connection interruption due to hard feeder link switch over, 5GC needs to know the start and completion time of the hard feeder link switch to timely transmit data to the new NTN-GW.

Proposal 1: For hard feeder link switch case, it is beneficial to introduce the new indicator in RAN Configuration Update to suspend or resume NG connection to solve packet loss.

Proposal 2: Discuss and agree the CRs on support of NG Suspend/Resume for NR NTN.

Proposal 3: Discuss and agree the TPs on support of NG Suspend/Resume for IoT NTN.

Proposal 4: It is proposed to send LS to SA2 on support of NG/S1 suspend and resume, the draft LS is provided in [6].

- Reference

- RAN3_129_agenda_20250828_1900.docx

- R3-256632 Support of NG Suspend and Resume (TS38.300 draftCR), CATT, Nokia, Nokia Shanghai Bell, ZTE Corporation, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

- R3-256675 NG Suspend and Resume during Hard FLSO (TS38.413 CR), ZTE Corporation, Nokia, Nokia Shanghai Bell, CATT, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

- R3-256614 Suspend/resume S1 during hard FLSO (TS36.300 draftCR), Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, ZTE, CATT, China Telecom, CMCC, LG Electronics, Xiaomi, Samsung

- R3-256732 Suspend/resume S1 during hard FLSO (TS36.413 CR), Nokia, Nokia Shanghai Bell, ZTE Corporation, CATT, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

- R3-256633 [DRAFT] LS on Support of NG/S1 Suspend and Resume, CATT