3GPP TSG-RAN WG3 Meeting #129-bis	R3-256635

Prague, CZ, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1319 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to A-IoT procedure texts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A couple of corrections to the procedure texts for A-IoT related procedures: - “If Inventory Complete Indication IE is included in the INVENTORY REPORT message, the A-IoT CN node shall consider that the inventory operation is completed in the NG-RAN node.” This is not exactly correct, as one or more inventory procedures maybe ongoing in the NG-RAN, we should clearly say that “the inventory operation for the A-IoT session denoted by the AIOTF Identifier IE and the A-IoT Correlation Identifier IE is completed in the NG-RAN node”. - In 8.20.2.2, interaction between the Inventory Complete Indication and the A-IoT Session Release procedure, we understand that not necessary to bind the complete indication in the Inventory Report and the A-IoT Session Release procedure. Thus, it’s better to say the A-IoT CN node may initiate A-IoT Session Release procedure. - For the procedure texts for Command Failure in 8.20.3, “the command” should be clarified, it’s better to use “A-IoT NAS PDU in the Command Request Transfer IE in the COMMAND REQUEST message”. - “Correlation ID” is not defined, it should be replaced with “A-IoT Correlation Identifier” in 8.20.4 and 8.20.5. - A few edritorials, e.g. duplicated “the NG-RAN node” in 8.20.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify which “inventory operation” is completed in the NG-RAN node in 8.20.2.2. - Change “should” to “may” for the Interactions with the A-IoT Session Release procedure in 8.20.2.2. - Clarify the “command” is “A-IoT NAS PDU received in the COMMAND REQUEST message” in 8.20.3.1 and 8.20.3.3. - Replace “Correlation ID” with “A-IoT Correlation Identifier” in 8.20.4 and 8.20.5. - Remove duplicated “ the NG-RAN node” in 8.20.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The A-IoT procedure texts are not exactly correct, which may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.20.2, 8.20.3, 8.20.4, 8.20.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------Start of the First Change-----------------

### 8.20.2	Inventory Report

#### 8.20.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide inventory report related information to the A-IoT CN node, following a successful Inventory Request procedure. This procedure applies only if the NG-RAN node is a gNB.

#### 8.20.2.2	Successful Operation

Figure 8.20.2.2-1: Inventory Report

The NG-RAN node initiates the procedure by sending an INVENTORY REPORT message for the Inventory service.

If Inventory Complete Indication IE is included in the INVENTORY REPORT message, the A-IoT CN node shall consider that the inventory operation for the A-IoT session denoted by the AIOTF Identifier IE and the A-IoT Correlation Identifier IE is completed in the NG-RAN node.

Interactions with the A-IoT Session Release procedure:

In case the Inventory Complete Indication IE is included in the INVENTORY REPORT message and set to “true”, if there is no follow-on command to be transmitted, or all the follow-on command transmissions have been completed, the A-IoT CN node may initiate the A-IoT Session Release procedure for the A-IoT session denoted by the AIOTF Identifier IE and the A-IoT Correlation Identifier IE contained in the INVENTORY REPORT message.

#### 8.20.2.3	Abnormal Conditions

Void.

### 8.20.3	Command Request

#### 8.20.3.1	General

The purpose of the Command Request procedure is to request the NG-RAN node to transfer the command (A-IoT NAS PDU) to the A-IoT device over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

#### 8.20.3.2	Successful Operation

Figure 8.20.3.2-1: Command Request, successful operation.

The A-IoT CN node triggers the procedure by sending a COMMAND REQUEST message to the NG-RAN node. In case of successful operation, the NG-RAN node responds with the COMMAND RESPONSE message.

The NG-RAN node shall, include the same values of the AIOTF Identifier IE, the A-IoT Correlation Identifier IE and the RAN A-IoT Device NGAP ID IE in the COMMAND RESPONSE message as received from the COMMAND REQUEST message.

If the Command Assistance Information IE is included in the COMMAND REQUEST message, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

#### 8.20.3.3	Unsuccessful Operation

Figure 8.20.3.3-1: Command Request, unsuccessful operation.

If the NG-RAN node is not able to transmit the received A-IoT NAS PDU in the Command Request Transfer IE in the COMMAND REQUEST message towards the A-IoT device, it shall send the COMMAND FAILURE message with an appropriate cause value.

#### 8.20.3.4	Abnormal Conditions

Void.

### 8.20.4	A-IoT Session Release

#### 8.20.4.1	General

The purpose of the A-IoT Session Release procedure is to enable the A-IoT CN node to terminate all activities related to the indicated A-IoT session. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

#### 8.20.4.2	Successful Operation

Figure 8.20.4.2-1: A-IoT Session Release, successful operation.

The A-IoT CN node triggers the procedure by sending a AIOT SESSION RELEASE COMMAND message to the NG-RAN node. Upon reception of AIOT SESSION RELEASE COMMAND the NG-RAN node shall terminate all activities and release the context and resources related to the A-IoT session denoted by the AIOTF Identifier IE and the A-IoT Correlation Identifier IE and respond with an AIOT SESSION RELEASE COMPLETE message.

#### 8.20.4.3	Unsuccessful Operation

Not applicable.

#### 8.20.4.4	Abnormal Conditions

Void.

### 8.20.5	A-IoT Session Release Request

#### 8.20.5.1	General

The purpose of the A-IoT Session Release Request procedure is to enable the NG-RAN node to request the A-IoT CN node to release the A-IoT session. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

#### 8.20.5.2	Successful Operation

Figure 8.20.5.2-1: A-IoT Session Release Request

The NG-RAN node initiates the procedure by sending an AIOT SESSION RELEASE REQUEST message to the A-IoT CN node.

Interactions with the A-IoT Session Release procedure:

Upon reception of the AIOT SESSION RELEASE REQEUST message, the A-IoT CN node should initiate the A-IoT Session Release procedure for the A-IoT session denoted by the AIOTF Identifier IE and the A-IoT Correlation Identifier contained in the AIOT SESSION RELEASE REQUEST message.

#### 8.20.5.3	Abnormal Conditions

Void.

-----------------End of the Changes-----------------