3GPP TSG RAN WG 3 # 129-bis											R3-256638

Prague, Czech Republic, October 13th-17th, 2025

Agenda Item:	10.2.1

Source:	CSCN

Title:	Discussion on the general principles and requirements of 6G NTN

Document for:	Discussion/Decision

- Introduction

In RAN#109 meeting, RP-252882[1] was approved on the section of requirements for architecture and migration, as shown below, and mentioned that “The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.” There is also detailed object captured in RP-252912[2] that “Aim at a harmonized 6G Radio design for TN and NTN, including their integration.” Meanwhile, RP-2873[3] endorsed 6G should study further whether and how to support NTN-specific device types.

## Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: ... - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

4.1 Objective of SI or Core part WI or Testing part WI ... The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] … j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration. … Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

Proposal 3: To investigate further: … - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. … Note: the terminology “device type” is subject to further discussion and possible refinement.

We believe that 6G should natively support NTN from Day1, and here we present our views on the general aspects including deployment scenarios, architecture and interface.

- Discussion

6G NTN deployment scenarios are defined to allow for the provision of services for those areas including vertical space (e.g., up to several kilometers altitude) where terrestrial network service is not available (on a permanent or temporary basis), and also for those services that can be more efficiently supported by Non-Terrestrial Networks.

Non-Terrestrial Networks can overcome coverage limitations of terrestrial networks especially in low-density populated areas and maritime areas.

As such, NTN will mainly contribute to:

- Achieve ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improve overall 6G network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables by supporting in priority critical communications and emergency communications (messaging, voice calls).

6G NTN should support both transparent and regenerative payloads as in 5G and also support any orbits ranging from VLEO (300 to 600 km altitude) to GSO (~35786 km altitude), including LEO (600 to 2000 km altitude) and MEO (2000 to ~35000 km altitude). NTN may be based on a combination of orbit types (e.g., GSO and LEO). In order to enhance the overall system capacity, resilience and service flexibility, multi-orbit collaboration among LEO, MEO and GSO satellites should considered. Given that the satellite constellations are currently in the deployment phase, 6G NTN should consider the evolutionary scaling from sparse to dense satellite constellations. To achieve seamless global coverage with ubiquitous connectivity, the implementation of a space-air-ground integrated network is essential. A critical component of this architecture involves the integration of NTN with TN. The NTN networking framework for 6G is illustrated in Figure 1.

Figure 1: NTN networking framework for 6G

  - Requirements for architecture and function

While the RAN#109 has provided guidance on the design of 6G architectures, including TN-NTN integration and architecture consideration for both NTN and TN, we believe that NTN characteristics still differ significantly those of 6G terrestrial networks in terms of cell deployment formats and data transmission (such as high latency, dynamic variations, and limited link budget, etc.).

    - Mobility

The 6G RAN should be able to support service continuity in both idle and connected modes and strive to minimize service degradation between TN and NTN network components. Therefore, TN-NTN mobility in 6G should be considered to ensure seamless service continuity. Given that NR NTN has been introduced in 5G, the 6G RAN architecture shall support inter-RAT mobility including 5G TN-6G NTN, 5G NTN-6G TN and 5G NTN-6G NTN.

RAN3 to study 6G architecture and functions for mobility including 6G TN-6G NTN, 5G TN-6G NTN, 5G NTN-6G TN and 5G NTN-6G NTN.

    - NTN-TN/Multi-orbit Coordination

In 5G, the NTN control function has been studied and captured in TR38.821 [4].

The gNB may be subdivided into non-NTN infrastructure gNB functions and the NTN Service Link provisioning system. The NTN infrastructure may be thought of being subdivided into the NTN Service Link provisioning system and the NTN Control function. ... The NTN control function controls the spaceborne (or airborne) vehicles as well as the radio resources of the NTN infrastructure (NTN payload(s) & NTN Gateway(s)). It provides control data, e.g. Ephemeris, to the non-NTN infrastructure gNB functions of the gNB.

Regarding NTN-TN integration scenarios, NTN configurations need to be aware of the TN infrastructure, and some TN control data also need to be provided for the NTN infrastructure. In multi-orbit satellite networks, large inter-satellite distances, high relative velocities, and dynamic neighbor relationships make it challenging for xNB-to-xNB interaction via inter-satellite/feeder links. Coordinated control node/function is needed to at least exchange necessary control data (e.g., ephemeris, frequency, etc.), perform overall resource allocation and mitigate interference for NTN-TN and multi-orbit coordination scenarios. Consequently, a centralized node/function for unified and coordinated control is required to introduce in 6G.

RAN3 to study coordinated control node/function for NTN-TN/Multi-orbit coordination.

  - Requirements for Interface

6G NTN interface design should consider the characteristics of Inter-Satellite Links (ISL) and feeder link (e.g. long delays, high dynamics and predictability).

Satellites operate at altitudes ranging from hundreds of kilometers (LEO) to 35786 km (GSO). The large propagation distances result in high transmission delays, from milliseconds to hundreds of milliseconds. These delays significantly affect signaling interactions between the onboard xNB and core network on ground. The interface between xNB and core network need to consider the long delay characteristic of feederlink.

In the regenerative payload, the movement of NGSO satellites leads to continuous and rapid changes in connectivity among UEs, satellites, and gateways. The topology relationships between xNBs and CN, as well as among xNBs, dynamically change. It is recommended to leverage the predictability of satellite orbital patterns to mitigate the impacts of satellite high mobility which has benefit for efficient and stable interfaces interaction.

RAN3 to consider the predictable characteristic of Inter-Satellite Links (ISL) and feeder link for 6G NTN interface design.

  - Requirements for MDT

The global coverage deployment of satellites introduces a challenge in inaccessible dangerous areas and incomplete test coverage of testing.MDT (Minimization of Drive Tests) is an essential way for NTN operators. The application of MDT in satellite communication enables the utilization of NTN UEs to automatically collect and report wireless measurement data. This method replaces costly and inefficient traditional drive tests, enabling efficient optimization of global satellite network performance. However, NTN UEs are typically constrained by power consumption and radio link conditions, resulting in incapable of supporting longtime, extensive, or complex MDT procedures. ‌Meanwhile, UEs at same locations may acquire test data from different regenerative satellites at different time.‌ Therefore, considering the aforementioned factors, it is recommended to design a more UE-friendly and efficient test data collection and reporting procedure specifically for NTN scenarios.

RAN3 to study UE-friendly and efficient test data collection and reporting procedure in MDT for 6G NTN.

- Conclusion

Proposal 1: RAN3 to study 6G architecture and functions for mobility including 6G TN-6G NTN, 5G TN-6G NTN, 5G NTN-6G TN and 5G NTN-6G NTN.

Proposal 2: RAN3 to study coordinated control node/function for NTN-TN/Multi-orbit coordination.

Proposal 3: RAN3 to consider the predictable characteristic of Inter-Satellite Links (ISL) and feeder link for 6G NTN interface design.

Proposal 4: RAN3 to study UE-friendly and efficient test data collection and reporting procedure in MDT for 6G NTN.

- Reference

- RP-252882, TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, CMCC, 3GPP TSG#109, Beijing, China, September 15-18, 2025

- RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, 3GPP TSG#109, Beijing, China, September 15-18, 2025

- RP-252873, Moderator's summary on 6G diverse device types (incl. minimum UE bandwidth), Apple, 3GPP TSG#109, Beijing, China, September 15-18, 2025

- TS 38.300, NR and NG-RAN Overall Description; Protocol specification, v18.6.0