3GPP TSG-RAN WG3 Meeting #129bis	R3-256643

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	Consideration on Interface Management procedures for A-IoT

Agenda Item:	9.2.7

Source:	Huawei, CMCC, Lenovo, China Unicom, China Telecom

Document for:	Discussion

# Introduction

Most of the NGAP interface mangamgent procedures are reused by A-IoT, including e.g., NG Setup and RAN Configuration Update, etc.

In this contribution, we discuss two issues for NG Setup and RAN Configuration Update procedures in case of A-IoT.

# Discussion

  - Missing A-IoT Support IE in RAN CONFIGURATION UPDATE message

The A-IoT Support IE ENUMERATED (A-IoT only, A-IoT and NR Uu, ...) was introduced in NG SETUP REQUEST message, but not in the RAN CONFIGURATION UPDATE message. Therefore, if the gNB only supports NR-Uu at the beginning, after NG setup, if it is needed to update the gNB to also support A-IoT radio, we will have to re-setup the NG interface, this will lead to severe impacts on the ongoing NR operations. Therefore, it is needed to also include the A-IoT Support IE in the RAN CONFIGURATION UPDATE message.

Proposal 1: Include the A-IoT Support IE in the RAN CONFIGURATION UPDATE message

  - Missing A-IoT Support PLMN List in NG SETUP REQUEST and RAN CONFIGURATION UPDATE messages

In case the gNB only supports A-IoT, the A-IoT Support IE is included in NG Setup and indicates that the gNB only supports A-IoT, then the CN node will ignore the received Supported TA List IE. Therefore, the CN node will not be able to get the supported PLMN List of the gNB.

In case the gNB supports both A-IoT and NR, although the Supported TA List IE will not be ignored, but it is unclear whether the contained PLMN info covers the supported PLMN of the A-IoT radio.

Considering that the AIOTF selects gNBs to perform inventory, it is needed to provide the A-IoT supported PLMN list to the CN node. Note that in case of SNPN, the NID will be provided together with the PLMN identity.

Proposal 2: Introduce an A-IoT Supported PLMN List IE in the NG SETUP REQUEST and RAN CONFIGURATION UPDATE messages.

# Conclusion

In this paper we discuss two issues for A-IoT interface management procedures, get the following proposals:

Proposal 1: Include the A-IoT Support IE in the RAN CONFIGURATION UPDATE message

Proposal 2: Introduce an A-IoT Supported PLMN List IE in the NG SETUP REQUEST and RAN CONFIGURATION UPDATE messages.

The corresponding NGAP CR is provided in [1].

# Reference

[1]	R3-256644 Correction on A-IoT interface management procedures, R19 NGAP CR