3GPP TSG-RAN WG3 #129bis                                                                          R3-256660

Prague, Czech, Oct. 13~17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1590 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on beam information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R19, the BFR beam information, the Target beam information after cell switch failure, and TA information are introduced to support MRO enhancement on intra-CU LTM. And currently the beam information directly uses SSB index. However, in order to support R20 MRO enhancement on inter-CU LTM, it has already been expected that the beam information may correspond not only to SSB index, but also to e.g. CSI-RS index. In order to ensure a future proof way of specifying R19 spec. It is better to use a more general IE name. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update IE name: BFR SSB Index IE -> BFR Beam information IE. And put SSB Index IE as a sub-IE of the updated IE. - Update IE name: Target SSB Index after Cell Switch Failure IE -> Target Beam Information after Cell Switch Failure IE. And put SSB Index IE as a sub-IE of the updated IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The future-proofness cannot be ensured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.1.2, 9.2.10.1, 9.2.10.2, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.401 CR0482 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8	F1AP procedures

[snip]

8.11	Self Optimisation Support procedures

[snip]

8.11.1	Access and Mobility Indication

8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the BFR Beam Information IE within the MRO for LTM Information IE, the gNB-DU shall, if supported, consider that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the Target Beam Information after Cell Switch Failure IE within the MRO for LTM Information IE, the gNB-DU shall, if supported, consider that a successful recovery or re-establishment occurred in this beam after an LTM Cell Switch Failure for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection or optimisation of Timing Advance related parameters for the UE identified by the gNB-DU UE F1AP ID IE.

If the ACCESS AND MOBILITY INDICATION message contains the TA Information IE within the MRO for LTM Information IE, the gNB-DU may take it into account for optimisation of Timing Advance related parameters or optimisation of target beam selection for the UE identified by the gNB-DU UE F1AP ID IE.

[snip]

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU. This IE is included in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| Reporting without RLF report |  | 0..1 |  |  |  |  |
| >NR CGI | M |  | 9.3.1.12 | NR CGI for the re-establishment or recovery cell. | - |  |
| >C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| MRO for LTM Information |  | 0..1 |  |  | YES | ignore |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| >BFR Beam Information |  | 0..1 |  |  | - |  |
| >>SSB Index | O |  | INTEGER (0..63) | SSB Index of the recovery beam used at successful Beam Failure Recovery. | - |  |
| >Target Beam Information after Cell Switch Failure |  | 0..1 |  |  | - |  |
| >> SSB Index | O |  | INTEGER (0..63) | SSB Index of the re-established or recovery beam after LTM Cell Switch Failure. | - |  |
| >TA Information | O |  | INTEGER (0..4095) | Indicates the TA value, as defined in TS 38.213 [31], used at successful Random Access during LTM recovery or re-establishment after a Cell Switch failure in same cell. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| > DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| MRO for LTM Information |  | 0..1 |  |  | YES | ignore |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >BFR Beam Information |  | 0..1 |  |  | - |  |
| >>SSB Index | O |  | INTEGER (0..63) | SSB Index of the recovery beam used at successful Beam Failure Recovery. | - |  |
| >Target Beam Information after Cell Switch Failure |  | 0..1 |  |  | - |  |
| >>SSB Index | O |  | INTEGER (0..63) | SSB Index of the re-established or recovery beam after LTM Cell Switch Failure. | - |  |
| >TA Information | O |  | INTEGER (0..4095) | Indicates the TA value, as defined in TS 38.213 [31], used at successful Random Access during LTM recovery or re-establishment after a Cell Switch failure in same cell. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |

[snip]

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

[snip]

FROM F1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM F1AP-CommonDataTypes

ProtocolExtensionContainer{},

F1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

[snip]

MROForLTM-Information ::= SEQUENCE {

gNB-CUorDU-UE-F1AP-ID		GNB-CUorDU-UE-F1AP-ID,

bFR-Beam-Information				BFR-Beam-Information						OPTIONAL,

target-Beam-Information-Failure	Target-Beam-Information-Failure						OPTIONAL,

tA-Value					TAValue							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MROForLTM-Information-ExtIEs } } OPTIONAL,

...

}

MROForLTM-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BFR-Beam-Information ::= SEQUENCE {

sSBIndex				INTEGER(0..63)	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { BFR-Beam-Information-ExtIEs} } OPTIONAL

}

BFR-Beam-Information-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Target-Beam-Information-Failure ::= SEQUENCE {

sSBIndex				INTEGER(0..63)	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { Target-Beam-Information-Failure-ExtIEs} } OPTIONAL

}

Target-Beam-Information-Failure-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}