3GPP TSG-RAN WG3 #129bis                                                                          R3-256661

Prague, Czech, Oct. 13~17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1324 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on AIoT IE encoding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In NGAP for AIoT, some IE encoding needs to be revisited. The latest TS 23.003 has clearly specified the structure of AIoT Device Permanent Identifier as well as the structure of Filtering Information shall be coded as the binary representation of their value. So the annotation in asn.1 part for the encoding of singleDevice and groupDevices can be removed in NGAP. The latest TS 29.369 has clearly specified the following text for lastKnownAiotfId for Nadm_DM API: lastKnownAiotfId: $ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId' Such description can also be used for the IE encoding of AIoTF identifier in NGAP. So the annottion in asn,1 paer for the encoding of AIOTFIdentifier can be removed in NGAP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the annotation associated with AIoT-DeviceIdentificationRequested and AIOTFIdentifier in ASN.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The encoding of IEs are not settled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.3.3.xx2	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identifier | M |  | OCTET STRING | Coded as the NfInstanceId IE defined in TS 29.571 [35]. |

9.3.3.xx5	A-IoT Device Identification Requested

This IE represents the Requested Identity of the A-IoT device(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Device Identification for Paging | M |  |  |  |
| >Single Device |  |  |  |  |
| >>Single Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the AIoT device Identifier defined in TS 23.369 [z]. |
| >Group Devices |  |  |  |  |
| >>Group Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the Filtering Information defined in TS 23.369 [z]. |
| >All Devices |  |  | NULL |  |

-------------------------------------------Start of changes-------------------------------------------

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

--Skip unchanged--

AIoT-DeviceIdentificationRequested ::= CHOICE {

singleDevice					BIT STRING (SIZE(1..1024, ...)),

groupDevices					BIT STRING (SIZE(1..1024, ...)),

allDevices						NULL,

choice-Extensions		ProtocolIE-SingleContainer { { AIoT-DeviceIdentificationRequested-ExtIEs} }

}

AIoT-DeviceIdentificationRequested-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

AIoT-DeviceReportList ::= SEQUENCE (SIZE(1..maxnoofDevices)) OF AIoT-DeviceReportItem

AIoT-DeviceReportItem ::= SEQUENCE {

aIoT-NASPDU					AIoT-NASPDU,

rAN-AIOT-Device-NGAP-ID		RAN-AIOT-Device-NGAP-ID									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-DeviceReportItem-ExtIEs} }	OPTIONAL,

...

}

AIoT-DeviceReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIOTFIdentifier ::= OCTET STRING

AIOTFName ::= SEQUENCE {

aIOTFNameVisibleString		AIOTFNameVisibleString				OPTIONAL,

aIOTFNameUTF8String			AIOTFNameUTF8String					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { AIOTFName-ExtIEs } } OPTIONAL,

...

}

AIOTFName-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-------------------------------------------End of changes-------------------------------------------