3GPP TSG-RAN WG3 #129bis                                    R3-256663

# Prague, Czech, Oct. 13~17, 2025

Agenda item:	9.2.14

Source: 	Samsung, CATT, ZTE, Huawei

Title: 	Discussion on positioning activation and deactivation procedure

Document for:	Discussion and decision

Introduction

In Release 18, RAN3 discussed positioning activation procedure for area-specific SRS transmission. However, in Rel 18, RAN3 mainly focused on how to activate periodic area-specific SRS transmission, while lacked the discussion on how to activate semi-persistent area-specific SRS transmission.

In addition, how to support SP SRS deactivation also needs further investigation from RAN3 perspective.

This open issue has been discussed last two meetings. According to the progress of last meeting, companies agreed to continue to discuss this open issue on the basis of minimizing spec impact.

In this contribution, we discuss how to support the SP SRS activation and deactivation procedure, targeting at minimizing spec impact.

Discussion

In legacy mechanism, for SP SRS activation over NRPPa, the LMF is able to trigger UE-associated Positioning Activation procedure to activate ONE SRS Resource Set and optionally with its spatial relation information as follows,

9.1.1.17	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 | This IE is ignored if the Aggregated Positioning SRS Resource Set List IE below is present. | - |  |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | YES | ignore |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | YES | ignore |

In addition, for SP SRS deactivation over NRPPa, the LMF is also able to trigger UE-associated Positioning Deactivation procedure to deactivate ONE SRS Resource Set or Release ALL as follows,

9.1.1.20	POSITIONING DEACTIVATION

This message is sent by the LMF to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE Abort Transmission | M |  |  |  | YES | ignore |
| >Deactivate SRS Resource Set |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - |  |
| >Release ALL |  |  | NULL | the NG-RAN node configures UE to stop transmitting SRS for the positioning purpose |  |  |

Observation 1: For SP SRS activation and deactivation over NRPPa, the LMF is able to activate and deactivate SRS transmission on a SRS Resource Set basis, which is different from Periodic SRS activation (i.e. the whole periodic SRS configuration including ALL configured SRS Resource Sets are activated for Periodic SRS activation).

In addition, it is RAN2 common understanding to reuse the legacy mechanism to activate/deactivate the SP SRS by means of SP SRS Activation/Deactivation MAC CE as defined in TS 38.321.

Observation 2: It is RAN2 common understanding to reuse the existing SP SRS Activation/Deactivation MAC CE for area-specific SP SRS activation/deactivation.

As a result, RAN3 needs to discuss which node decides the Activated/Deactivated SRS Resource Set(s) and how to perform the corresponding coordination at the network side for area-specific SP SRS.

Observation 3: RAN3 needs to discuss which node decides the Activated/Deactivated SRS Resource Set(s) and how to perform the corresponding coordination at the network side for area-specific SP SRS, also taking split architecture into account.

2.1 SP SRS Activation procedure

Generally speaking, there are three options for activation decision for area-specific SP SRS,

- Option 1: LMF makes the activation decision

- Option 2: Last Serving gNB makes the activation decision

- Option 3: Receiving gNB makes the activation decision

And both anchor relocation case and no anchor relocation case should be considered.

As captured in R3-255187 submitted last meeting, the analysis on each option is provided as follows,

| Activation decision by | Anchor relocation | No anchor relocation |
| --- | --- | --- |
| LMF | Follows similar logic as for non-area-specific SRS configuration. Two additional messages during activation procedure. No stage 3 impact./ Small Stage 3 impact on XnAP. | Follows similar logic as for non-area-specific SRS configuration. Two additional messages during activation procedure. Small Stage 3 impact on XnAP. |
| Last Serving gNB | No additional message during activation procedure. Small Stage 3 impact on XnAP. Small Stage 3 impact on NRPPa. | No additional message during activation procedure. Small Stage 3 impact on XnAP. Small Stage 3 impact on NRPPa. |
| Receiving gNB | One additional message during activation procedure. Small Stage 3 impact on NRPPa. | No/Two additional message(s) during activation procedure. Big Stage 3 impact on XnAP. Big Stage 3 impact on NRPPa. |

According to the discussion last two meetings, companies seemed to have different flavours on which option to go. More specifically, some company commented that Option3 (Receiving gNB activation) is more compliant to the original intention in R18; some other company preferred to follow Option1 (LMF activation) which may have the minimum spec impact; while some other company suggested to adopt Option2 (Last Serving gNB activation) since it may be the simplest approach.

In order to move forward, we suggest companies to take the above table into account when down selecting options. Considering this is a late stage, we prefer the option with minimum spec impact (i.e. Option1).

Proposal 1: LMF makes the activation decision for area-specific SP SRS.

If the LMF makes the activation decision for area-specific SP SRS transmission, the detailed activation procedure is provided as follows,

For no anchor relocation case,

The activation procedure for Option 1 is as follows,

Under this case,

- For Stage 2, two additional messages for NRPPa Positioning Activation procedure is needed between LMF and Receiving gNB after the Receiving gNB becomes the new Anchor gNB.

- For Stage 3, Activated SRS Resource Set ID(s) is needed to be added in XnAP RETRIEVE UE CONTEXT FAILURE message.

Observation 4: If LMF makes the activation decision, in case of no anchor relocation,

- For Stage 2, two additional messages for NRPPa Positioning Activation procedure is needed between LMF and Receiving gNB after the Receiving gNB becomes the new Anchor gNB.

- For Stage 3, Activated SRS Resource Set ID(s) is needed to be added in XnAP RETRIEVE UE CONTEXT FAILURE message.

For anchor relocation case,

The activation procedure for Option 1 is as follows,

Under this case,

- For Stage 2, two additional messages for NRPPa Positioning Activation procedure is needed between LMF and Receiving gNB after the Receiving gNB becomes the new Anchor gNB.

Observation 5: If LMF makes the activation decision, in case of anchor relocation,

- For Stage 2, two additional messages for NRPPa Positioning Activation procedure is needed between LMF and Receiving gNB after the Receiving gNB becomes the new Anchor gNB.

Another thing needed to be discussed is how the LMF sends the NRPPa POSITIONING ACTIVATION REQUEST message.

Alternative 1: LMF sends the Activation message to Last Serving gNB after Step 3 (i.e. Step X1 and Y1 in the above Figure).

Alternative 2: LMF sends the Activation messge to Receiving gNB after Step 6 (i.e. Step X2 and Y2 in the above Figure).

Note that in no anchor relocation case, since the UE context is not relocated, the LMF can only send the Activation message to Last Serving gNB after Step 3, which is similar to Alternative 1.

In addition, when the LMF receives the NRPPa POSITIONING INFORMATION UPDATE message at Step 3, the LMF does not know whether the Last Serving gNB will relocate the UE context or not. Therefore, if we follow Alternative 2 in case of anchor relocation, the LMF actually does not know what is the proper time to send the NRPPa POSITIONING ACTIVATION REQUEST message, and does not know which node to send the message to.

As a result, in our understanding, following Alternative 1 is more suitable, which means that no matter there is anchor relocation or not, the LMF will immediately send the NRPPa POSITIONING ACTIVATION REQUEST message to the Last Serving gNB, after the reception of the NRPPa POSITIONING INFORMATION UPDATE message at Step 3.

Proposal 2: The LMF sends the NRPPa POSITIONING ACTIVATION REQUEST message to the Last Serving gNB, after the reception of the NRPPa POSITIONING INFORMATION UPDATE message at Step 3.

As a consequence, even in case of anchor relocation, the Last Serving gNB needs to send SP SRS activation related information in XnAP RETRIEVE UE CONTEXT FAILURE message at Step 4.

Proposal 3: Introduce SP SRS activation related information (e.g. Activated SRS Resource Set ID, spatial relation, etc.) in XnAP RETRIEVE UE CONTEXT RESPONSE/FAILURE message to support SP SRS Activation procedure.

Moreover, in current NRPPa spec, in POSITIONING ACTIVATION RESPONSE message, the activation time (SFN + Slot Number) may be signalled from NG-RAN node to LMF, which is excerpted as follows,

9.1.1.18	POSITIONING ACTIVATION RESPONSE

This message is sent by NG-RAN node to confirm successful UL SRS activation in the UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

As described in the NRPPa spec, the SFN and/or the Slot Number indicates the activation time of SRS transmission by the UE.

Firstly, it should be noted that the activation time related IEs were introduced for RRC Connected UEs. So the gNB-DU/gNB serving the UE is able to obtain the exact activation time when the UE starts the SRS transmission, and send the exact activation time back to the LMF without any ambiguity.

While for the area-specific SRS transmission, in our understanding, the exact activation time for the SRS transmission should be the time when the receiving gNB sends RRCRelease message to the UE (at Step 7); however, even for the periodic area-specific SRS transmission, the LMF actually is unable to obtain the exact activation time for the SRS transmission.

Therefore, if we reuse the Positioning Activation related messages over NRPPa for area-specific semi-persistent SRS transmission, we also need to discuss whether the SFN as well as the Slot Number may be included in the POSITIONING ACTIVATION RESPONSE message; if not, whether we need to capture something in the spec.

Proposal 4: RAN3 is kindly asked to discuss whether SFN and Slot Number IE may be included in the NRPPa POSITIONING ACTIVATION RESPONSE message for area-specific SP SRS activation.

2.2 SP SRS Deactivation procedure

Generally speaking, there are two options for deactivation decision for area-specific SP SRS,

- Option 1: LMF makes the deactivation decision

- Option 2: Anchor gNB makes the deactivation decision

In our understanding, compared to Option 2, it is more proper to follow Option 1 because LMF has more knowledge on the progress of the measurement; while if the anchor gNB makes the deactivation decision, the anchor gNB has to make the decision blindly which is not an appropriate behaviour.

Proposal 5: LMF makes the deactivation decision for area-specific SP SRS.

And the basic call flow for deactivation procedure is as follows (considering the LMF makes the deactivation decision),

It can be seen from the above figure that,

- Before Step 6, the Anchor gNB1 will decide whether to perform anchor relocation. If gNB1 decides to relocate the anchor, it will send XnAP RETRIEVE UE CONTEXT RESPONSE message to gNB2 in Step 6, and after Step 6 the gNB2 will become the new anchor gNB; else if gNB1 decides not to relocate the anchor, it will send XnAP RETRIEVE UE CONTEXT FAILURE message to gNB2 in Step 6.

- No matter which XnAP message is transmitted in Step 6, the information on Deactivated SRS Resource Set ID should be included in the message.

Proposal 6: Introduce SP SRS deactivation related information (e.g. Deactivated SRS Resource Set ID) in XnAP RETRIEVE UE CONTEXT RESPONSE/FAILURE message to support SP SRS Deactivation procedure.

2.3 F1AP impact

It can be observed from above figures that for SP SRS activation/deactivation, the gNB-DU needs to send SP SRS Activation/Deactivation MAC CE to the UE. As a result, the gNB-CU needs to send SP SRS activation/deactivation related information to gNB-DU via F1AP.

Observation 6: The gNB-CU needs to send SP SRS activation/deactivation related information to gNB-DU via F1AP.

In current TS 38.401, when the UE goes from RRC inactive to other RRC states, the following figure is captured,

From the perspective of SP SRS transmission activation/deactivation, it can be observed from the above figure that, after the receiving gNB-CU obtains the RRC Release message (either generated by itself in case of anchor relocation, or obtained from the last serving gNB in case of no anchor relocation), the RRC Release message is sent from receiving gNB-CU to gNB-DU via F1AP DL RRC MESSAGE TRANSFER message.

In order to support SP SRS activation/deactivation which requires the receiving gNB-DU to send SP SRS Activation/Deactivation MAC CE to the UE, the SP SRS activation/deactivation related information should also be included in F1AP DL RRC MESSAGE TRANSFER message.

Proposal 7: Introduce SP SRS activation/deactivation related information (e.g. SRS Resource Set ID, spatial relation, etc.) in F1AP DL RRC MESSAGE TRANSFER message.

Conclusion

In this paper, we provides considerations on SP SRS activation and deactivation. The following observations and proposals are provided,

Activation Procedure

Observation 1: For SP SRS activation and deactivation over NRPPa, the LMF is able to activate and deactivate SRS transmission on a SRS Resource Set basis, which is different from Periodic SRS activation (i.e. the whole periodic SRS configuration including ALL configured SRS Resource Sets are activated for Periodic SRS activation).

Observation 2: It is RAN2 common understanding to reuse the existing SP SRS Activation/Deactivation MAC CE for area-specific SP SRS activation/deactivation.

Observation 3: RAN3 needs to discuss which node decides the Activated/Deactivated SRS Resource Set(s) and how to perform the corresponding coordination at the network side for area-specific SP SRS, also taking split architecture into account.

Proposal 1: LMF makes the activation decision for area-specific SP SRS.

Observation 4: If LMF makes the activation decision, in case of no anchor relocation,

- For Stage 2, two additional messages for NRPPa Positioning Activation procedure is needed between LMF and Receiving gNB after the Receiving gNB becomes the new Anchor gNB.

- For Stage 3, Activated SRS Resource Set ID(s) is needed to be added in XnAP RETRIEVE UE CONTEXT FAILURE message.

Observation 5: If LMF makes the activation decision, in case of anchor relocation,

- For Stage 2, two additional messages for NRPPa Positioning Activation procedure is needed between LMF and Receiving gNB after the Receiving gNB becomes the new Anchor gNB.

Proposal 2: The LMF sends the NRPPa POSITIONING ACTIVATION REQUEST message to the Last Serving gNB, after the reception of the NRPPa POSITIONING INFORMATION UPDATE message at Step 3.

Proposal 3: Introduce SP SRS activation related information (e.g. Activated SRS Resource Set ID, spatial relation, etc.) in XnAP RETRIEVE UE CONTEXT RESPONSE/FAILURE message to support SP SRS Activation procedure.

Proposal 4: RAN3 is kindly asked to discuss whether SFN and Slot Number IE may be included in the NRPPa POSITIONING ACTIVATION RESPONSE message for area-specific SP SRS activation.

Deactivation Procedure

Proposal 5: LMF makes the deactivation decision for area-specific SP SRS.

Proposal 6: Introduce SP SRS deactivation related information (e.g. Deactivated SRS Resource Set ID) in XnAP RETRIEVE UE CONTEXT RESPONSE/FAILURE message to support SP SRS Deactivation procedure.

F1AP Impact

Observation 6: The gNB-CU needs to send SP SRS activation/deactivation related information to gNB-DU via F1AP.

Proposal 7: Introduce SP SRS activation/deactivation related information (e.g. SRS Resource Set ID, spatial relation, etc.) in F1AP DL RRC MESSAGE TRANSFER message.