3GPP TSG-RAN WG3 #129bis                                                                          R3-256664

Prague, Czech, Oct. 13~17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1591 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to positioning activation and deactivation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, CATT, ZTE Corporation, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, the Area-specific SRS Configuration Activation Procedure is supported; however, the activation and deactivation of Semi-Persistent (SP) SRS transmission in still unclear. In fact, enhancement on F1AP is needed in order to support activation and deactivation of SP SRS transmission. More specifically, the receiving gNB-CU needs to inform the SP SRS transmission activation/deactivation related information via F1AP DL RRC MESSAGE TRANSFER in order for the receiving gNB-DU to generate the SP SRS Activation/Deactivation MAC CE sent to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Area-specific Semi-persistent SRS Positioning Information IE in DL RRC MESSAGE TRANSFER message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact from protocol and functional point of view. The impact can be considered isolated because it only impacts the functionality to support NR positioning SP SRS transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functionality of Area-specific SRS configuration activation and deactivation is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2, 9.2.3.2, 9.3.1.xxx, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.305 CR TS 38.423 CRXXX TS 38.455 CRXXX |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

8.4.2	DL RRC Message Transfer

8.4.2.1	General

The purpose of the DL RRC Message Transfer procedure is to transfer an RRC message The procedure uses UE-associated signalling.

8.4.2.2	Successful operation

Figure 8.4.2.2-1: DL RRC Message Transfer procedure

The gNB-CU initiates the procedure by sending a DL RRC MESSAGE TRANSFER message. If a UE-associated logical F1-connection exists, the DL RRC MESSAGE TRANSFER message shall contain the gNB-DU UE F1AP ID IE, which should be used by gNB-DU to lookup the stored UE context.If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established at reception of the DL RRC MESSAGE TRANSFER message.

If the Index to RAT/Frequency Selection Priority IE is included in the DL RRC MESSAGE TRANSFER, the gNB-DU may use it for RRM purposes. If the Additional RRM Policy Index IE is included in the DL RRC MESSAGE TRANSFER, the gNB-DU may use it for RRM purposes.

The DL RRC MESSAGE TRANSFER message shall include, if available, the old gNB-DU UE F1AP ID IE so that the gNB-DU can retrieve the existing UE context in RRC connection reestablishment procedure, as defined in TS 38.401 [4].

The DL RRC MESSAGE TRANSFER message shall include, if SRB duplication is activated, the Execute Duplication IE, so that the gNB-DU can perform CA based duplication or multi-path relay based duplication for the SRB.

If the gNB-DU identifies the UE-associated logical F1-connection by the gNB-DU UE F1AP ID IE in the DL RRC MESSAGE TRANSFER message and the old gNB-DU UE F1AP ID IE is included, it shall release the old gNB-DU UE F1AP ID and the related configurations associated with the old gNB-DU UE F1AP ID.

If the UE Context not retrievable IE set to "true" is included in the DL RRC MESSAGE TRANSFER, the DL RRC MESSAGE TRANSFER may contain the Redirected RRC message IE and use it as specified in TS 38.401 [4].

If the UE Context not retrievable IE set to "true" is included in the DL RRC MESSAGE TRANSFER, the DL RRC MESSAGE TRANSFER may contain either the PLMN Assistance Info for Network Sharing IE or the PLMN Index NR Assistance Info for Network Sharing IE, if available at the gNB-CU and may use it as specified in TS 38.401 [4].

If the DL RRC MESSAGE TRANSFER message contains the New gNB-CU UE F1AP ID IE, the gNB-DU shall, if supported, replace the value received in the gNB-CU UE F1AP ID IE by the value of the New gNB-CU UE F1AP ID and use it for further signalling.

If the DL RRC MESSAGE TRANSFER contains the SRB Mapping Info IE, the gNB-DU shall, if supported, use it for the Remote UE’s SRB0 or SRB1 transfer.

If the DL RRC MESSAGE TRANSFER contains the Area-specific Semi-persistent SRS Positioning Information IE, the gNB-DU shall, if supported, take it into account to activate or deactivate the semi-persistent SRS transmission for area-specific SRS configuration.

Interactions with UE Context Release Request procedure:

If the UE Context not retrievable IE set to "true" is included in the DL RRC MESSAGE TRANSFER, the gNB-DU may trigger the UE Context Release Request procedure, as specified in TS 38.401 [4].

8.4.2.3	Abnormal Conditions

Not applicable.

-------------------------------------------Next change-------------------------------------------

9.2.3.2	DL RRC MESSAGE TRANSFER

This message is sent by the gNB-CU to transfer the layer 3 message to the gNB-DU over the F1 interface.

Direction: gNB-CU gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| old gNB-DU UE F1AP ID | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | YES | reject |
| SRB ID | M |  | 9.3.1.7 |  | YES | reject |
| Execute Duplication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| RRC-Container | M |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| UE Context not retrievable | O |  | ENUMERATED (true, ...) |  | YES | reject |
| Redirected RRC message | O |  | RRC-Container 9.3.1.6 | Includes the UL-CCCH-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| PLMN Assistance Info for Network Sharing | O |  | PLMN Identity 9.3.1.14 | This IE is ignored if the PLMN Index NR Assistance Info for Network Sharing IE is present. | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the Remote UE’s SRB0 or SRB1. | YES | ignore |
| PLMN Index NR Assistance Info for Network Sharing | O |  | INTEGER (1..maxnoofBPLMNsNR) | Index of the PLMN or NPN in SIB1. | YES | ignore |
| Area-specific Semi-persistent SRS Positioning Information | O |  | 9.3.1.xx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

-------------------------------------------Next change-------------------------------------------

9.3.1.xxx	Area-specific Semi-persistent SRS Positioning Information

This IE contains area-specific semi-persistent SRS positioning information to notify the activation or deactivation of semi-persistent SRS transmission for area-specific SRS configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS Transmission Type | M | ENUMERATED (activate, deactivate, …) |  |  |
| SRS Resource Set ID | M | 9.3.1.180 |  |  |
| Spatial Relation Information | O | Spatial Relation Information 9.3.1.181 |  | Applicable only if the SRS Transmission Type IE is set to ‘activate’. |
| Spatial Relation Information per SRS Resource | O | 9.3.1.210 |  | Applicable only if the SRS Transmission Type IE is set to ‘activate’. |

-------------------------------------------Next change-------------------------------------------

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

MobilityInitiation,

PLMNIndexNR,

AreaSpecificSemiPersistentSRSPosInfo

-- Omit unchanged

id-PreconfiguredSRSInformation,

id-MobilityInitiation,

id-PLMNIndexNRAssistanceInfoForNetShar,

id-AreaSpecificSemiPersistentSRSPosInfo,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofMRBsforUE,

maxnoofServingCellMOs

-- Omit unchanged

-- **************************************************************

--

##### -- DL RRC Message Transfer

--

-- **************************************************************

DLRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DLRRCMessageTransferIEs}},

...

}

DLRRCMessageTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-oldgNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE optional	}|

{ ID id-SRBID										CRITICALITY reject	TYPE SRBID											PRESENCE mandatory	}|

{ ID id-ExecuteDuplication							CRITICALITY ignore	TYPE ExecuteDuplication							PRESENCE optional}|

{ ID id-RRCContainer								CRITICALITY reject	TYPE RRCContainer									PRESENCE mandatory	}|

{ ID id-RAT-FrequencyPriorityInformation			CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation		PRESENCE optional	}|

{ ID id-RRCDeliveryStatusRequest					CRITICALITY ignore	TYPE RRCDeliveryStatusRequest					PRESENCE optional }|

{ ID id-UEContextNotRetrievable						CRITICALITY reject	TYPE UEContextNotRetrievable					PRESENCE optional }|

{ ID id-RedirectedRRCmessage						CRITICALITY reject	TYPE OCTET STRING									PRESENCE optional }|

{ ID id-PLMNAssistanceInfoForNetShar				CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-new-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex					CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex				PRESENCE optional }|

{ ID id-SRBMappingInfo								CRITICALITY ignore	TYPE UuRLCChannelID					 				PRESENCE optional }|

{ ID id-PLMNIndexNRAssistanceInfoForNetShar					CRITICALITY ignore	TYPE PLMNIndexNR											PRESENCE optional }|

{ ID id-AreaSpecificSemiPersistentSRSPosInfo		CRITICALITY ignore	TYPE AreaSpecificSemiPersistentSRSPosInfo 	PRESENCE optional },

...

}

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

-- Omit unchanged

AveragingWindow  ::= INTEGER (0..4095, ...)

AreaScope ::= ENUMERATED {true, ...}

AreaSpecificSemiPersistentSRSPosInfo ::= SEQUENCE {

sRSTransmissionType	ENUMERATED{activate, deactivate,...},

sRSResourceSetID		SRSResourceSetID,

spatialRelationInfo		SpatialRelationInfo															OPTIONAL,

spatialRelationPerSRSResource	SpatialRelationPerSRSResource										OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { AreaSpecificSemiPersistentSRSPosInfo-ExtIEs } }	OPTIONAL

}

AreaSpecificSemiPersistentSRSPosInfo-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- Omit unchanged

id-candidatePSCellsToCancel							ProtocolIE-ID ::= 858

id-MobilityInitiation								ProtocolIE-ID ::= 859

id-ValidityAreaSpecificSRSInformationExtended		ProtocolIE-ID ::= 860

id-PLMNIndexNRAssistanceInfoForNetShar						ProtocolIE-ID ::= 861

id-AreaSpecificSemiPersistentSRSPosInfo			ProtocolIE-ID ::= xxx

END

-- ASN1STOP

-------------------------------------------End of changes-------------------------------------------