3GPP TSG-RAN WG3 #129bis                                    R3-256667

# Prague, Czech, Oct. 13~17, 2025

Agenda item:	11.2.2

Source: 	Samsung

Title: 	(TP for SON BLCR to TS 38.401 and TS 38.300) Discussion on MRO enhancement on intra-CU conditional LTM

Document for:	Discussion and decision

Introduction

Rel-20 SON WI has been approved [1], and the work needed to be done in RAN3 is as follows,

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

In this contribution, we discuss the support of MRO on intra-CU conditional LTM.

Discussion

As indicated in the WID, the MRO enhancement on intra-CU Conditional LTM (CLTM) is in the scope.

Based on the latest TS 38.300, the following general description is captured for CLTM,

CLTM cell switch is executed by the UE when L1-based or L3-based LTM cell switch execution conditions are met.

Based on the latest TS 38.401, both intra-DU CLTM and intra-CU inter-DU CLTM cases are supported. In addition, TS 38.401 captures the following description,

If the gNB-CU decides to initiate the L1 event-triggered conditional LTM, it also provides a list of candidate cells to which the L1 event-triggered conditional LTM is applied and requests the gNB-DU to generate the corresponding L1-based execution condition(s).

According to the description above, we can observe that,

- Similar to intra-CU LTM, the gNB-CU provides the candidate cell list for intra-CU CLTM.

- For CLTM, the UE can be configured with L1 execution condition and/or L3 execution condition.

- For CLTM, the L1 execution condition is determined by DU, and the L3 execution condition is determined by CU.

Based on the above observations, the reasons to cause failures can be due to,

- Inappropriate L1 CLTM execution condition generated by DU.

- Inappropriate L3 CLTM execution condition generated by CU.

- Inappropriate candidate cell list provided by CU.

Observation 1: The possible reasons for CLTM failure cases are as follows,

- Inappropriate L1 CLTM execution condition generated by candidate/source DU.

- Inappropriate L3 CLTM execution condition generated by CU.

- Inappropriate candidate cell list provided by CU.

Firstly, in our understanding, similar to intra-CU LTM and/or CHO, the intra-CU CLTM also has three failure cases, i.e. Too Late CLTM execution, Too Early CLTM execution, and CLTM execution to wrong cell.

Proposal 1: Three failure cases are identified for intra-CU CLTM: Too Late CLTM execution, Too Early CLTM execution, and CLTM execution to wrong cell.

After the failure occurs and UE re-establishes/recovers to another cell within the same CU, when the CU receives the RLF report from the UE, the CU performs initial analysis. If the CU identifies the failure is caused by the CU, the CU will do optimization by itself; while if the CU identifies the failure may be caused by the DU (due to inappropriate L1 CLTM execution condition), the CU needs to forward the RLF report to the last serving/source DU.

Proposal 2: In case of failures due to inappropriate configuration on L1 CLTM execution condition, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late CLTM execution, or to the source gNB-DU in case of too early CLTM execution or CLTM execution to wrong cell.

In addition, we need to identify whether additional failure cases for CLTM are needed to be considered.

Recall that in R19, three additional failure cases are identified for intra-CU LTM,

- Case 1: BFR to the same cell shortly after successful LTM cell switch.

- Case 2: RACH-less LTM cell switch to a wrong beam under the same cell.

- Case 3: The TA is outdated.

In our understanding, for Case 1 and Case 2, different from the intra-CU LTM for which the beam is determined by the source DU, for CLTM the beam is selected by the UE. In CLTM, even if the selected beam is reported by the UE to the gNB, the gNB (CU or DU) is unable to perform further network optimisation. As a result, Case 1 and Case 2 seem to be not applied to CLTM.

For Case 3, as captured in the latest TS 38.300 for the signalling procedure for CLTM as follows,

Figure 9.2.3.X.x-1. Signalling procedure for CLTM

It can be observed from the above figure that in Step 6e/6f, the LTM Candidate Timing Advance Command MAC CE is possible to be provided to UE for the purpose of early synchronization for CLTM. So there’s a chance that the UE performs CLTM execution to a candidate cell and finds that the TA provided in the previous MAC CE is outdated.

As a result, at the initial stage we firstly focus on TA outdated case. And whether there’s RAN3 spec impact for this case, and whether other failure cases should also be considered can be further discussed.

Proposal 3: Consider the following failure case for intra-CU CLTM: RACH-less CLTM execution is failed because of the TA is outdated, and the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell.

In order for the gNB-CU/gNB-DU to identify the failure reason as mentioned in Observation 1 as well as P3, the UE may need to include the following information in the RLF report:

- CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure. It is similar to what we have already introduced for CHO and LTM, but RAN2 may further discuss whether to reuse the current codepoint or define a new one.

- L1 execution failure: It indicates the last executed CLTM is an L1 execution before the last detected connection failure. Such information can be used for identifying Too early CLTM execution or CLTM execution to wrong cell.

- L3 execution failure: It indicates the last executed CLTM is an L3 execution before the last detected connection failure. Such information can be used for identifying Too early CLTM execution or CLTM execution to wrong cell.

- Cell ID for which the execution is performed.

- Cell list for which the execution conditions are fulfilled. Such information can be included when the execution conditions for multiple candidate cells are fulfilled (and UE selects one of the candidate cells for CLTM execution by implementation).

- Time from the execution condition met to failure. Such information can be used for distinguishing Too late CLTM execution from Too early CLTM execution/CLTM execution to wrong cell (i.e. the absence of such information will implicitly indicate that a Too late CLTM execution happens).

- Time from adding execution condition (upon receiving RRCReconfiguration message) to failure. Such information can be used for optimising CLTM configuration.

- CLTM Time alignment timer remaining time: It indicates the time from the time starting/restarting CLTM Time alignment timer to the time the CLTM Time alignment timer is expected to be expired. Such information can be used for identifying TA outdated failure case and can be used e.g. by the gNB-CU/gNB-DU to determine a better timing on the triggering of early sync acquisition.

- CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell. Such information can be used for identifying TA outdated failure case and can be used e.g. by the gNB-DU to provide a more proper CLTM RACH-less configuration.

Proposal 4: RAN3 is kindly asked to discuss whether to introduce the following information in the RLF report from UE for CLTM,

- CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure.

- No execution failure: It indicates neither L1 nor L3 execution conditions are fulfilled before the last detected connection failure.

- L1 execution failure: It indicates the last executed CLTM is an L1 execution before the last detected connection failure.

- L3 execution failure: It indicates the last executed CLTM is an L3 execution before the last detected connection failure.

- Cell ID for which the execution is performed.

- Cell list for which the execution conditions are fulfilled.

- Time from the execution condition met to failure.

- Time from adding execution condition (upon receiving RRCReconfiguration message) to failure.

- CLTM Time alignment timer remaining time: It indicates the time from the time starting/restarting CLTM Time alignment timer to the time the CLTM Time alignment timer is expected to be expired.

- CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell.

From the other aspect, we need to identify those non-failure cases for CLTM.

One possible non-failure case could be: A UE is provided with RACH-less configuration and the execution condition is met, however, the RACH-less CLTM execution cannot be triggered because the UE doesn’t get a TA or the acquired TA is outdated, so the UE performs a RACH-based access successfully to the same target cell. Under such case, if the network can optimise the TA configuration, RACH-less access would be successful. As a result, it is suggested to consider such non-failure case in the discussion.

And we are open to discuss whether other non-failure cases are also needed.

Proposal 5: RAN3 is kindly asked to discuss non-failure case for intra-CU CLTM, and start with the following non-failure case: UE cannot perform a RACH-less CLTM execution to a candidate cell because the UE doesn’t get a TA or the acquired TA is outdated, and the UE successfully performs a RACH-based access to the same target cell.

Proposal 6: Agree the TPs and the LS in the Annex.

Conclusion

In this paper, we provide discussion on MRO enhancement on intra-CU conditional LTM. The following proposals are provided,

Observation 1: The possible reasons for CLTM failure cases are as follows,

- Inappropriate L1 CLTM execution condition generated by candidate/source DU.

- Inappropriate L3 CLTM execution condition generated by CU.

- Inappropriate candidate cell list provided by CU.

Proposal 1: Three failure cases are identified for intra-CU CLTM: Too Late CLTM execution, Too Early CLTM execution, and CLTM execution to wrong cell.

Proposal 2: In case of failures due to inappropriate configuration on L1 CLTM execution condition, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late CLTM execution, or to the source gNB-DU in case of too early CLTM execution or CLTM execution to wrong cell.

Proposal 3: Consider the following failure case for intra-CU CLTM: RACH-less CLTM execution is failed because of the TA is outdated, and the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell.

Proposal 4: RAN3 is kindly asked to discuss whether to introduce the following information in the RLF report from UE for CLTM,

- CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure.

- No execution failure: It indicates neither L1 nor L3 execution conditions are fulfilled before the last detected connection failure.

- L1 execution failure: It indicates the last executed CLTM is an L1 execution before the last detected connection failure.

- L3 execution failure: It indicates the last executed CLTM is an L3 execution before the last detected connection failure.

- Cell ID for which the execution is performed.

- Cell list for which the execution conditions are fulfilled.

- Time from the execution condition met to failure.

- Time from adding execution condition (upon receiving RRCReconfiguration message) to failure.

- CLTM Time alignment timer remaining time: It indicates the time from the time starting/restarting CLTM Time alignment timer to the time the CLTM Time alignment timer is expected to be expired.

- CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell.

Proposal 5: RAN3 is kindly asked to discuss non-failure case for intra-CU CLTM, and start with the following non-failure case: UE cannot perform a RACH-less CLTM execution to a candidate cell because the UE doesn’t get a TA or the acquired TA is outdated, and the UE successfully performs a RACH-based access to the same target cell.

Proposal 6: Agree the TPs and the LS in the Annex.

Reference

[1] RP-252560

Annex A: TP to TS 38.401

<< Start of Change >>

7.X	MRO support for LTM

The gNB-CU receives an RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis. In case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late LTM cell switch, or to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell. In case of failures due to inappropriate configuration on L1 CLTM execution condition, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late CLTM execution, or to the source gNB-DU in case of too early CLTM execution or CLTM execution to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt). The target gNB-DU performs initial analysis and sends the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. The target gNB-DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU.

<< End of Change >>

Annex B: TP to TS 38.300

<< Start of Change >>

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM/CLTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM/CLTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM/CLTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM/CLTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM/CLTM cell switch, Too Early LTM/CLTM cell switch and LTM/CLTM cell switch to Wrong Cell.

In case of CHO with candidate SCG(s), the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO with candidate SCG(s), Too Early CHO with candidate SCG and CHO Execution to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM/CLTM or CHO with candidate SCG(s) is configured but the CHO/LTM/CLTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover/LTM/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM/CLTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover/LTM/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM/CLTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM/CLTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM/CLTM execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of Too Late Handover when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may send a Handover Report over Xn to the candidate NG-RAN node(s) controlling the candidate PSCell(s) whose mobility configuration caused the failure.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

<< End of Change >>

Annex C: draft LS to RAN2

Title:	[DRAFT] LS on MRO enhancement on CLTM

Response to:

Release:	Release 20

Work Item:	NR_SON_MDT_Ph5-Core

Source:	Samsung (to be RAN3)

To:	RAN2

Cc:

Contact Person:

Name:	Xingyu HAN

Tel. Number:

E-mail Address:	xingyu.han@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 is discussing the MRO enhancement on conditional LTM, and identifies that the following information is beneficial to be provided by UE in RLF report,

- CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure.

- No execution failure: It indicates neither L1 nor L3 execution conditions are fulfilled before the last detected connection failure.

- L1 execution failure: It indicates the last executed CLTM is an L1 execution before the last detected connection failure.

- L3 execution failure: It indicates the last executed CLTM is an L3 execution before the last detected connection failure.

- Cell ID for which the execution is performed.

- Cell list for which the execution conditions are fulfilled.

- Time from the execution condition met to failure.

- Time from adding execution condition (upon receiving RRCReconfiguration message) to failure.

- CLTM Time alignment timer remaining time: It indicates the time from the time starting/restarting CLTM Time alignment timer to the time the CLTM Time alignment timer is expected to be expired.

- CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell.

2. Actions:

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take the above information into account, and provide feedback if any.

3. Date of Next TSG RAN3 Meetings:

TSG-RAN3 Meeting #130           17-21 November 2025                          Dallas, US

TSG-RAN3 Meeting #131           09-13 February 2026                           Goteborg, SE