3GPP TSG-RAN WG3 #129bis                                    R3-256668

# Prague, Czech, Oct. 13~17, 2025

Agenda item:	14.2

Source: 	Samsung

Title: 	Discussion on Topology 2 for AIoT

Document for:	Discussion and decision

Introduction

Rel-20 AIoT WI has been approved [1], and the work needed to be done in RAN3 is as follows,

- Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2.

- Specify signaling for Topology 2

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection.

  - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

In this contribution, we discuss the support of Topology 2 for AIoT from RAN3 perspective.

Discussion

As indicated in the WID, RAN3 will start the normative work on support of Topology 2 for Ambient IoT.

The first issue needed to discussed is which architecture to use, RRC-based or UP-based.

Recall that both RRC-based architecture and UP-based architecture are captured in TR 38.769 as follows,

6.4.2.1.1	Solution 1: RRC based solution

Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa.

Figure 6.4.2.1.1-1: RRC based solution of Topology 2

6.4.2.1.3	Solution 3: UP based solution

The A-IoT related messages between the AIoTF and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s PDU Session, the A-IoT-enabled gNB handles the A-IoT-enabled UE’s user plane data as legacy, i.e., over NG-U GTP-U tunnels.

Figure 6.4.2.1.3-1: UP based solution of Topology 2

As can be observed from above, from RAN3 perspective, for RRC-based solution, the AIoT related messages are carried by NGAP; while for the UP-based solution, the AIoT related messages are carried via PDU session which is transparent to AIoT RAN node.

A quick observation is that RRC-based solution is more aligned with the current NGAP signalling to support AIoT Topology 1. If we would like to reuse the current NGAP signalling design also for Topology 2 as much as possible, using RRC-based solution is the most straight-forward approach.

Observation 1: RRC-based solution for Topology 2 is more compliant to the current NGAP signalling framework for AIoT.

In addition, it should be noted that since Rel19, it is a common understanding that the AIoT RAN node shall be responsible for AIoT resource allocation. If the UP based solution is followed, the RRC should be used between AIoT RAN node and UE reader, so different protocol stack will be used to convey AIoT related messages and to perform AIoT resource allocation.

Observation 2: If the UP based solution is followed, conveying AIoT related messages and performing AIoT resource allocation will use different protocol stack.

By considering above reasons, we propose to use RRC based solution for Topology 2.

Proposal 1: Use RRC based architecture for Topology 2.

The next objective is to discuss UE reader authorization, more specifically, whether and how the UE reader authorization related information is transferred over NG/Xn/F1 interface.

In our understanding, transferring UE reader authorized information over NGAP and XnAP is similar to transferring V2X/ProSe Services Authorized information, which means that,

- Include UE Reader Authorized information in NGAP INITIAL CONTEXT SETUP REQUEST message, NGAP UE CONTEXT MODIFICATION REQUEST message, NGAP HANDOVER REQUEST message, NGAP PATH SWITCH REQUEST ACKNOWLOEDGE message.

- Include UE Reader Authorized information in XnAP HANDOVER REQUEST message, XnAP RETRIEVE UE CONTEXT RESPONSE message.

On whether to transfer UE reader authorized information over F1AP, in our understanding, the answer depends on whether gNB-DU will perform any AIoT related operation, such as providing lower layer AIoT configuration for UE reader, or performing AIoT resource allocation, etc. Since such discussion is also dependent on RAN2, we’d like to further check this issue when RAN2 has more progress.

Proposal 2: Include UE Reader Authorized information in the following messages: NGAP INITIAL CONTEXT SETUP REQUEST message, NGAP UE CONTEXT MODIFICATION REQUEST message, NGAP HANDOVER REQUEST message, NGAP PATH SWITCH REQUEST ACKNOWLOEDGE message, XnAP HANDOVER REQUEST message, XnAP RETRIEVE UE CONTEXT RESPONSE message. FFS on whether the UE Reader Authorized information is also needed over F1AP.

For UE reader selection, several descriptions have been captured in our TR or TS,

- In the latest TS 38.300, the gNB shall select BS readers if the Requested Reader List is absent.

- In the latest TS 38.300, the gNB shall take the list into account if the Requested Reader List is present.

- In the latest TR 38.769, the sentence stating that in RRC based solution, UE Reader selection may need coordination between AIoT RAN and AIoT CN is captured.

It can be observed from current spec that the gNB has the final say on which BS reader is selected.

Observation 3: In current spec, the gNB can make the final decision on which BS reader is selected.

While for the UE reader, different from the BS reader which is fixed in the location, the UE reader may move so the location of the UE reader can change from time to time. If we reuse similar design logic as BS reader selection, and still allow the gNB to have the final say on which UE reader to be selected, the first thing needed to be checked is whether the gNB is able to obtain the location of the UE reader with enough precision.

- The gNB is able to obtain the location info of a UE by means of MDT, which is also dependent on User consent. So if the UE reader does not consent to perform MDT related measurement, the gNB is unable to obtain the location info of the UE.

- For current UE positioning method (including GNSS method or NR RAT dependent positioning methods), the calculation of the UE location is performed either by UE or LMF for all of the positioning method, for which the UE location info is agnostic to the gNB. So at least the gNB is unable to obtain the UE location info by means of current UE positioning method.

- The gNB is able to obtain the Cell ID, Timing Advance, SS/CSI-RSRP/RSRQ and AoA of a UE by means of E-CID positioning method. Whether the gNB is allowed to use such information to perform location estimate by implementation, and whether the positioning accuracy is good enough is still unclear.

As a result, for UE reader selection, if the Requested Service Area Information contained in the Inventory Request message contains,

- Neither the Requested AIoT Area List nor the Requested Reader List, the gNB shall select all UE readers it serves

- The Requested AIoT Area List, whether the gNB is able to select UE readers within the indicated AIoT Area is unclear

- The Requested Reader List, the gNB shall take the reader list into account, but whether the gNB is able to select UE readers out of the Requested Reader List is unclear

In our understanding, the LMF is able to obtain the UE location info, but whether the LMF always exists in AIoT CN, and whether such UE location info can be used by AIOTF when generating the Requested Service Area Information needs further discussion in SA2.

On the other hand, another open issue we need to discuss is whether the UE reader can move in a pre-planned path. In current spec, the UAV path info can be obtained from UE by the gNB. In out understanding, in some cases e.g. Smart factory, the UE reader can be mounted on unmanned vehicle which follows a fixed path in motion, so it is beneficial to introduce the path info for a UE reader, and if the UE reader path info can be obtained by the gNB, the gNB is also able to decide the UE readers to be used for a service request.

In order to move forward, firstly when neither the Requested AIoT Area List nor the Requested Reader List is contained, the gNB shall select all UE readers it serves, similar to legacy behaviour.

While for other cases, there are two alternatives,

- Alternative 1: RAN3 confirms that the gNB is able to obtain the UE location related info with enough accuracy by implementation. For Alternative 1, UE reader selection is performed by the gNB.

- Alternative 2: RAN3 assumes the AIOTF is able to obtain the UE location related info with potential enhancement in AIoT CN, and sends LS to SA2 to confirm such assumption. For Alternative 2, UE reader selection is performed by the AIoT CN. For Alternative 2, when only the Requested AIoT Area List is provided, the gNB shall select all UE readers served by all the cells within the AIoT Area List.

Proposal 3: RAN3 is kindly asked to discuss whether it is possible that a UE Reader will follow a pre-planned moving path, and whether it is beneficial to adopt the concept of UE reader path info in the spec.

Proposal 4: When neither the Requested AIoT Area List nor the Requested Reader List is contained in Inventory Request message, the gNB shall select all UE readers it serves.

Proposal 5: RAN3 is kindly asked to discuss which alternative to follow,

- Alternative 1: RAN3 confirms that the gNB is able to obtain the UE location related info with enough accuracy by implementation, or by the UE reader path info. For Alternative 1, UE reader to be used is decided by the gNB.

- Alternative 2: RAN3 assumes the AIOTF is able to obtain the UE location related info with potential enhancement in AIoT CN, and sends LS to SA2 to confirm such assumption. For Alternative 2, UE reader to be used is decided by the AIoT CN. For Alternative 2, when only the Requested AIoT Area List is provided, the gNB shall select all UE readers served by all the cells within the AIoT Area List.

For the support of inter-gNB RRC Connected UE reader mobility, in our understanding, whether it is possible that the UE reader continues the service request after it is handed over to a new cell should be firstly decided. If the UE reader will always stop the ongoing service request after handover, no XnAP spec impact is identified so far. However, according to the WID, the objective may have already implied that the UE reader is possible to continue the ongoing service request after handover.

Observation 4: The current WID may indicate that the UE reader is possible to continue the ongoing service request after handover.

If it is possible to continue the ongoing service request after handover, the next question is whether the UE reader will immediately send an AIoT Paging message containing a different Transaction ID after the handover (i.e. the UE reader discards all AIoT device related context stored before the handover), or the UE reader is also possible to keep the same Transaction ID after the handover (i.e. the UE reader is also possible to keep AIoT device related context, and whether to keep or discard the AIoT device related context can be decided and configured by the target gNB). Such discussion is dependent on RAN2 progress. But RAN3 may also trigger the discussion from target gNB perspective.

Proposal 6: RAN3 is kindly asked to discuss whether the target node is able to decide whether to keep or discard the AIoT device related context stored at the UE reader.

Another issue is that whether and how the UE Reader Index is updated.

Recall that in R19, the BS Reader Index is configured to AIoT CN by OAM; while for UE Reader, since the UE Reader mobility may happen, it is no longer proper to also configure UE Reader Index by OAM; instead, the UE Reader Index may change when the UE Reader changes the serving gNB.

As a result, we need to figure out a solution on how to change the UE Reader Index when the UE Reader changes the serving gNB.

Basically there are two options,

- Option 1: The UE Reader Index is allocated by AIoT CN, and the updated UE Reader Index can be contained in existing UE-associated NGAP message, such as NGAP UE CONTEXT MODIFICATION REQUEST message.

- Option 2: The UE Reader Index is allocated by AIoT RAN node, and the updated UE Reader Index can be contained in existing UE-associated NGAP message, such as NGAP PATH SWITCH REQUEST message.

In fact, we do not see strong need to introduce new message to convey the updated UE Reader Index. Actually in our understanding, if there is UE reader, the AIoT CN should contain AMF to handle the mobility of the UE reader; when the mobility happens by UE reader, the AMF which is responsible of Mobility Management anyway needs to update the UE reader mobility related information with AIOTF. So reusing the existing NGAP UE-associated message is the simplest and the most straight-forward way to update the UE Reader Index.

Compared to Option 2, in our opinion, Option 1 is more proper since AIoT CN has more knowledge on UE Reader such as the UE reader location, the UE Reader Index assignment for all UE Readers in the network, so AIoT CN may have a better decision on the UE Reader Index selection. Therefore, we slightly prefer the UE Reader Index is allocated by AIoT CN.

Proposal 7: When inter-gNB mobility happens, the UE Reader Index is allocated and signalled to AIoT RAN node by AIoT CN via existing UE-associated NGAP message.

Conclusion

In this paper, we provide discussion on Topology 2 for Ambient IoT. The following proposals are provided,

Observation 1: RRC-based solution for Topology 2 is more compliant to the current NGAP signalling framework for AIoT.

Observation 2: If the UP based solution is followed, conveying AIoT related messages and performing AIoT resource allocation will use different protocol stack.

Proposal 1: Use RRC based architecture for Topology 2.

Proposal 2: Include UE Reader Authorized information in the following messages: NGAP INITIAL CONTEXT SETUP REQUEST message, NGAP UE CONTEXT MODIFICATION REQUEST message, NGAP HANDOVER REQUEST message, NGAP PATH SWITCH REQUEST ACKNOWLOEDGE message, XnAP HANDOVER REQUEST message, XnAP RETRIEVE UE CONTEXT RESPONSE message. FFS on whether the UE Reader Authorized information is also needed over F1AP.

Observation 3: In current spec, the gNB can make the final decision on which BS reader is selected.

Proposal 3: RAN3 is kindly asked to discuss whether it is possible that a UE Reader will follow a pre-planned moving path, and whether it is beneficial to adopt the concept of UE reader path info in the spec.

Proposal 4: When neither the Requested AIoT Area List nor the Requested Reader List is contained in Inventory Request message, the gNB shall select all UE readers it serves.

Proposal 5: RAN3 is kindly asked to discuss which alternative to follow,

- Alternative 1: RAN3 confirms that the gNB is able to obtain the UE location related info with enough accuracy by implementation, or by the UE reader path info. For Alternative 1, UE reader to be used is decided by the gNB.

- Alternative 2: RAN3 assumes the AIOTF is able to obtain the UE location related info with potential enhancement in AIoT CN, and sends LS to SA2 to confirm such assumption. For Alternative 2, UE reader to be used is decided by the AIoT CN. For Alternative 2, when only the Requested AIoT Area List is provided, the gNB shall select all UE readers served by all the cells within the AIoT Area List.

Observation 4: The current WID may indicate that the UE reader is possible to continue the ongoing service request after handover.

Proposal 6: RAN3 is kindly asked to discuss whether the target node is able to decide whether to keep or discard the AIoT device related context stored at the UE reader.

Proposal 7: When inter-gNB mobility happens, the UE Reader Index is allocated and signalled to AIoT RAN node by AIoT CN via existing UE-associated NGAP message.

Reference

[1] RP-251885