3GPP TSG-RAN WG3 Meeting #129-bis	R3-256677

Prague, Czech Republic, 13th - 17th Oct, 2025

Title: 	(TP for ISAC TR) RAN-CN procedures and signaling for ISAC

Source: 	Huawei

Agenda item:	13.3

Document Type:	pCR

# 1. Introduction

The new study item on Study on Integrated Sensing and Communication (ISAC) for NR was agreed in [1], with the following contents.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution provides our views on the procedures and signalling for gNB-based mono-static for UAV use case, by taking SA2 solutions into account.

# 2. Discussion

## 2.1 SA2 progress on the signalling and procedures

In the past SA2#168, SA2#169 and SA2#170 meeting, SA2 identified overall six key issues to be solved in TR 23.700-14 in [2]. There are two key issues related to the RAN3 procedures and signalling between RAN and CN:

- Key Issue #4: Sensing Data and the Associated Information Collection and Transport

- Key Issue #6: Configuration of Parameters for Sensing Entities

And in [2], there are multiple solutions related to the above two key issues. And their flowcharts are described and given in the Annex for reference in non-exhaustive way.

Observation 1:  The SA2 key issue#4 and #6 focuses on the sensing parameters configuration and the sensing data transport. And multiple solutions were proposed in TR 23.700-14.

## 2.2 The general signalling procedure

### 2.2.1 The interface setup procedure

In our companion paper in [3], it was proposed that basic procedures and signalling discussed in RAN3 should be generic, without any dependent on the network architecture options which would be concluded in SA2.

As observed from [3] and summary in the Annex, in some solutions (#4, #6, #15, #16, #36), the interaction between gNB and SF for sensing capability is proposed, which is assumed that each gNB will register its sensing capability at the (authorizing) SF. For example, the gNB may register its location or supported sensing service area and supported sensing mode and its role (as sensing transmitter, sensing receiver, or both).

From our understanding, there is no need to signal the static gNB sensing related capability parameters (e.g. supported sensing mode and its role) to the sensing CN node. First there is no need to indicate the detailed sensing related capability, e.g., the supporting sensing mode (only gNB mono-static sensing supported) or its role. Second, these static parameters should be managed and configured by the operator (e.g., via OAM) and not changed dynamically.

- No need to indicate the static sensing related capability from the gNB to the sensing CN.

On the other hand, the gNB should setup the interfaces with the sensing CN for transfer application layer information for an NG-C interface instance. As discussed in our companion paper in [3], RAN3 can define and use the Sensing CN as the terminology, which may include the AMF and Sensing function, or Sensing Function only, up to SA2 decision. It is natural to reuse the NG-interface between the gNB and the sensing CN. And the NG interface management message can be reused to indicate e.g., the TAC list, and PLMN list etc, while other information can be further discussed.

- RAN3 to reuse the NG interface management procedures for transfer the application layer information (e.g., the TAC list, PLMN list etc) between the gNB and the sensing CN.

### 2.2.2 The Sensing request and sensing report procedure

After the interface setup procedure, the sensing procedure for the basic gNB-based mono-static sensing service can be summarized as two main phases:

- Phase 1: the sensing CN triggers the sensing procedure towards the gNB

As observed from the SA2 solutions in the Annex captured from [2], it is common thinking that the sensing CN can trigger the sensing request to the gNB, in which the message including the necessary request parameters. This procedure is a class 1 procedure composed of request message and response message. So far in [2] there are different names of the procedure or message, e.g., Sensing Request/Response, Sensing Service Request/Response, Sensing Subscribe Request/Response, Sensing Configuration Request, Sensing Active Request, Sensing Session Establishment, Sensing Task Assignment etc.

From RAN3 point of view, it is proposed that a class 1 Sensing Measurement procedure composed of request/response/failure messages can be defined. The request message can include at least the sensing area info e.g., TAC list, and/or cell list etc. For sensing area, we considering the following options, to be decided together with SA2:

-	Sensing cell. For each gNB, it may serve several Sensing cells;

-	Sensing Tracking Area. It can be across gNB(s) and one or more gNB can belong to the same Sensing Tracking Area. When the gNB establishes the NG setup connection with Sensing CN, the gNB includes the supported Tracking Area.

The example of the Sensing Tracking Area and Sensing cell is given in the following Figure. For the rest information, the details can be further considered.

- Introduce a general class 1 Sensing Measurement procedure (naming can be FFS) containing the request/response/failure messages.

- In the SENSING MEASUREMENT REQUEST message, include at least the sensing area info (e.g., the TAC list, Cell list etc).

Note that in the SA2 TR [3], there are also some procedures for sensing service update and sensing service release, e.g., in solution #37 and solution #39. These can be further addressed when the general signalling procedure is steady.

- Phase 2: the gNB transmits the sensing data

For the phase 2 sensing data transmission, it is noticed that there are two different options on how to transmit the sensing data in [2]:

  - Option 1: the gNB transmits the sensing data via the control plane signaling.

  - Option 2: the gNB transmits the sensing data over GTP-U tunnel.

This issue should be cooperated with SA2 to decide how to transmit the sensing data. From RAN3 point of view, it requires to discuss and compare these solutions in terms of different comparison metrics, e.g., including data size limitation, latency and complexity etc when the sensing measurement results format is determined. In general, we consider that Option2 UP solution is advantageous to transmit the sensing data, considering that the user plane tunnel is more suitable for transmitting large volumes of sensing data and future-proof.

- Introduce the Sensing measurement report from the gNB to Sensing CN. RAN3 to discuss and compare the CP and UP solutions in terms of data size limitation, latency and gNB complexity etc. Cooperation with SA2 is needed.

About the exact sensing measurement results, the previous RAN1-121 meeting studied the following options:

### [FL1][H] Proposal 7.1-1-rev3 - The following measurements can be further studied for NR ISAC. - Spectrum of delay, and/or Doppler, and/or angle, i.e., the powers or the amplitudes/phases are reported for at least a range of samples of delay, and/or Doppler, and/or angle - Intermediate sensing results per detected path - Delay, Doppler, angle, and power per path - Intermediate sensing results per detected point - Option 1: Position, velocity, and power per point - Option 2: Range, velocity, angle, and power per point - Final sensing results per detected object/target - Option 1: Position and velocity - Option 2: Range, velocity, and angle - Note: Trajectory report is separate discussion - SINR, RSRP from perspective of sensing

From RAN3’s perspective, the exact sensing measurements are pending to RAN1 evaluation and quantization. Generally, we consider that the intermediate sensing result per detected point (i.e., cloud point) is very advantageous especially compared with the final sensing results due to the following reasons:

- relatively lower transmission overhead while still effectively capturing the original characteristics of the measurements;

- allows the sensing CN to have precise sensing calculation when considering more information sources (e.g., non-3GPP sensing data, the future UE-provided data), which is not possible when the gNB provides final sensing result directly;

- No significant gNB upgradation to support future sensing types in a large quantitively way while sensing CN upgradation would be much simpler.

And the point definition in TS 23.032 can be taken as starting point, which has already captured the format of various measurements including range, direction, velocity and uncertainty. Typically, the following format in TS 23.032 can be treated as starting point:

- 5.10	Range and Direction

- 8.4a	Relative Velocity with Uncertainty

- In the Sensing Results Reporting procedure, the cloud points can be reported as the sensing measurements referring to TS 23.032, while the final evaluation on the sensing measurement, and quantization is up to RAN1.

# 3. Conclusion

Based on the discussion in this paper, we make the following observations and proposals.

SA2 progress on the signalling and procedures

Observation 1:  The SA2 key issue#4 and #6 focuses on the sensing parameters configuration and the sensing data transport. And multiple solutions were proposed in TR 23.700-14.

The general signalling procedure

- No need to indicate the static sensing related capability from the gNB to the sensing CN.

- RAN3 to reuse the NG interface management procedures for transfer the application layer information (e.g., the TAC list, PLMN list etc) between the gNB and the sensing CN.

- Introduce a general class 1 Sensing Measurement procedure (naming can be FFS) containing the request/response/failure messages.

- In the SENSING MEASUREMENT REQUEST message, include at least the sensing area info (e.g., the TAC list, Cell list etc).

- Introduce the Sensing measurement report from the gNB to Sensing CN. RAN3 to discuss and compare the CP and UP solutions in terms of data size limitation, latency and gNB complexity etc. Cooperation with SA2 is needed.

- In the Sensing Results Reporting procedure, the cloud points can be reported as the sensing measurements referring to TS 23.032, while the final evaluation on the sensing measurement, and quantization is up to RAN1.

The Annex 5 provides the pCR on the RAN-CN procedures and signaling for the sensing.

# 4. Reference

- RP-252819 New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- TR 23.700-14 Study on Integrated Sensing and Communication

- R3-256676	(TP for ISAC TR) Network architecture for ISAC	Huawei

# Annex – pCR to ISAC TR

CHANGES START

# 8	RAN-CN Procedures and Signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

### 8.1	General

The purpose of this clause is to provide the signalling procedures between the RAN and CN for gNB-based monostatic sensing.

### 8.2	Sensing measurement initiation and report

Figure 8.2-1 illustrates the call flow of sensing service initiation between CN and RAN.

Figure 8.2-1: Sensing service initiation procedures

- The gNB establishes the NG interface with the Sensing CN, containing the TAC list etc

- The Sensing CN send sends the Sensing Measurement Request message including the requested sensing area (e.g., TAC list, cell list) to the gNB.

Editor’s note1: In step 2, the details included in the sensing request message are to be defined.

- The gNB sends the Sensing Service Response message to the Sensing CN if the gNB accepts the request.

- The Sensing CN/gNB trigger to establish the tunnel for sensing data transmission.

- The gNB performs the Sensing measurement and sends the sensing results to the sensing CN.

Editor’s note2: In step 5, the details e.g., the CP/UP report and the sensing results are to be defined.

# Annex - Solutions in [2] related to the signlaing and procedures between the gNB and CN

| Solution | Description | Figure |
| --- | --- | --- |
| #1 | - 2. The SF sends a trigger for the Sensing Entity to set up the direct interface. - 4. The SF sends a Sensing Service request to the Sensing Entity. - 6. The Sensing Entity(ies) uses the direct connection to provide the sensing data/result to the SF. - 7. The Sensing Entity(ies) may use the direct connection to provide additional reports based on the configurations received in step 4, e.g. for a periodical sensing operations or event triggered sensing operations. | Figure 6.1.2.1-1 |
| #3 | - 9. The SF determines the information for the subsequent Sensing measurement and sends the sensing service request message including the configuration parameters to the Sensing Entity. - 10. The Sensing Entity may trigger the establishment of the path for 3GPP sensing data reporting. - 11. The Sensing Entity performs the Sensing measurement and obtains 3GPP sensing data. - 12. The 3GPP sensing data is transmitted to the SF. |  |
| #4 | - 0. Sensing entities are already registered to SeMF with their sensing capabilities. - 6. The SeMF configures the selected sensing entities with necessary parameters for the sensing request. - 7. The SeMF requests and collects sensing data from the configured sensing entities. Transport for the sensing data collection may occur over CP (N1/N2) or UP (N3). |  |
| #5 | - 9/10. The SF invokes the Namf_Sensing_Subscribe request service operation toward the selected AMF(s) and includes the information for the event reporting and the sensing-related parameters. - 11. The AMF performs the discovery and selection of sensing entity/entities. Alternatively, the SF may perform discovery and selection of sensing entity/entities. - 12. The AMF transmits the Sensing Subscribe request message - 13. The sensing entity/entities carry out the sensing measurement and collect the sensing data. - 14. The sensing data is transmitted to the AMF. - 15. The AMF forwards the sensing data to the SF. |  |
| #6 | - 5. When receiving a sensing operation request, the (distributed) sensing NF for the target sensing service area may perform discovery and selection procedure - 6. The (distributed) sensing NF coordinates the sensing operation with the selected sensing entities. - 7. Sensing entities perform sensing operations as coordinated by the (distributed) Sensing NF. - 8. Sensing entities may provide the collected sensing data to the (distributed) Sensing NF (via signalling message or data connection). |  |
| #7 | - 6. The sensing entities perform the sensing as configured. A gNB may collect sensing data from UEs. UEs may send sensing data to the SSMF via the AMF. . - 7. The gNB (and any UE sensors) perform the instructed sensing, then encapsulate measurements in NGAP SensingDataReport messages sent to the AMF. |  |
| #15 | - 7. SF sends Activate to activate all needed Sensing Entities that will be included for the sensing service. This may also include deactivation or reconfiguration of already active Sensing Entities. - 8. Sensing Entity perform the requested configuration and starts the requested sensing service. |  |
|  | - 1. Sensing Entities sends Register and include supported capabilities to the SF |  |
| #16 | - 2. The SE sends sensing entity association request to the SF, which includes SE ID, sensing capability information, supported sensing service area, supported sensing service type. - 4. The SF sends sensing entity association response to the SE. - 8. The SF sends a Sensing request message to the selected SE including the sensing service ID, sensing area, object description, sensing KPIs, target reporting information (e.g. CP or UP reporting and the target address) for the sensing data. |  |
| #18 | - 3. the Sensing Entity establish data connection with the Sensing Function according to the address of the Sensing Function. - 4. The Sensing Entity sends an acknowledgement to Sensing Function to indicate a success of data connection establishment for sensing service. - 5. messages are transferred between Sensing Entity and Sensing Function for sensing data and the associated information transport. |  |
| #20 | - 6. The SCF performs the sensing configuration parameters provisioning on sensing entity(s). |  |
| #21 | - 4. The SF sends DL Service Message Transfer to the selected NG-RAN. The message includes correlation ID and sensing request message. The correlation ID is allocated by SF to identify the sensing service request received in step 0. - 5. The NG-RAN sends UL Service Message Transfer to SF. The message includes correlation ID and sensing response. The correlation ID is received in step 4. - 7. [Conditional] Based on the sensing request, the NG-RAN sends UL Service Message Transfer to SF periodically. The message includes correlation ID and sensing report. The correlation ID is received in step 4. |  |
| #22 | - 2. The Sensing Function sends the Sensing service request to the selected gNB. - 3. The Sensing Entity sends a Sensing service response. - 4. The Sensing Entity performs sensing service operations to obtain the sensing data. - 5. the Sensing Entity sends the sensing data and the associated information to the Sensing Function via the established data tunnel. |  |
| #24 | - 7. A data connection is established between the SE(s) and SDCF for sensing data transfer. - 8. The Sensing Entity(s) performs the sensing operations according to the configurations received in step 5. The sensing data generated is reported to the SDCF. |  |
| #25 | - 3. The SCF sends direct sensing connection establishment information to the selected NG-RAN node, for sensing data transmission. - 4. Sensing data and related information are transmitted between the sensing entity and the sensing function over the direct gNB-SPF connection. The SPF generates sensing results and sends them to the SCF. |  |
| #26 | - 6. The SCF sends the sensing service request and the corresponding configuration information (e.g. the IP address of the SPF, the Sensing Task ID) to the seleted gNB/gNBs via a new interface defined between the SCF and the gNB. - 7. The gNB/gNBs establish the data reporting tunnel with the SPF. - 8. Based on the received sensing service request, the gNB/gNBs perform the sensing measurement to collect the sensing data. |  |
| #27 | For the On Demand Sensing scenario: - 4. The SF then determines the appropriate sensing method, discovers and selects the corresponding sensing entities (i.e. sensing Tx, Rx) and configures them to start sensing session and report the measurement data. For the periodic or event-based Sensing scenario: - 4. The SF then determines the appropriate sensing method, discovers and selects the corresponding sensing entities (i.e. sensing Tx, Rx) and configures them for sensing operation. - 6. At the configured periodicity or at the detection of the configured event(s), sensing operation is performed, the sensing data measurements are collected and reported to SF. |  |
| #28 | - 5. The SnCF sends the sensing service request to the selected NG-RAN node(s). Inside the request, the SnCF includes, for instance, Target Sensing Area information, type of sensing to perform, relevant parameters from the AF request in Step 1, and other parameters derived by the SnCF for the authorized sensing service. - 8. The NG-RAN node(s) provides the sensing data (i.e., sensing measurements processed by the NG-RAN) to the SnCF. |  |
| #30 | - 4. Sensing Configuration / Trigger: The Sensing configures and triggers the selected sensing entities according to the subscription parameters. (as per KI#3 and KI#6) - 5. Sensing Notification / Data Collection: The sensing entities perform the configured sensing operations and provide measurement data to the Sensing Function. (as per KI#4) |  |
| #33 | - 6. When receiving a sensing operation request with a target sensing service area, a (distributed) sensing NF for the target sensing service area may perform discovery and selection procedure. The (distributed) sensing NF selects gNode B as sensing entity for periodic sensing operation with the requested period. - 9. The gNode B may provide the collected sensing measurement data with a sensing operation ID and assistance information to the sensing NF (via signaling message or data connection). |  |
| #34 | - 3. The SF send the sensing request to sensing entity with the configuration parameters. The parameters includes two parts: Service parameters from AF Parameters created by SF to describe the sensing target object characteristics - 4. The sensing entity sends the response to SF. - 5. The sensing entity performs the sensing measurement. - 5a. The Sensing Entity establishes the connection with SF to transmit the sensing data. - 6. The sensing entity sends the sensing data to SF. If Control and Processing Split is applied, the sensing data is sent to SPF. If Control and Processing Split is not applied, the sensing data is sent to SF. |  |
| #36 | - 1. Sensing RAN node sends sensing association request to the SF - 2. SF sends sensing association response message to the sensing RAN node - 4. SF triggers sensing task assignment procedure towards the selected sensing RAN node. - 7. The sensing RAN node sends notification to the SF - 9. SF sends sensing task ID and sensing pause/revoke indication to the sensing RAN node. |  |