3GPP TSG-RAN WG3 Meeting #129-bis	R3-256678

Prague, Czech Republic, 13th - 17th Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0197 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to positioning activation and deactivation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Samsung, CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, the Area-specific SRS Configuration Activation Procedure is supported; however, the activation and deactivation of Semi-Persistent (SP) SRS transmission in still unclear. Different from the activation of periodic area-specific SRS configuration for which the gNB activates the SRS transmission, it is more suitable for the LMF to activate the SP area-specific SRS transmission no matter whether the context relocation is performed or not. Therefore, there is need to specify the detailed usage for the SRS Transmission Status IE in POSITIONING INFORMATION UPDATE message for the SP SRS activation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the procedure texts for the SRS Transmission Status IE in POSITIONING INFORMATION UPDATE message for SP SRS activation. - Add the interactions with the Positioning Activation procedure for Positioning Information Update. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact from protocol and functional point of view. The impact can be considered isolated because it only impacts the NR positioning SP SRS activation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of Area-specific SP SRS configuration activation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.305 draftCR TS 38.423 CR 1527 TS 38.473 CR 1591 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

### 8.2.7	Positioning Information Update

#### 8.2.7.1	General

The Positioning Information Update procedure is initiated by the NG-RAN node to indicate to the LMF that a change has occurred in the SRS configuration or in the UE Tx TEG association. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: Positioning Information Update procedure, successful operation

The NG-RAN node initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the LMF. If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

If the UE Tx TEG Association List IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider it as the UE Tx TEG association for the SRS resources that have changed their TEG association during the latest reporting interval.

If the SRS Transmission Status IE is included in the POSITIONING INFORMATION UPDATE message and set to "stopped", the LMF shall consider that the SRS transmission has stopped. If the SRS Transmission Status IE is set to "area-specific SRS activated", the LMF shall consider the periodic area-specific (pre-)configured SRS is activated or the semi-persistent area-specific (pre-)configured SRS is requested to be activated in the current serving cell.

If the Remote UE Status IE is included in the POSITIONING INFORMATION UPDATE message and set to "L2 U2N Remote", the LMF shall consider that the UE is operating as a Layer 2 UE-to-Network Remote UE. If the Remote UE Status IE is set to "no", the LMF shall consider that the UE is not operating as a Layer 2 UE-to-Network Remote UE..

If the New Cell Identity IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider that as the new cell information of the UE.

Interaction with the Positioning Activation procedure:

If the SRS Transmission Status IE included in the POSITIONING INFORMATION UPDATE message is set to "area-specific SRS activated" for the semi-persistent area-specific (pre-)configured SRS, the LMF shall, if supported, take it into account for sending the POSITIONING ACTIVATION REQUEST message.

#### 8.2.7.3	Unsuccessful Operation

Not Applicable.

#### 8.2.7.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

CHANGES END