3GPP TSG-RAN WG3 Meeting #129-bis	R3-256685

Prague, Czech Republic, 13th–17th October, 2025

Title: 	Discussion on Network Architecture for NR ISAC

Source: 	Tejas Networks

Agenda item:	13.2

Document Type:	Discussion

# Introduction

A new study on integrated sensing and communications (ISAC) has been approved in RAN #108 [1], where the objectives of the study for 5GA-based ISAC with UAV use case are as follows:

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this contribution, we discuss the network architecture for supporting Integrated Sensing and Communication (ISAC) for NR

# Discussion

While gNB-based sensing has been agreed as the scope of Rel-20, reflecting consensus within the work item to focus on sensing functionalities anchored at the gNB. Furthermore, gNB-based sensing can be categorized into two distinct methods based on the location and extent of sensing result processing. Sensing result format depends on whether complete processing is performed at gNB (gNB based sensing) or partially processing is performed (gNB assisted sensing).

Method 1 – gNB-based sensing (Complete processing at the gNB): In this method, all sensing-related signal processing, feature extraction, and result generation are performed entirely at the gNB.

Method 2 – gNB-assisted sensing (Partial processing at the gNB): Here, the gNB performs initial signal processing and feature extraction, while further processing or interpretation of sensing results may be offloaded to sensing function in core network.

| gNB-based Sensing | gNB-assisted Sensing |
| --- | --- |
| Figure 1 Sensing result report for gNB-based sensing Figure 1 Sensing result report for gNB-based sensing |  |
|  | Figure 2 Sensing result report for gNB-assisted sensing Figure 2 Sensing result report for gNB-assisted sensing |

Both these methods report different information as shown in Figure 1 and Figure 2  Many of these parameters are reports in existing standards for different features such as positioning. The reporting resolution of these parameters in existing standards for positioning in illustrated in Table 1. This information can be used to compare the reporting overhead of each of the gNB based sensing resolution. The relative comparison of both the sensing methods is detailed in Table 2.

| Parameters | Reporting resolution | Reporting Overhead per measurement |
| --- | --- | --- |
| Target location | As per reporting resolution of TS 38.355 and TS 38.445. | 60 bits + 8 bits (uncertainty) |
| Target speed | 1 m/s | 10 bits |
| Target classification accuracy | 0.1 | 4 bits |
| Link state information | 0.1 | 4 bits |
| CIR/PDP/DP/CFR | 32/64/128/256 | 72 bits to 512 bits |
| Timing information | As per reporting resolution of TS 38.355 and TS 38.445. | 24 bits |
| Angle information | 0.2 degree | 9 bits |
| Power Information | As per reporting resolution of TS 38.355 and TS 38.445. | 8 bits |
| Velocity Information | 1 m/s | 10 bits |

Table 1  Sensing parameters reporting resolution

Table 2 compares overhead, latency, data rate requirements and multi-TRP measurement fusion between Method 1 and 2.

|  |  | gNB-based Sensing | gNB-assisted Sensing |
| --- | --- | --- | --- |
| Overhead (measurements only) |  | Intermediate measurements: Low (conservative: 102 ×Nmeas bit per report) where Nmeas denotes the number of potential targets reported. Direct sensing measurements: (74 to 86) ×Nmeas bit per report. | High (72-512 bit per antenna port per resource per resource set) |
| Backhaul Requirements | Data-rate | Low | High |
|  | latency | Medium | Low |
| Multi-TRP measurement Fusion |  | Low (gNB uses only its own measurements for sensing) | High (SF can collate measurements from multi-gNBs) |
| Sensing Latency |  | Low | High |
| Sensing Accuracy |  | Relatively low | High |

Table 2 gNB based sensing vs gNB assisted sensing

Observation 1: gNB-based sensing has less overhead in terms of data rate so control plane can be used to send sensing result to sensing function, gNB-assisted sensing provides compatibility with more sensing use cases and algorithms as Sensing Function can decide on the algorithms and sensing multi-TRP measurement fusion when required.

Proposal 1: The study should focus on supporting both the methods (gNB-based and gNB-assisted) to accommodate different types of sensing use cases

In general, there are two potential ways to transport sensing result including sensing information, measurement id, resource Id, resource set id etc., using control plane or using data plane. The transport mechanism depends on sensing result size, frequency of the report etc.

Observation 2: gNB-assisted sensing will require high data rate to send raw sensing data to sensing function and may require a data plane between RAN and the SF.

Proposal 2: Transport of sensing result via both control plane and data plane shall be part of the study

The below figures show the architectures in 5GS applicable to ISAC for gNB-based sensing.

## Proposed architecture 1:

The Sensing Function (SF) receives a request for sensing from a third party. The SF then sends a sensing service request to an AMF. The AMF forwards the sensing service request towards gNB(s) via NGAP. The gNB process the sensing request, performs sensing and reports sensing result to the SF in control plane (via AMF).

Architecture 1 supports sensing in RAN with changes to control plane alone. This allows SF to transparently connect to RAN nodes via AMF using a new NR Sensing protocol (NRSP). The sensing configuration and sensing result are exchanged over NRSP.

Figure 3 Sensing overall reference architecture -I for NG-RAN

Figure 4 Protocol Layering for SF to NG-RAN Signaling

## Proposed architecture 2:

In this architecture, gNB sends sensing result to SF in data plane either via UPF or directly to SF bypassing UPF.

Architecture 2 provides option to send sensing result via data plane. The sensing configuration is exchanged over control plane via NRSP. When sensing result is large (in the case of gNB assisted sensing) or frequent, sending sensing result over data plane is recommended.

Figure 5 Sensing overall reference architecture -II for NG-RAN

Proposal 3-1: In case of Architecture 1 and Architecture 2, NG interface is reused, RAN3 need to enhance NGAP specification to support new sensing functionalities, procedures and messages.

Proposal 3-2: In case of Architecture 2, a new data transport mechanism for sending sensing result from gNB to SF needs to be studied

In case of Architecture 1 & 2, NGAP is terminated in the gNB and in the AMF; NRSP is terminated in the gNB and in the SF. A separate transport mechanism to relay NRSP messages between the AMF and the SF will be needed; such mechanism is out of RAN3 scope.

- Proposal 4: A separate transport mechanism (out of RAN3 scope) is needed to relay NRSP messages between the AMF and the SF.

## Proposed architecture 3:

Figure 6 Sensing overall reference architecture -III for NG-RAN

In Architecture 3, gNB connects to Sensing Function with a new direct interface dedicated for sensing and without involving AMF and UPF/SMF. Along the initial UAV use case, many more use cases will be considered for ISAC. As number of use cases increases, the sensing traffic between RAN and the SF is expected to be significant. A new interface is proposed to reduce signaling latency and enable more efficient sensing message interactions between RAN and the SF.

Proposal 5: The new interface may only have the control protocol stack or both control plane and data plane between NG-RAN node and the Sensing function depending on the sensing use cases.

In case of Alternative 3, RAN connecting to the new SF directly, RAN3 may need to define a new interface between the RAN node and the SF. If a new interface is introduced, considering that the new protocol is used for context management and signalling transmission over new interface for sensing, the protocol stack of XXAP (name FFS) can use the same protocol stack as NG-C, i.e., including XXAP/SCTP/IP/Date link layer/Physical layer. Need for a direct data plane between RAN and the SF shall be part of study.

Proposal 6: In case of Architecture 3, RAN3 may need to introduce a new interface (name FFS), and the new interface should use protocol stack XXAP/SCTP/IP as other RAN3 control plane interfaces and GTP/UPDP/IP for data plane

Proposal 7: The study should focus on two baseline ISAC architectures, the first furnishing sensing functionality via control plane (either Architecture 1 or 3), and the second striving for high data rate sensing requirements using data plane (either Architecture 2 or 3).

In case of split gNB architecture, a gNB-DU will include TRP and sensing functionality. gNB-CU will pass the sensing configuration received from SF to gNB-DU via F1AP. gNB-DU will perform the sensing and generates sensing result and sends it to gNB-CU via F1AP. As discussed above, a new data plane may be needed between gNB-CU and gNB-DU based on sensing result size, frequency etc.

Proposal 8: In case of split gNB architecture, F1AP interface needs to be enhanced to exchange sensing messages between gNB-CU and gNB-DU

# Conclusions

Observation 1: gNB-based sensing has less overhead in terms of data rate so control plane can be used to send sensing result to sensing function, gNB-assisted sensing provides compatibility with more sensing use cases and algorithms as Sensing Function can decide on the algorithms and sensing data fusion when required.

Proposal 1: The study should focus on supporting both the methods (gNB-based and gNB-assisted) to accommodate different types of sensing use cases

Observation 2: gNB-assisted sensing will require high data rate to send sensing result to sensing function and may require a data plane between RAN and the SF.

Proposal 2: Transport of sensing result via both control plane and data plane shall be part of the study

Proposal 3-1: In case of Architecture 1 and Architecture 2, NG interface is reused, RAN3 need to enhance NGAP specification to support new sensing functionalities, procedures and messages.

Proposal 3-2: In case of Architecture 2, a new data transport mechanism for sending sensing result from gNB to SF needs to be studied

- Proposal 4: A separate transport mechanism (out of RAN3 scope) is needed to relay NRSP messages between the AMF and the SF.

Proposal 5: The new interface may only have the control protocol stack or both control plane and user plane between NG-RAN node and the Sensing function depending on the sensing use cases.

Proposal 6: In case of Architecture 3, RAN3 may need to introduce a new interface (name FFS), and the new interface should use protocol stack XXAP/SCTP/IP as other RAN3 control plane interfaces and GTP/UPDP/IP for data plane

Proposal 7: The study should focus on two baseline ISAC architectures, the first furnishing sensing functionality via control plane (either Architecture 1 or 3), and the second striving for high data rate sensing requirements using data plane (either Architecture 2 or 3).

Proposal 8: In case of split gNB architecture, F1AP interface needs to be enhanced to exchange sensing messages between gNB-CU and gNB-DU

# References

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR

- TR 22.837: Study on Integrated Sensing and Communication

- TS 22.137: Integrated Sensing and Communication

- TS 38.355: NR; Sidelink Positioning Protocol (SLPP)

- TS 38.455: NG-RAN; NR Positioning Protocol A (NRPPa)