3GPP TSG-RAN WG3 Meeting #129bis	R3-256689

# Prague, Czech, 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0178 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the reference of Average Packet Loss |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the semantic descriptions of the Average Packet Loss DL IE and Average Packet Loss UL IE in the Slice Based UE Performance, there is no clear reference for the DL PDCP SDU Drop Rate or UL PDCP SDU Loss Rate, because there was no definition for them in TS28.558 or other SA5 specs. According to the LS R3-256518, There is no problem for RAN3 to refer to TS 28.558 for the definition for DL PDCP SDU Drop Rate or UL PDCP SDU Loss Rate in TS28.558. The semantic descriptions of the associated IEs can be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference to TS 28.558 for the following two IEs in the UE Performance IE: - Average Packet Loss DL IE - Average Packet Loss UL IE Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no clear reference for the definition of Average Packet Loss DL IE and Average Packet Loss UL IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.149 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1529 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.3.1.149	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.3.1.150 |  |
| Average Packet Loss DL | O |  | INTEGER(0..1000000, …) | Corresponds to DL PDCP SDU Drop Rate as specified in TS 28.558 [37], clause 6.3.1.x. |
| Average Packet Loss UL | O |  | INTEGER(0..1000000, …) | Corresponds to UL PDCP SDU Loss Rate as specified in TS 28.558 [37], clause 6.3.1.y. |

<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>