3GPP TSG-RAN WG3 Meeting #129bis	R3-256693

# Prague, Czech, 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0199 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the ASN.1 of Positioning Data Information for data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Positioning Data Information IE in the tabular includes a TRP Measurement List which is composed of a seuqnce of TRP Measurement Item. However, in the ASN.1, the PositioningDataInformation is directly composed of the sequence of TRP-Measurement-Item, which makes the ASN.1 not aligned with the taublar, and also makes the Positioning Data Information IE not extendable. Besides, it is better to define the TRP Measurement List IE as optional, as its upper level ‘Postioning Data Information’ IE is already optional in the corresponding messages containing this IE.s |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change the ASN.1 of the Postioning Data Informtion IE to align with the tabular. 2. Change the presence of TRP Measurement List in Positioning Data Information IE to optional . Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASN.1 of Positioning Data Information IE is not aligned with the tabular, and the TRP Measurement List IE is not extenable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.107	Positioning Data Information

This information element contains positioning information for data collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP Measurement List |  | 1 |  | Indicates the area of interest. |
| >TRP Measurement Item |  | 1..<maxnoofMeasTRPs> |  |  |
| >>TRP ID | M |  | 9.2.24 |  |
| >>UL RTOA | O |  | 9.2.39 |  |
| >>gNB Rx-Tx Time Difference | O |  | 9.2.40 |  |
| >>LoS/NLoS Information | O |  | 9.2.77 |  |
| >>Time Stamp | M |  | 9.2.42 |  |
| >>Measurement Quality | O |  | 9.2.43 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<Next changes>>>>>>>>>>>>>>>>>>>>>>>>

9.3.5	Information Element definitions

PositioningDataInformation ::= SEQUENCE {

tRP-Measurement-List		TRP-Measurement-List

iE-Extensions				ProtocolExtensionContainer { { TRP-Measurement-List-ExtIEs} } OPTIONAL,

...

}

TRP-Measurement-List-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-Measurement-List ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-Measurement-Item

TRP-Measurement-Item ::= SEQUENCE {

tRP-ID				TRP-ID,

uL-RTOA				UL-RTOAMeasurement	OPTIONAL,

gNB-RxTxTimeDiff	GNB-RxTxTimeDiff	OPTIONAL,

los-nlos-info		LoS-NLoSInformation	OPTIONAL,

timeStamp			TimeStamp,

measurementQuality	TrpMeasurementQuality	OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { TRP-Measurement-Item-ExtIEs } } OPTIONAL,

...

}

TRP-Measurement-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>