3GPP TSG-RAN WG3 Meeting #129bis		R3-256697

Prague, Czech Republic, 13th – 17th Oct., 2025

Source:	vivo

Title:	Discussion on RAN2 LS on per-UE enable disable LP-WUS by NAS

Agenda Item:	18.2

Document for:	Discussion

- Introduction

In this contribution, we discuss RAN2 LS on enabling/disabling LP-WUS per UE with NAS signalling in [1].

- Discussion

In RAN2#130 meeting, RAN2 discussed the enabling/disabling LP-WUS per UE method and send an LS to RAN3:

RAN2 discussed to support enabling/disabling LP-WUS for a UE with NAS signalling and made the following agreements: - RAN2 assumes NAS signalling is introduced to support enabling/disabling LP-WUS per UE. - RAN2 assumes that NAS signalling needs to be extended to enable/disable LP-WUS per UE in IDLE and INACTIVE. Detail signalling is up to SA2 and CT1. - RAN2 assumes that CN needs to inform gNB that LP-WUS is enabled/disabled per UE in IDLE and INACTIVE. Details signalling is up to SA2, CT1 and RAN3. - RAN2 assumes that without such NAS signalling, UE is allowed to use LP-WUS in IDLE and INACTIVE.

We believe the above enabling/disabling LP-WUS method will have RAN3 impacts based on the agreement that RAN2 assumes that CN needs to inform gNB that LP-WUS is enabled/disabled per UE in IDLE and INACTIVE. In this paper, we will discuss the impacts on NGAP interface, XnAP interface and F1AP interface.

NGAP interface enhancement

In current LP-WUS CR for NGAP, the LP-WUSPS Assistance Information which carries the CN assigned LP-WUS subgroup ID is included in NG Paging message to inform gNB to send the corresponding LP-WUS in its cell for RRC_IDLE UE. Besides, the LP-WUSPS Assistance Information is also included in Core Network Assistance Information for RRC_INACTIVE IE during the initial context setup procedure, UE context modification procedure, Handover preparation procedure and Path switch procedure to ensure the gNB could send the corresponding LP-WUS with CN assigned subgroup ID for RRC_INACTIVE UE.

Observation 1: Once the received NG Paging message includes LP-WUSPS Assistance Information, gNB may send LP-WUS for this RRC_IDLE UE.

Observation 2: Once the received Core Network Assistance Information for RRC_INACTIVE IE includes LP-WUSPS Assistance Information, gNB may send LP-WUS for this RRC_INACTIVE UE.

However, since RAN2 introduced the LP-WUS enabling/disabling method, the above observations will be valid in case LP-WUS monitoring for this UE is enabled.

And According to the agreement that: RAN2 assumes that without such NAS signalling, UE is allowed to use LP-WUS in IDLE and INACTIVE. It could be assumed that in case there is no such NAS signalling, and LP-WUSPS Assistance Information is included in the NG Paging message, UE will consider the LP-WUS is enabled. However, when LP-WUS needs to be disabled or to be re-enabled, an indication will be introduced in NAS signalling to indicate the LP-WUS is disabled or re-enabled. Similar as the LP-WUSPS Assistance Information, the information on whether the LP-WUS is enabled/disabled for the UE should be incouded in NG Paging and Core Network Assistance Information for RRC_INACTIVE IE. Based on above, we propose:

Proposal 1a: An indication on the enable/disable LP-WUS will be included in NG Paging message and Core Network Assistance Information for RRC_INACTIVE IE.

Proposal 1b: Without such indication in NG Paging message or Core Network Assistance Information for RRC_INACTIVE IE, gNB understands LP-WUS for this UE is enabled by default.

XnAP interface enhancement

For RRC_INACTIVE UE, the last used gNB could acquire the information on whether LP-WUS monitoring is enabled/disabled, similar as the LP-WUSPS Assistance Information, the last used gNB should include this informaiton to other gNB to inform them whether send LP-WUS for this UE or not. In our view, the same indication design as the NGAP interface could be resued for XnAP interface.

Proposal 2a: An indication on the enable/disable LP-WUS will be included in Xn Paging message.

Proposal 2b: Without such indication in Xn Paging, gNB understands LP-WUS for this UE is enabled by default.

F1AP interface enhancement

Based on the above discussion, the NG-RAN node could acquire the information on whether LP-WUS monitoring is enabled/disabled for RRC_IDLE UEs via NG paging message or for RRC_INACTIVE UEs via Core Network Assistance Information for RRC_INACTIVE IE or Xn paging message. However, only the CU of the NG-RAN node could know the information while the DU of the NG-RAN node couldn’t get it. Considering the LP-WUS signal is generated and transmitted by the DU, the CU needs to inform DU whether the LP-WUS for this UE is enabled or disabled when request the DU to send paging or LP-WUS.

Thus, we propose:

Proposal 3a: An indication on the enable/disable LP-WUS will be included in F1 Paging message.

Proposal 3b: Without such indication in F1 Paging, DU understands LP-WUS for this UE is enabled by default.

Based on the above understanding, we propose to consider the following signalling chart on the information interaction on whether LP-WUS monitoring is enabled/disabled between UE, gNB and AMF, which is shown as follows:

Fig. 1: Overall signalling chart on whether LP-WUS monitoring is enabled/disabled

- Conclusions and Proposals

In this contribution, we discussed the RAN2 LS on enabling/disabling LP-WUS per UE with NAS signalling. Based on the discussion, we have the following observations and proposals.

Observation 1: Once the received NG Paging message includes LP-WUSPS Assistance Information, gNB may send LP-WUS for this RRC_IDLE UE.

Observation 2: Once the received Core Network Assistance Information for RRC_INACTIVE IE includes LP-WUSPS Assistance Information, gNB may send LP-WUS for this RRC_INACTIVE UE.

Proposal 1a: An indication on the enable/disable LP-WUS will be included in NG Paging message and Core Network Assistance Information for RRC_INACTIVE IE.

Proposal 1b: Without such indication in NG Paging message or Core Network Assistance Information for RRC_INACTIVE IE, gNB understands LP-WUS for this UE is enabled by default.

Proposal 2a: An indication on the enable/disable LP-WUS will be included in Xn Paging message.

Proposal 2b: Without such indication in Xn Paging, gNB understands LP-WUS for this UE is enabled by default.

Proposal 3a: An indication on the enable/disable LP-WUS will be included in F1 Paging message.

Proposal 3b: Without such indication in F1 Paging, DU understands LP-WUS for this UE is enabled by default.

- References

R2-2506246, LS on enabling/disabling LP-WUS per UE with NAS signalling, RAN2/Huawei