3GPP TSG-RAN WG3 Meeting #129bis	R3-256700

Prague, Czech Republic, 13th ~17th October, 2025

Title:	Consideration on RAN internal function split and interfaces

Agenda Item:	10.4

Source:	Huawei

Document for:	Discussion

- Introduction

The approved 6G SID [1] contains contents regarding RAN architecture, which includes the following bullet:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we provide our views on the RAN internal functional split and interfaces.

- Discussion

  - Introducing “Xn-like” interface between 6G RAN nodes

6G RAN node

“Xn-like” interface

6G RAN node

Figure 1 Xn-like interface between 6G RAN nodes.

For the RAN interfaces, the “Xn-like” interface between 6G RAN nodes should be introduced for supporting the UE’s mobility, as well as some other features (e.g., dual connectivity, energy saving, resource coordination, etc.) if the necessity can be justified. In addition, the new services (e.g., AI, Sensing) in 6G may also have new requirements on coordination between 6G RAN nodes.

--------------Excerpt from TS 38.420--------------

The general principles for the specification of the Xn interface are as follows:

-	the Xn interface is open;

-	the Xn interface supports the exchange of signalling information between two NG-RAN nodes, and the forwarding of PDUs to the respective tunnel endpoints;

-	from a logical standpoint, the Xn is a point-to-point interface between two NG-RAN nodes. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the two NG-RAN nodes.

Regarding the general principles of such “Xn-like” interface for 6G, we think it can be similar as the principles for Xn interface in 5G, which are excerpted as below [2].

Proposal 1: For RAN internal interfaces, “Xn-like” interface between 6G RAN nodes should be supported, taking the new service requirements into account. RAN3 to reuse the general principles of the Xn interface for 6G “Xn-like” interface as much as possible, in particular:

-	the “Xn-like” interface shall support the exchange of signalling information between two endpoints, and the forwarding of PDUs to the respective endpoints;

-	from a logical standpoint, the “Xn-like” is a point-to-point interface between two endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

  - RAN internal functional split and interfaces

    - 6G CU-DU split and CP-UP separation

Now that 6G will study the RAN internal function split, it is worth to review the design of 5G RAN architecture first and learn some lessons there. The 5G RAN architecture supports CU-DU split, and CP-UP separation. F1 interface was introduced between gNB-CU and gNB-DU, and the E1 interface was designed between gNB-CU-CP and gNB-CU-UP. We try to recollect the motivations and current status for such split RAN architecture as below.

5G CU-DU split

As discussed during the NR study item phase, the motivation/benefits proposed by some proponents for the 5G CU-DU split includes the following[3][4]:

- CU-DU split architecture can facilitate flexible CU/DU deployment, which will be beneficial for meeting various requirements for multiple traffics.

- The CU/DU split architecture can gain from resource pooling in the central unit.

- The CU/DU split architecture can gain from centralized coordination (e.g., interference management, load management, etc.) considering that 5G network maybe be multi-tired.

During the NR SI phase, there were several split options, considering the protocol stack of air interface designed by RAN2.  And after evaluation on these options, the PDCP/RLC split based option was recommended and finally specified during the WI phase.

Observation 1: About 5G CU-DU split, the proponents claimed that such split architecture can gain from: flexible deployment, resource pooling at CU, and centralized coordination for multi-tired network.

5G CP-UP separation

According to 5G TR38.801, the motivation/benefit for the 5G RAN CP-UP separation includes[5]:

-	A centralization of CP functions, controlling different transmission points, has the potential to achieve enhanced radio performance.

-	Flexibility to operate and manage complex networks, supporting different network topologies, resources and new service requirements.

-	Alignment with SDN concept that would result in a functional decomposition of the radio access, based on a partial de-coupled architecture, between user and control plane entities and on network abstractions [10].

-	For functions purely handling with CP or UP processes, independent scaling and realization for control and user plane functions operation.

-	Support of multi-vendor interoperability.

Observation 2: The motivation of 5G RAN CP-UP separation architecture includes performance enhancement, flexibility, and interoperability.

----------Excerption from TR 38.914-----------

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

...

-	The 6G RAN architecture shall allow for control plane and user plane separation.

....

It is worth noting that the 6G requirement for architecture and migration captured one bullet regarding separation of control plane and user plane [6], copied as below.

Our understanding on such requirement is that the 6G RAN architecture should allow the separate handling of user plane data and control plane signalling over 6G interfaces, just like the 4G/5G RAN architecture supports separate design of CP and UP protocol stacks in multiple interfaces (e.g., Uu, Xn, X2, NG, etc.). So, this requirement does not mean that the 6G RAN node should support two different entities (one for CP and one for UP) and need a “E1-like” interface.

Observation 3: The requirement “The 6G RAN architecture shall allow for control plane and user plane separation” aims at supporting separate handling of user plane data and control plane signalling over 6G interfaces, it does not mean that 6G RAN node should support separate CP/UP entities and “E1-like” interfaces.

The status of commercial deployment of CU-DU split/CP-UP separation in 5G RAN

As far as we know, the split RAN architecture with F1/E1 interface has not been massively deployed now. Such situation may be resulted from that some practical factors impose negative impacts on the deployment of split architecture. For example, lacking of optical backhaul network/resources for F1 interface may be one reason. And the CU-DU split will introduce additional processing delay plus transport latency (especially for non-ideal backhaul) over the F1 interface.

Observation 4: The 5G F1/E1 interface is not so common in commercial cases, due to some practical factors, e.g., lacking of ideal backhaul resources, additional latency introduced by functional split.

But for the 6G, the backhaul network might be improved. Moreover, we think operators’ view are worth to be considered, and the motivations from both technical point of view and business point of view for 6G RAN functional split should be comprehensively discussed.

Proposal 2: About whether to support CU-DU split and CP-UP separation in 6G RAN architecture, RAN3 to analysis the technical and business motivations.

    - Fronthaul

3GPP shall capture the following text in italic in a normative Annex of TSxx.401 or TSxx.300 (RAN3):

- 6G NodeB architecture includes a distinct logical Radio Unit (RU).

- The functions hosted by the RU as well as the interface between RU and other parts of 6G NodeB are not specified in 3GPP.

Note-1: This interface can be implemented according to O-RAN ALLIANCE specifications for multi-vendor interoperability.

Note-2: This interface can be implemented according to e.g. (e)CPRI-based or fully proprietary solution(s)

About the 6G fronthaul, it has been extensively discussed in RAN. And the outcome is that 3GPP RAN#109 meeting endorsed a way forward[7] for this 6G fronthaul. The contents of this endorsed WF are copied below.

So, it is clear that the RU function/fronthaul are beyond technical discussion in RAN3. It is proposed

Proposal 3: RAN3 will not discuss fronthaul during 6G study phase.

- Conclusion

This paper investigated the interface between 6G RAN nodes, and the RAN internal functional split, the following observations and proposals are derived:

Xn-like interface between RAN nodes

Proposal 1: For RAN internal interfaces, “Xn-like” interface between 6G RAN nodes should be supported, taking the new service requirements into account. RAN3 to reuse the general principles of the Xn interface for 6G “Xn-like” interface as much as possible, in particular:

-	the “Xn-like” interface shall support the exchange of signalling information between two endpoints, and the forwarding of PDUs to the respective endpoints;

-	from a logical standpoint, the “Xn-like” is a point-to-point interface between two endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

RAN internal split: CU-DU split, CP-UP separation

Observation 1: About 5G CU-DU split, the proponents claimed that such split architecture can gain from: flexible deployment, resource pooling at CU, and centralized coordination for multi-tired network.

Observation 2: The motivation of 5G RAN CP-UP separation architecture includes performance enhancement, flexibility, and interoperability.

Observation 3: The requirement “The 6G RAN architecture shall allow for control plane and user plane separation” aims at supporting separate handling of user plane data and control plane signalling over 6G interfaces, it does not mean that 6G RAN node should support separate CP/UP entities and “E1-like” interfaces.

Observation 4: The 5G F1/E1 interface is not so common in commercial cases, due to some practical factors, e.g., lacking of ideal backhaul resources, additional latency introduced by functional split.

Proposal 2: About whether to support CU-DU split and CP-UP separation in 6G RAN architecture, RAN3 to analysis the technical and business motivations.

Fronthaul

Proposal 3: RAN3 will not discuss fronthaul during 6G study phase.

- Reference

- RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

- 3GPP TS 38.420, NG-RAN; Xn general aspects and principles.

- RWS-150033, DT & T-Mobile USA, View on 5G, 3GPP RAN 5G Workshop.

- R3-160670, AT&T, Flexible Split of Next Generation RAN Architecture Functions.

- 3GPP TR 38.801, Study on new radio access technology: Radio access architecture and interfaces (Release 14).

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements; (Release 20).

- RP-252881, Way forward on 6G Fronthaul. 3GPP TSG RAN #109 meeting.

Annex: Text Proposal for 6G TR (based on R3-255517)

Start of Change

6	RAN Architecture and Interfaces

Editor’s note: The aim of this section is to describe all aspects related to the Interface between gNB to CN.

6.1	General Principles and Requirements

6.2 		RAN-CN interface

6.2.1	RAN-CN interface options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services and include control plane and user plane aspects considering interim milestone.

6.2.2	Evaluation of RAN-CN interface options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 5.2.1

6.3       RAN internal functional split and interfaces

Editor’s Note: This chapter should comprehensively address the CU-DU and CP-UP aspects considering interim milestone.

Editor’s Note: This chapter should also address the interfaces between 6G NBs (equivalent to Xn)

6.3.X	Interface between RAN nodes

The “Xn-like” interface is applicable for the connection between two RAN nodes.

6.3.X.1	General principles of interface between RAN nodes

The general principles for the specification of interface between RAN nodes are as follows:

-	the “Xn-like“ interface shall support the exchange of signalling information between two endpoints, and the forwarding of PDUs to the respective endpoints;

-	from a logical standpoint, the “Xn-like“ is a point-to-point interface between two endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.