3GPP TSG-RAN WG3 Meeting #129bis	R3-256709

Prague, Czech Republic, 13th – 17th October, 2025

Agenda Item:	10.2.2

Source: 	OPPO

Title:  	Discussion on 6G overall RAN architecture

Document for:	Discussion and Decision

- Introduction

In Rel-20 6G SID [1], the objectives led by RAN3 are captured as follows.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

This contribution would like to provide our considerations on 6G overall RAN architecture.

- Discussion

In 5G NR, the functional split between RAN node and CN is captured in 38300, as summarized as the following figure.

Figure 4.2-1: Functional Split between NG-RAN and 5GC

In details, for 5G NG-RAN, the gNB and ng-eNB host the following functions:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling);

-	IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data;

-	Selection of an AMF at UE attachment when no routing to an AMF can be determined from the information provided by the UE;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state;

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Dual Connectivity;

-	Tight interworking between NR and E-UTRA;

-	Maintain security and radio configuration for User Plane CIoT 5GS Optimisation, as defined in TS 23.501 [3] (ng-eNB only).

Now, when we discuss RAN-CN functionality split in 6G, we may take 5G NR functions above as the baseline (other than some LTE-related functions), to determine each function of 6G RAN case by case.

Note that for some functions cross-WGs coordination is needed. For example,

- “Dual Connectivity” is pending the decision of RANP;

- “Support of UEs in RRC_INACTIVE state” is up to RAN2 decision on RRC states in 6G;

- “Routing of Control Plane information towards AMF” and “Routing of User Plane data towards UPF(s)” are up to the decision on SBI-based RAN-CN interface; and

- Some core network (e.g., AMF/UPF) related functions also need to coordinate with SA2.

- For RAN-CN functionality split, RAN3 take 5G NR as starting point for further study, in coordination with other WGs.

- For RAN-CN functionality split, RAN3 should consider the impact due to SBI-based RAN-CN interface.

Besides, for new services and functionalities (e.g. AI/ML, sensing, etc) to be introduced in 6G, RAN3 may wait until there is any the progress in the leading WGs.

- For new services and functionalities (e.g. AI/ML, sensing, etc) to be introduced in 6G, RAN3 may wait until any progress made in the leading WGs.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- For RAN-CN functionality split, RAN3 take 5G NR as starting point for further study, in coordination with other WGs.

- For RAN-CN functionality split, RAN3 should consider the impact due to SBI-based RAN-CN interface.

- For new services and functionalities (e.g. AI/ML, sensing, etc) to be introduced in 6G, RAN3 may wait until any progress made in the leading WGs.

4. Reference

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO