3GPP TSG-RAN WG3 Meeting #129bis	R3-256734

# Prague, CZ, 13 – 17 Oct 2025

Agenda item:	9.2.6

Source:	Nokia, Nokia Shanghai Bell, ZTE Corporation, CATT, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Title:	Discussion on UE Retention at hard FLSO

Document for:	Discussion and Decision

# 1	Introduction

This  contribution discuss the following issues left from last meeting.

short NG/S1 interruption, causing e.g. packet loss, that may be predictable or unpredictable, e.g. hard FLSO to be further discussed in Q4

More specifically, this contribution discusses whether the UE retention feature can be used to solve the above issues during hard FLSO.

# 2	Discussion

Problem Statement for NTN hard FLSO

The problem is mainly for NTN hard FLSO where the gNB/satellite can only connect with a single NTN GW at any time.  In case Hard FLSO is performed, the gNB only connect with the new NTN GW after it disconnect from the old NTN GW. The hard FLSO takes time, e.g. disconnect from old NTN GW, adjust antenna to new NTN GW, setup layer 1/2 connection, setup IPSec tunnel, setup SCTP, initiate NGAP RAN Configuration Update procedure via new SCTP, etc. The Transport connection for NG is unavailable from the gNB disconnect from old NTN GW till the TNL is established with the new NTN GW. This causes an interruption period to NG. During this period, the CN may not know the hard FLSO happens. The CN may continue sending the DL CP/UP packet to the old NTN GW. The old NTN GW has no way to send them to the gNB/satellite since the gNB/satellite has disconnected from the old NTN GW.  The old NTN GW can only discard the packets. This causes packet loss.

In addition, a geo-area is only served by a specific NGSO satellite for a short period. In case this geo-area is in the middle of the serving area of two NTN GWs, each NGSO satellite will perform a FLSO while serving this geo-area. Even though the hard FLSO may only cause a short interruption, this geo-area is subject to a short interruption/hard FLSO from each NGSO satellite. Each NGSO satellite only serves this geo-area for a short period, e.g. minutes. The total interruption due to hard FLSO from all satellites is much longer than the interruption due to the occasional/rare path failure in terrestrial network.  This will cause a bad experience to the end-users.

Observation 1: In NR NTN, the interruption due to hard FLSO is per satellite. For a specific geo-area, the total interruption due to hard FLSO is much larger than the interruption due to occasional path failure in terrestrial network.

Observation 2: Long interruption negatively affects user experience.

Contribution ([2]) proposes the UE retention feature can be used to solve the issue in NR NTN. The UE retention feature was introduced to address the short SCTP path failure. By using this feature, the UE context can be kept in the RAN and CN when the SCTP path failure occurs. After the SCTP path is available, the RAN node informs the CN to use the retained UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained” in the NG SETUP REQUEST message.

However, the UE retention feature cannot solve the packet loss issue because 1) it is performed after the SCTP path failure is detected, and 2) it relies on the length SCTP path failure detection mechanism defined in IETF RFC 4960 (copied as below)

8.2. Path Failure Detection When its peer endpoint is multi-homed, an endpoint should keep an error counter for each of the destination transport addresses of the peer endpoint. Each time the T3-rtx timer expires on any address, or when a HEARTBEAT sent to an idle address is not acknowledged within an RTO, the error counter of that destination address will be incremented. When the value in the error counter exceeds the protocol parameter ’Path.Max.Retrans’ of that destination address, the endpoint should mark the destination transport address as inactive, and a notification SHOULD be sent to the upper layer. 15. Suggested SCTP Protocol Parameter Values The following protocol parameters are RECOMMENDED: RTO.Initial - 3 seconds RTO.Min - 1 second RTO.Max - 60 seconds … Path.Max.Retrans - 5 attempts (per destination address) Max.Init.Retransmits - 8 attempts HB.interval - 30 seconds HB.Max.Burst - 1

By using the default SCTP protocol parameter values, it takes 5 seconds to detect a SCTP path failure per destination transport address. In case multi-homing is used, it takes more time. Of course, the implementation may choose small protocol parameter values, but it has other negative impacts. In terrestrial network, it may be not a problem to use the default SCTP protocol parameter values, since the SCTP path failure is very rare, especially when multi-homing is used.

Observation 3: it takes seconds to detect SCTP path failure.

In other words, the CN only takes action after the SCTP path failure detection is completed, i.e. stop DL transmission seconds later.

Before the path failure detection is completed (e.g. seconds later), the CN continue the downlink transmission which will be lost due to transport network failure (i.e. the gNB/satellite has disconnected from the old NTN GW). As discussed in the beginning, the packet loss is the main issue in hard FLSO. GTP-U is not a loss-less protocol. There is no retransmission mechanism in GTP-U. Once a GTP-U packet is transmitted and lost, there is no way to recover it. So the UE retention feature cannot solve the packet loss occurred during the period from gNB/satellite disconnect from old NTN GW, to the path failure is detected.

Observation 4: the UE retention feature only helps to retain the UE context, but does not solve the packet loss issue, since the CN continue the downlink transmission of user plane until the path failure detection is completed.

One may argue that RAN3 did not introduce enhancement for the path failure issue in terrestrial network, and why RAN3 need to introduce enhancement for NR NTN hard FLSO. The reasons are following:

- In TN, the path failure can happen at any time. There is no way for RAN or CN to know the path failure in advance. So there is nothing that can be done in RAN or CN before the path failure is detected. It is evitable for the packet loss until the path failure is detected. By implementation, Operator may design the transport network to mitigate the transport network failure, and the path failure may be very rare in terrestrial network.

- In NR NTN, hard FLSO is very often in NGSO, especially in LEO system. The total interruption due to hard FLSO is much larger than the interruption due to occasional/rare path failure in terrestrial network. Without solving this issue, the end-user experience in certain geographical area is very bad due to frequent packet loss.  On the other hand, the satellite/RAN know when the hard FLSO is started/completed. So the RAN/CN can take quick action to avoid the packet loss, instead of using the lengthy SCTP path detection mechanism.

Based on above analysis, it is beneficial for the RAN to inform CN, so the CN can take quick actions to avoid the packet loss, e.g. stop DL CP/UP transmission before hard FLSO is started, and resume DL CP/UP transmission after hard FLSO is completed.

Proposal 1: The RAN informs CN before hard FLSO is started, so the CN can stop the downlink transmission to the RAN node. After the hard FLSO is completed, the RAN informs CN to resume the downlink transmission.

# 3	Conclusion

In this contribution, we discussed the remaining issues for hard FLSO. Our proposals are:

Observation 1: In NR NTN, the interruption due to hard FLSO is per satellite. For a specific geo-area, the total interruption due to hard FLSO is much larger than the interruption due to occasional path failure in terrestrial network.

Observation 2: Long interruption negatively affects user experience.

Observation 3: it takes seconds to detect SCTP path failure.

Observation 4: the UE retention feature does not solve the packet loss issue, since the CN continue the downlink transmission of user plane until the path failure detection is completed.

Proposal 1: The RAN informs CN before hard FLSO is started, so the CN can stop the downlink transmission to the RAN node. After the hard FLSO is completed, the RAN informs CN to resume the downlink transmission.

# References

- TS 38.413

- R3-255780, UE Retention at SCTP Path Failure