3GPP TSG-RAN WG3 Meeting #129Bis                              R3-256749 Prague, Czech, Oct 13 – 17, 2025

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	IETF Network Slice Application

Document for:	Approval

Introduction

In the incoming LS[R3-256526], the IETF TEAS WG requests 3GPP, including RAN Working Groups like RAN3, to review its informational document to verify the accuracy of the 3GPP 5G End-to-End Network Slice (E2E NS) description. The core objective is to ensure that the 3GPP slice architecture and parameters (such as S-NSSAI) are accurately mapped for the realization of services within the underlying Transport Network (TN) slice domain defined by IETF. This contribution provide our view on the LS.

Discussion

The high-level description of the E2E NS spanning RAN, Transport Network (TN), and Core Network (CN) segments, identified by S-NSSAI, is accurate.However, we identify several misalignment between 3GPP and IETF spec from RAN3 point of view, in the following part we list the inconsistency and common part between RAN3 and IETF slicing.

Inconsistency description of the DU and CU-UP interface

One of the inconsistency part is shown as below:

7.1.  Example according to CE-mode (OPTION 1)

+-----+        Association between X and Y              +-----+

|     |    according to 3GPP (i.e., NgU/N3 interface)   |     |

| gNB |<----------------------------------------------->| UPF |

|     |                                                 |     |

+-----+                                                 +-----+

+-----+       Association between DU and CU-UP          +-----+

|     |    according to O-RAN (i.e., F1-U interface)    |     |

|  DU |<----------------------------------------------->|CU-UP|

|     |                                                 |     |

+-----+                                                 +-----+

The interface between the Distributed Unit (DU) and the Centralized Unit – User Plane (CU-UP) is the F1-U interface, which is standardized by 3GPP, not O-RAN.

In the 3GPP 5G System (5GS) architecture, the gNB may be functionally split into a Central Unit (CU) and a Distributed Unit (DU). This split architecture defines the internal interfaces:

- The F1 interface is the control plane (F1-C) and user plane (F1-U) interface between the gNB-CU and the gNB-DU.

- The gNB-CU-User Plane (gNB-CU-UP) is a logical node that hosts the user plane part of the PDCP protocol and the SDAP protocol of the gNB-CU. The gNB-CU-UP terminates the F1-U interface connected with the gNB-DU.

Despite correctly citing 3GPP TS 38.470 in the surrounding text, the graphical representation in Section 7.1 of the IETF document (Figure 20, as implied by the context) contains an erroneous association:

- Figure 20 (CE-mode slice realization example between DU and CU-UP) includes a textual association box labeled: "Association between DU and CU-UP according to O-RAN (i.e., F1-U interface)".

- This statement is technically incorrect because the F1-U interface, F1-U protocol stack, and F1 Application Protocol (F1AP) are standard 3GPP specifications (TS 38.401, TS 38.470, TS 38.473).

The same issue is also apply to Figure 24.

Proposal : The specific caption in Figure 20 and Figure 24 attributing the F1-U interface association directly to "O-RAN" is the point of inaccuracy that needs correction/clarification, as the underlying technical definitions and specifications for F1-U originate from 3GPP.

Missing IP address mask

One issue identified by IETF in their specification regarding the missing IP address mask is a gap primarily related to 3GPP Management Plane specifications and is not contained within the operational scope of RAN3's core protocols (TS 38.300, 38.413, 38.423, or 38.470).

- The IP address information flagged as deficient (lacking the subnet mask) resides within the EP_Transport Information Object Class (IOC). This IOC is defined in the 3GPP Management NRM specification TS 28.541.

- RAN protocols (NGAP, XnAP, F1AP) exchange Transport Layer Addresses for user plane tunnels (TNLs), which are IP addresses (e.g., UPF N3/N9 addresses). However, these protocols do not define or exchange the subnet mask/prefix length associated with these addresses.

- The missing mask is required by the IETF Network Slice Controller (NSC) for configuring the underlying Layer 3 transport infrastructure (e.g., defining the attachment circuit configuration or determining routing decisions for CE-mode slice realization), a task handled at the boundary between the 3GPP Management Domain and the IETF Transport Domain.

Observation: he responsibility for defining and including the necessary mask information rests with the 3GPP management/architecture working groups (SA5/SA2), not RAN3.

Conclusion

In this contribution, we provide the following observations and proposal:

Proposal 1: The specific caption in Figure 20 and Figure 24 attributing the F1-U interface association directly to "O-RAN" is the point of inaccuracy that needs correction/clarification, as the underlying technical definitions and specifications for F1-U originate from 3GPP.

Observation: he responsibility for defining and including the necessary mask information rests with the 3GPP management/architecture working groups (SA5/SA2), not RAN3.

Proposal 2: To agree the Reply LS in R3-256750.

Reference

- R3-256526