3GPP TSG-RAN WG3 Meeting #129Bis                              R3-256756 Prague, Czech, Oct 13 – 17, 2025

Agenda Item:	11.2.2

Source:	ZTE Corporation

Title:	MRO for Conditional LTM

Document for:	Discussion and Approval

Introduction

In the Rel-20 WID in RP-251869, the following objective was identified.

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

This contribution provide our initial views focus on MRO for conditional LTM.

Discussion

MRO for intra-CU Conditional LTM

Figure1 :Conditional intra-CU LTM (Inter-gNB-DU)

The Conditional L1/L2 Triggered Mobility (C-LTM), defined as an LTM cell switch procedure executed only when defined execution condition(s) are met, is a Rel-19 mobility mechanism. It aims to combine the high robustness achieved by conditional mobility procedures (like CHO) with the short interruption time offered by L1/L2 triggered mobility (LTM).

From the Mobility Robustness Optimization (MRO) perspective the focus is on optimizing this UE-executed conditional procedure, specifically for intra-CU conditional LTM (covering both Intra-gNB-DU and Inter-gNB-DU scenarios) and inter-CU LTM.

C-LTM relies on UE evaluation of specific conditions before performing a cell switch. These conditions are configurable based on L1 measurements or L3 measurements.

L1 conditions include events like event3 (Beam of candidate cell becomes offset better than serving cell beam),and event5.

Although Rel-19 C-LTM was limited to intra-gNB LTM when Dual Connectivity is not configured, the Intra-gNB-DU and Inter-gNB-DU should all be covered in Rel-20.

Compared with normal LTM, the Network need to decide the not only candidate cells but also associated Execution Condition(s) for each candidate cell. Therefore, the most import task for MRO is to help Network know whether the selected C-LTM candidate cell and associated Execution Condition is suitable.

Proposal 1: RAN3 to work on MRO for Conditional LTM for Intra-gNB DU and inter-gNB DU. And RAN3 to work on failure and near failure case related to UE evaluation L1 measurements or L3 measurements for Conditional LTM. Compared with normal LTM, the Network need to decide the not only candidate cells but also associated Execution Condition(s) for each candidate cell.

R20 MRO User case

The goal of MRO for C-LTM is to optimize the network configuration of execution conditions to mitigate common mobility failures, such as "too early," "too late," or "wrong cell".The user case of MRO for C-LTM is similar as MRO for CHO in Rel-17. It is straightforward to consider similar user cases from MRO for CHO as potential user case for MRO for C-LTM.

Potential user case for too late C-LTM

Figure  Potential cases for too late C-LTM

Case 1: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

This case provides the most straightforward evidence that the initial C-LTM configuration was flawed Too Late. MRO requires the RLF-Report (generated after RLF detection) to capture the configured C-LTM parameters, the latest radio measurements in the source cell, and the crucial time elapsed since configuration (timeSinceCHOReconfig equivalent). Analyzing these data points allows the network to relax the C-LTM execution conditions (L1/L3 thresholds or TTT) for future similar mobility situations.

Case 2: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery in a C-LTM candidate cell.

This case is vital because it tests the robustness of the C-LTM recovery mechanism itself, not just the initial trigger. The MRO system needs to know: 1) Which candidate cell (choCellId equivalent) was chosen for recovery, and 2) Did the C-LTM execution procedure fail during recovery? Since C-LTM can be RACH-less, recovery failure here could be linked to an invalid Timing Advance (TA) value for the candidate cell, which is unique to LTM/C-LTM procedures. The MRO report must capture the candidate cell ID and the failure cause, even if the eventual recovery is successful.

Case 3: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

This is the most valuable failure chain for MRO. Rel-17 MRO for CHO mandated reporting consecutive failures to track the complexity of conditional mobility issues. For C-LTM, MRO needs to: 1) Confirm the initial "Too Late" diagnosis; 2) Analyze why the C-LTM recovery attempt (Failure 2) failed (e.g., incorrect L2 configuration, security failure, or invalid TA); 3) Track the timeUntilReconnection from the first RLF until the eventual success. Optimizing the C-LTM configuration requires understanding the full system impact of the initial configuration error.

Proposal 2: For too late C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

Case 2: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery in a C-LTM candidate cell.

Case 3: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

Potential scenarios for too early C-LTM

Figure 2  Potential scenarios for too early C-LTM

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in the source cell.

The failure in Case 1 is the immediate and explicit consequence of a "Too Early" configuration error. MRO must study this case to:

Diagnose Premature Triggering. Confirm that the configured L1/L3 execution condition was satisfied when the target cell link was still unstable or resources were unavailable. The configured thresholds (e.g., ltm-Threshold, ltm3-Offset) likely need to be tightened or the time filtering (timeToTrigger) needs to be lengthened.

Verify L1/L2 Execution Robustness: C-LTM execution involves a MAC reset and attempts to complete the switch quickly. MRO must analyze if the failure was related to the execution step, particularly the RACH-less procedure.

Case 2: the UE receives C-LTM configuration; the C-LTM execution successes; an RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish the radio link connection in the source cell.

This case 2 signifies a "Too Early" configuration that incorrectly deemed the target cell suitable for long-term use.

Validate Execution Condition and Target Quality: This failure provides critical post-execution data. MRO needs to analyze the reported latest radio measurement results (especially RSRP/RSRQ) of the newly connected cell right before the RLF.

Determine Time of Robustness Collapse: MRO must use the time information to confirm the "shortly after" duration.

Proposal 3: For too early C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in the source cell.

Case 2: the UE receives C-LTM configuration; the C-LTM execution successes; an RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish the radio link connection in the source cell.

Potential scenarios for wrong cell C-LTM

Figure 3 Potential scenarios for C-LTM to wrong cell

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

This case 1 confirms that the C-LTM execution was triggered towards a Wrong Cell (T1) which led to failure (HOF equivalent). The most critical MRO input is identifying the cell where the connection was ultimately successful, via the reconnectCellId field in the final RLF report.

Case 2: the UE receives C-LTM configuration; the C-LTM execution fails; the UE successfully performs C-LTM recovery in another C-LTM candidate cell.

This case proves that the network's low-latency recovery mechanism (attemptLTM-Switch) works when navigating between configured LTM candidates post-failure. MRO needs to capture the target cell ID (T1, the failed attempt) and the recovery cell ID (T2, the successful recovery destination) to differentiate between initial misconfiguration and recovery success.

Case 3: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

This case is a successive failure scenario where the network needs to diagnose two failures from a single configuration set. The initial failure at T1 confirms "Wrong Cell" selection. The second failure (recovery attempt at T2) confirms T2 was also unsuitable, or that the recovery procedure itself failed (e.g., invalid TA/security context).

Case 4: the UE receives C-LTM configuration; the C-LTM execution fails;  the UE attempts to C-LTM recovery to a C-LTM candidate cell and successes; a RLF occurs shortly after C-LTM recovery; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

This case diagnoses a "Wrong Cell" failure (T1) followed by a quality-based failure ("Too Early" migration) in the subsequent recovery cell (T2). Even though the C-LTM recovery (Success 1) succeeded, T2’s quality rapidly collapsed, forcing re-establishment to C3.

Case 5: the UE receives C-LTM configuration; the C-LTM execution successes; a RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish or recovery the radio link connection in a cell other than the source cell and the previously selected target cells.

This is the clearest indicator of a Wrong Cell configuration despite a technical success. The configured condition (L1/L3) was satisfied, the UE executed the switch (successful C-LTM), but the link immediately failed in T1 (new PCell). MRO must analyze the precise measurements and execution condition fulfillment that led to the successful but ultimately poor migration to T1.

Proposal 4: For wrong cell C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 2: the UE receives C-LTM configuration; the C-LTM execution fails; the UE successfully performs C-LTM recovery in another C-LTM candidate cell.

Case 3: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 4: the UE receives C-LTM configuration; the C-LTM execution fails;  the UE attempts to C-LTM recovery to a C-LTM candidate cell and successes; a RLF occurs shortly after C-LTM recovery; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 5: the UE receives C-LTM configuration; the C-LTM execution successes; a RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish or recovery the radio link connection in a cell other than the source cell and the previously selected target cells.

Conclusion

In this contribution, we provide the following observations and proposals:

Proposal 1: RAN3 to work on MRO for Conditional LTM for Intra-gNB DU and inter-gNB DU. And RAN3 to work on failure and near failure case related to UE evaluation L1 measurements or L3 measurements for Conditional LTM.

Proposal 2: For too late C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

Case 2: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery in a C-LTM candidate cell.

Case 3: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

Proposal 3: For too early C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in the source cell.

Case 2: the UE receives C-LTM configuration; the C-LTM execution successes; an RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish the radio link connection in the source cell.

Proposal 4: For wrong cell C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 2: the UE receives C-LTM configuration; the C-LTM execution fails; the UE successfully performs C-LTM recovery in another C-LTM candidate cell.

Case 3: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 4: the UE receives C-LTM configuration; the C-LTM execution fails;  the UE attempts to C-LTM recovery to a C-LTM candidate cell and successes; a RLF occurs shortly after C-LTM recovery; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 5: the UE receives C-LTM configuration; the C-LTM execution successes; a RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish or recovery the radio link connection in a cell other than the source cell and the previously selected target cells.

Reference

[1] RP-251869