3GPP TSG-RAN WG3 Meeting #129bis	R3-256762

# Prague, Czech, 13 – 17, October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | draftCR | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on NR Femto Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In 4.10.2.1, the function of AMF selection is falsely included in the NR Femto node functionalities. - In 4.10.2.2, including the Global RAN Node ID of the NR Femto node in the HANDOVER REQUEST message should belong to the functions in case of the NR Femto node connecting with a GW, but it is included in the functions in case the NR Femto node connecting with AMF. - In 4.10.4, plmn-IdentityInfoList, npn-IdentityInfoList-r16, cellReservedForOtherUse miss the “IE” suffix. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move the function of AMF selection from the NR Femto node functionalities to the GW functionalities. - Include the function of including the Global RAN Node ID of the NR Femto node in the HANDOVER REQUEST message in functionalities in case an NR Femto node connecting to GW. - 3 editorial changes. - Add “IE” in 4.10.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functionalities for NR Femto node, GW or AMF are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.2.1, 4.10.2.2, 4.10.2.3, 4.10.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

4.10.2	Functional Split

4.10.2.1	NR Femto node

An NR Femto node hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable serving NR Femto GW;

NOTE: The NR Femto node may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto node shall only connect to a single NR Femto GW at one time:

-	The NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF.

NOTE:	NR Femto nodes may be deployed without network planning. An NR Femto node may be moved from one geographical area to another and therefore it may need to connect to a different NR Femto GW depending on its location.

-		The TAC and PLMN ID(s) used by the NR Femto node shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is not performed by the NR Femto node:

-	An NR Femto node shall provide the GUAMI or requested NSSAI, if received from the UE, to the NR Femto GW in the INITIAL UE MESSAGE message.

-	At Xn handover:

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the serving AMF in the PATH SWITCH REQUEST message.

-	Allowing the AMF to identify the termination of the source UE associated signalling connection:

-	Including the Source AMF UE NGAP ID in the PATH SWITCH REQUEST message to the NR Femto GW.

4.10.2.2	AMF

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	In case of an NR Femto node connected to an NR Femto GW

-	Routing of handover messages and Downlink RAN Configuration Transfer message towards NR Femto GWs based on the Selected TAI contained in these messages.

-	In NG handover towards the NR Femto node, including the Global RAN Node ID of the NR Femto node in the HANDOVER REQUEST message to the NR Femto GW.

-	In case of an NR Femto node directly connected to an AMF:

-	At NG Setup, verifying, as defined in TS 33.545 [70], that the identity used by the NR Femto node is valid;

-	At Initial UE Message and NG Handover procedures, verifying, as defined in TS 33.545 [70], that the reported CAG ID list is valid for the indicated cell of the NR Femto node;

-	At NG PWS Restart Indication and PWS Failure Indication, verifying, as defined in TS 33.545 [70], that the indicated cell identity is valid.

4.10.2.3	NR Femto GW

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NGAP messages between the AMF and the NR Femto node serving the UE, applying the following additional functions:

-	Terminating the UE Context Release Request procedure if an explicit GW Context Release Indication is included. In this case, the NR Femto GW releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by another NR Femto node attached to it.

-	At UE context establishment (Initial Context Setup or NG Handover) the NR Femto GW sends to the NR Femto node the serving AMF’s GUAMI as well as the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	At Path Switch, the NR Femto GW sends to the NR Femto node the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	At Initial UE Message and NG Handover procedures, the NR Femto GW verifies, as defined in TS 33.545 [70], that the reported CAG ID list is valid for the indicated cell of the NR Femto node.

-	Terminating non-UE associated NGAP procedures towards the NR Femto node and towards the AMF, applying the following additional functions:

-	At NG Setup, the NR Femto GW verifies, as defined in TS 33.545 [70], that the identity used by the NR Femto node is valid.

-	In case of NG PWS Restart Indication and PWS Failure Indication, the NR Femto GW verifies, as defined in TS 33.545 [70], that the indicated cell identity is valid and replaces the Global gNB ID of the NR Femto node with the NR Femto GW ID before sending the respective message to the AMF.

-	At Overload Start/Stop, the NR Femto GW should provide the NR Femto node with the identities of the affected AMF node(s). The NR Femto node uses the received information to identify the traffic and the AMF to which the overload indication applies. The NR Femto node shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto node receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

-	Supporting TAC and PLMN ID used by the NR Femto node.

-	Relaying the PATH SWITCH REQUEST message towards the AMF indicated by the GUAMI of the source AMF received from the NR Femto node.

-	Selecting AMF at UE attachment based on the GUAMI or requested NSSAI provided by the NR Femto in the INITIAL UE MESSAGE message.

A TAI supported by a NR Femto GW shall not be supported by another NR Femto GW.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

4.10.4	Access Control

Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE:		The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto physical node may activate a physical cell shared by both PLMN and PNI-NPN, as specified in clause 4.6, through broadcasting both the plmn-IdentityInfoList IE and the npn-IdentityInfoList-r16 IE in the SIB1, but without the cellReservedForOtherUse IE.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>